SYSTEM SPECIFICATION (SS)

For The

JOINT PRIMARY AIRCRAFT TRAINING SYSTEM (JPATS)

GROUND BASED TRAINING SYSTEM (GBTS)

Configuration Item No. 133-020

Contract No. F33657-94-C-0006

BS25302 Revision A

Issued:  1 July 1997
Revised:  6 October 2000

Raytheon Aircraft Company

9709 E. Central

Wichita, Kansas 67201

CAGE Code 70898

REVISION STATUS

| REV | WRITTEN BY | DATE | APPROVED BY | DESCRIPTION OF CHANGES |
| --- | --- | --- | --- | --- |
| A | Karen Burdick | 10/06/2000 |  | Original issue released herewith. Entire document updated to this revision, which incorporates SCNs 1 through 13 and makes minor changes and corrections. |

TABLE OF CONTENTS

Revision Status	2

Table of Contents	3

List of Tables	11

List of Figures	11

1.0  Scope	12

2.0  Applicable Documents	12

2.1  Government Documents	12

2.2  Non-Government Documents	12

2.3  Order of Precedence	13

2.4  Tiered References	13

2.5  Referencing Within This Document	13

3.0  Requirements	13

3.1  GBTS Description	13

4.3.1  Verification of GBTS Description	15

3.1.1  Interface Definitions	15

4.3.1.1  Verification of Interface Definitions	15

3.1.2  GBTS General Description	15

4.3.1.2  Verification of GBTS General Description	15

3.1.2.1  Aircrew Training Devices	16

4.3.1.2.1  Verification of Aircrew Training Devices	16

3.1.2.2  Development Courses	16

4.3.1.2.2  Verification of Development Courses	16

3.1.2.2.1  Principal Courses	16

4.3.1.2.2.1  Verification of Principal Courses	17

3.1.2.2.2  NFO Courses (Reserved)	17

4.3.1.2.2.2  Verification of NFO Courses (Reserved)	17

3.1.2.2.3  Administrative Courses	17

4.3.1.2.2.3  Verification of Administrative Courses.	17

3.1.2.3  Standardized Courses	18

4.3.1.2.3  Verification of Standardized Courses.	18

3.1.2.3.1  T-37B Detail:  Convert to Authorware	21

4.3.1.2.3.1  Verification of T-37B Detail:  Convert to Authorware	21

3.1.2.3.2  T-1A Detail:  TIMS CMI Interface Modification	22

4.3.1.2.3.2  Verification of T-1A Detail:  TIMS CMI Interface Modification	22

3.1.2.3.3  Navigator Detail:  TIMS CMI Interface and On-Line Test Modification	22

4.3.1.2.3.3  Verification of Navigator Detail:  TIMS CMI Interface and On-LineTest Modification	23

3.1.2.3.4  T-38A/C Detail:  TIMS Courseware Installation and On-Line Test Modification	23

4.3.1.2.3.4  Verification of T-38A/C Detail:  TIMS Courseware Installation and On-Line 
Test Modification	24

3.1.2.3.5  Navy NAS Whiting/NAS Pensacola/NAS Corpus Christi Detail:  TIMS CMI 
Interface and On-Line Test Modification	24

4.3.1.2.3.5  Verification of Navy NAS Whiting/NAS Pensacola/NAS Corpus Christi Detail:  
TIMS CMI Interface and On-Line Test Modification	25

3.1.2.3.6  Test-Only Courses Detail:  Test Modification	25

4.3.1.2.3.6  Verification of Test-Only Courses Detail:  Test Modification	25

3.1.2.4  Operational Support Segment	25

4.3.1.2.4  Verification of Operational Support Segment	25

3.1.2.4.1  Computer Based Training System	26

4.3.1.2.4.1  Verification of Computer Based Training System	26

3.1.2.4.2  Training Integration Management System	26

4.3.1.2.4.2  Verification of Training Integration Management System	26

3.1.2.4.3  Modification and Update Support System	26

4.3.1.2.4.3  Verification of Modification and Update Support System	26

3.1.2.5  TIMS Management Courses	26

4.3.1.2.5  Verification of TIMS Management Courses	26

3.2  System-Level Requirements for Development Courses	30

4.3.2  Verification of System-Level Requirements for Development Courses	30

3.2.1  Syllabus	30

4.3.2.1  Verification of Syllabus.	30

3.2.2  Qualification Requirements	31

4.3.2.2  Verification of Qualification Requirements	31

3.2.2.1  Performance Evaluation	31

4.3.2.2.1  Verification of Performance Evaluation.	31

3.2.3  Principal Courses (Flight Training)	31

SCN 1

4.3.2.3  Verification of Principal Courses (Flight Training)	31

SCN 1

3.2.3.1  Simulated Flight Environment	32

4.3.2.3.1  Verification of Simulated Flight Environment	32

3.2.3.2  Local Area Procedures	32

4.3.2.3.2  Verification of Local Area Procedures.	32

3.2.4  Training Traceability	32

4.3.2.4  Verification of Training Traceability	32

3.2.5  Course Constraints	32

4.3.2.5  Verification of Course Constraints.	32

3.2.5.1  Flying Hours	33

4.3.2.5.1  Verification of Flying Hours	33

3.2.5.2  Duty Day	33

4.3.2.5.2  Verification of Duty Day	33

3.3  Reserved	33

4.3.3  Reserved	33

3.4  System Design and Construction	33

4.3.4  Verification of System Design and Construction	33

3.4.1  Operating Life	33

4.3.4.1  Verification of Operating Life	33

3.4.2  Environmental Conditions	34

4.3.4.2  Verification of Environmental Conditions	34

3.4.3  Human Engineering	34

4.3.4.3  Verification of Human Engineering	35

3.4.3.1  Human Design and Operation	35

4.3.4.3.1  Verification of Human Design and Operation	35

3.4.3.2  Controls and Displays	35

4.3.4.3.2  Verification of Controls and Displays	35

3.4.3.3  Human-Computer Interface	36

4.3.4.3.3  Verification of Human-Computer Interface	36

3.4.3.4  Ambient Noise	36

4.3.4.3.4  Verification of Ambient Noise	36

3.4.4  Power System	36

4.3.4.4  Verification of Power System	36

3.4.4.1  Electromagnetic Interference and Electromagnetic Compatibility	37

4.3.4.4.1  Verification of Electromagnetic Interference and Electromagnetic Compatibility	37

3.4.4.2  Power Interruption and Transients	37

4.3.4.4.2  Verification of Power Interruption and Transients	37

3.4.5  Workmanship	37

4.3.4.5  Verification of Workmanship	37

3.4.5.1  Protective Coatings	37

4.3.4.5.1  Verification of Protective Coatings	37

3.4.6  Reserved	38

4.3.4.6  Reserved	38

3.4.7  Safety	38

4.3.4.7  Verification of Safety	38

3.4.7.1  Hazardous Materials	38

4.3.4.7.1  Verification of Hazardous Materials	38

3.4.7.1.1  Restricted Hazardous Materials	38

4.3.4.7.1.1  Verification of Restricted Hazardous Materials	39

3.4.7.1.2  Targeted Hazardous Materials	39

4.3.4.7.1.2  Verification of Targeted Hazardous Materials	40

3.4.8  Nameplates and Product Marking	40

4.3.4.8  Verification of Nameplates and Product Marking	40

3.4.9  System Security and Training Integrity	40

4.3.4.9  Verification of System Security and Training Integrity	40

3.4.10  Computation Systems	40

4.3.4.10  Verification of Computation Systems	40

3.4.10.1  Computation System Hardware	41

4.3.4.10.1  Verification of Computation System Hardware	41

3.4.10.1.1  PC Energy Efficiency	41

4.3.4.10.1.1  Verification of PC Energy Efficiency	41

3.4.10.2  Computation System Software	41

4.3.4.10.2  Verification of Computation System Software	41

3.4.10.2.1  Computer Programming Language	41

4.3.4.10.2.1  Verification of Computer Programming Language	41

3.4.10.2.2  Design and Coding Constraints	41

4.3.4.10.2.2  Verification of Design and Coding Constraints	41

3.4.10.2.3  Software Portability	42

4.3.4.10.2.3  Verification of Software	42

3.4.10.3  Computation System Spare and Growth	42

4.3.4.10.3  Verification of Computation System Spare and Growth	42

3.4.10.4  Interoperability	43

4.3.4.10.4  Verification of Interoperability	43

3.4.11  Reserved	43

4.3.4.11  Reserved	43

3.4.12  Mission Effectiveness	43

4.3.4.12  Verification of Mission Effectiveness	43

3.4.12.1  ATD Mission Effectiveness	43

4.3.4.12.1  Verification of ATD Mission Effectiveness	43

3.4.12.2  CBTS Mission Effectiveness	44

4.3.4.12.2  Verification of CBTS Mission Effectiveness	44

3.4.12.3  TIMS Mission Effectiveness	44

4.3.4.12.3  Verification of TIMS Mission Effectiveness	45

3.4.13  Facilities	45

4.3.4.13  Verification of Facilities	45

3.4.14  Daily Readiness Test	45

4.3.4.14  Verification of Daily Readiness Test	45

3.5  Documentation and Data Repository	45

4.3.5  Verification of Documentation and Data Repository	45

3.5.1  Master Technical Data Library	46

4.3.5.1  Verification of Master Technical Data Library	46

3.5.2  GBTS On-Site Libraries	46

4.3.5.2  Verification of GBTS On-Site Libraries	46

3.6  Manpower, Personnel and Training	47

4.3.6  Verification of Manpower, Personnel and Training	47

3.7  Logistics	47

4.3.7  Verification of Logistics	47

3.8  Segment and Component Detailed Characteristics	47

4.3.8  Verification of Segment and Component Detailed Characteristics	47

3.8.1  Aircrew Training Device Segment	47

4.3.8.1  Verification of Aircrew Training Device Segment	47

3.8.1.1  Flight Training Devices	47

4.3.8.1.1  Verification of Flight Training Devices.	48

3.8.1.1.1  FTD Components	48

4.3.8.1.1.1  Verification of FTD Components.	48

3.8.1.1.1.1  Operational Flight Trainer	48

4.3.8.1.1.1.1  Verification of Operational Flight Trainer	49

3.8.1.1.1.2  Instrument Flight Trainer	49

4.3.8.1.1.1.2  Verification of Instrument Flight Trainer	49

3.8.1.1.1.3  Unit Training Device	49

4.3.8.1.1.1.3  Verification of Unit Training Device	49

3.8.1.1.2  FTD Elements	50

4.3.8.1.1.2  Verification of FTD Elements	50

3.8.1.1.2.1  Central Training Complex	50

4.3.8.1.1.2.1  Verification of Central Training Complex	50

3.8.1.1.2.2  Instructor Operator Station	50

4.3.8.1.1.2.2  Verification of Instructor Operator Station	50

3.8.1.1.2.3  Remote Instructor Station	50

4.3.8.1.1.2.3  Verification of Remote Instructor Station	51

3.8.1.1.2.4  Canopy	51

4.3.8.1.1.2.4  Verification of Canopy	51

3.8.1.1.2.5  Image Generator	51

4.3.8.1.1.2.5  Verification of Image Generator	51

3.8.1.1.2.6  Wide Field of View Visual Display	51

4.3.8.1.1.2.6  Verification of Wide Field of View Visual Display	51

3.8.1.1.2.7  Narrow Field of View Visual Display	51

4.3.8.1.1.2.7  Verification of Narrow Field of View Visual Display	52

3.8.1.2  Procedural Training Devices	52

4.3.8.1.2  Verification of Procedural Training Devices	52

3.8.1.2.1  Egress Procedures Trainer	52

4.3.8.1.2.1  Verification of Egress Procedures Trainer	52

3.8.1.2.2  Ejection Seat Trainer	53

4.3.8.1.2.2  Verification of Ejection Seat Trainer.	53

3.8.1.2.3  Parachute Procedures Trainer	53

4.3.8.1.2.3  Verification of Parachute Procedures Trainer	53

3.8.2  Development Courseware Elements	53

4.3.8.2  Verification of Development Courseware Elements	53

3.8.2.1  ISD Materials	54

4.3.8.2.1  Verification of ISD Materials	54

3.8.2.2  Instructor Materials	54

4.3.8.2.2  Verification of Instructor Materials	54

3.8.2.2.1  Abbreviated Syllabus	54

4.3.8.2.2.1  Verification of Abbreviated Syllabus	54

3.8.2.3  Course Assessment	54

4.3.8.2.3  Verification of Course Assessment	54

3.8.2.4  Student Materials	55

4.3.8.2.4  Verification of Student Materials	55

3.8.2.5  Computer-Aided Instruction	55

4.3.8.2.5  Verification of Computer Aided Instruction	55

3.8.2.6  Training Aids and Classroom Equipment	55

4.3.8.2.6  Verification of Training Aids and Classroom Equipment	55

3.8.2.7  FTD Lesson Sortie Files	56

4.3.8.2.7  Verification of FTD Lesson Sortie Files	56

3.8.2.7.1  Simulated Flight Data Bases	56

4.3.8.2.7.1  Verification of Simulated Flight Data Base	56

3.8.2.8  Electronic Management Files	56

4.3.8.2.8  Verification of Electronic Management Files	57

3.8.3  Standardized Courseware Elements	57

4.3.8.3  Verification of Standardized Courseware Elements	57

3.8.3.1  Standardized Courseware	57

4.3.8.3.1  Verification of Standardized Courseware	57

3.8.3.2  TIMS Management Courses	57

4.3.8.3.2  Verification of TIMS Management Courses	57

3.8.4  Operational Support Segment	58

4.3.8.4  Verification of Operational Support Segment	58

3.8.4.1  Computer-Based Training System	59

4.3.8.4.1  Verification of Computer-Based Training System	59

3.8.4.1.1  CAI Laboratory	59

4.3.8.4.1.1  Verification of CAI Laboratory.	60

3.8.4.1.1.1  Student Stations	60

4.3.8.4.1.1.1  Verification of Student Stations	61

3.8.4.1.2  MIL Classroom	61

4.3.8.4.1.2  Verification of MIL Classroom	61

3.8.4.1.3  Reserved	61

4.3.8.4.1.3  Reserved	61

3.8.4.1.4  CBTS Computers	62

4.3.8.4.1.4  Verification of CBTS Computers	62

3.8.4.1.4.1  Real-Time Imagery	62

4.3.8.4.1.4.1  Verification of Real-Time Imagery	62

3.8.4.2  Training Integration Management System	62

4.3.8.4.2  Verification of Training Integration Management System	63

3.8.4.2.1  Scheduling	64

4.3.8.4.2.1  Verification of Scheduling	64

3.8.4.3  Modification and Update Support System	64

4.3.8.4.3  Verification of Modification and Update Support System	64

3.8.4.3.1  Aircrew Training Device Support System (ATDSS)	64

4.3.8.4.3.1  Verification of Aircrew Training Device Support System (ATDSS)	65

3.8.4.3.1.1  Traceability	65

4.3.8.4.3.1.1  Verification of Traceability	65

3.8.4.3.1.2  Computer Aided Software Engineering Tools	65

4.3.8.4.3.1.2  Verification of Computer Aided Software Engineering Tools	65

3.8.4.3.1.3  ATDSS Usage	65

4.3.8.4.3.1.3  Verification of ATDSS Usage	65

3.8.4.3.2  Curriculum Support System (CSS)	66

4.3.8.4.3.2  Verification of Curriculum Support System	66

3.8.4.3.2.1  Traceability of Objectives	66

4.3.8.4.3.2.1  Verification of Traceability of Objectives	66

3.8.4.3.2.2  CSS Usage	67

4.3.8.4.3.2.2  Verification of CSS Usage	67

3.8.4.3.2.3  Call In Support	67

4.3.8.4.3.2.3  Verification of Call In Support	67

3.8.4.3.3  TIMS Support System (TIMS-SS)	67

4.3.8.4.3.3  Verification of TIMS Support System (TIMS-SS)	67

3.8.4.3.3.1  Requirements Traceability	67

4.3.8.4.3.3.1  Verification of Requirements Traceability	67

3.8.4.3.3.2  Computer Program Support Environment	68

4.3.8.4.3.3.2  Verification of Computer Program Support Environment	68

3.8.4.3.3.3  TIMS-SS Usage	68

4.3.8.4.3.3.3  Verification of TIMS-SS Usage	68

3.8.4.3.3.4  Call In Support	68

4.3.8.4.3.3.4  Verification of Call in Support	68

4.0  Quality Assurance Provisions	69

4.1  General	69

4.1.1  Responsibility for Tests	69

4.2  Specific Verification Provisions	69

4.2.1  Inspection	69

4.2.2  Analysis	69

4.2.3  Demonstration	70

4.2.4  Test	70

4.3  Verification of Requirements	70

5.0  Reserved	70

6.0  Notes	70

6.1  Definitions	70

LIST OF TABLES

TABLE 3.1.2.2.1	Principal Courses	17

TABLE 3.1.2.2.2	NFO Courses (Reserved)	17

TABLE 3.1.2.2.3	Administrative Courses	18

TABLE 3.1.2.3	Standardized Courses (USAF)	19

TABLE 3.1.2.4	Standardized Courses (Navy)	20

TABLE 3.1.2.5	Reserved	21

TABLE 3.1.2.6	TIMS Management Courses (USAF/USN)	27

TABLE 3.8.2.1	FTD Element Definition	48

TABLE 3.8.4.1-1	CAI Laboratories by Base	60

TABLE 3.8.4.1-2	MIL Classrooms by Base	61

LIST OF FIGURES

FIGURE 3.1	GBTS Hierarchy	14

# 1.0  Scope

This System Specification  (SS) establishes the requirements for the Ground Based Training System (GBTS) portion of the Joint Primary Aircraft Training System (JPATS).  The GBTS shall provide a set of educational tools that in conjunction with the JPATS Aircraft enable the United States Air Force (USAF) and the United States Navy (USN) personnel (both military and contracted) to conduct undergraduate flying training.  The GBTS shall also support other USAF and USN undergraduate flying training as required herein.

# 2.0  Applicable Documents

# 2.1  Government Documents

The following government documents form a part of this specification to the extent specified.  Unless otherwise specified, the issues shall be those in effect on the date of the GBTS solicitation.

| Number | Revision | Date | Title |
| --- | --- | --- | --- |
| AFOSH 127-66 |  | 01 Mar 82 | General Industrial Operations |
| AFOSH 127-118/T |  | 1 Sep 91 | Training System Fire Protection |
| MIL STD 1472 | D | Mar 89 | Human Engineering Design Criteria for Military Systems, Equipment and Facilities |
|  | 2.0 | 27 Aug 99 | Joint Interoperability Profile (JIP) for the JPATS GBTS |

# 2.2  Non-Government Documents

The following documents form a part of this specification to the extent specified.  Unless otherwise specified, the issues shall be those in effect on the date of the solicitation.

| Number | Revision | Date | Title |
| --- | --- | --- | --- |
| NFPA-70 |  | 1993 | NFPA National Electrical Code |
| ANSI/HFS 100 |  | 1988 | Human Factors Engineering of Visual Display Terminal Workstations |
| ANSI X3.135-1992 |  | 1992 | American National Standard Database Language SQL |

## 2.3  Order of Precedence

In the event of a conflict between the contents herein and the cited references, the contents of this document shall take precedence.

# 2.4  Tiered References

Tiered references shall not apply as requirements, except for the JIP.  Tiered references apply as requirements only to the first level in the JIP.  Documents referenced within other referenced documents, or beyond the first tier in the JIP, shall only be applicable as guidance.

# 2.5  Referencing Within This Document

Referencing a paragraph references all subparagraphs and subsections of that paragraph.

# 3.0  Requirements

# 3.1  GBTS Description

The GBTS is organized as shown in Figure 3.1 on the following page.  The GBTS shall be deployed at all, or any combination of the Air Force Bases and Naval Air Stations listed below (depending on specific contractual actions).

SCN 1

a.	Randolph AFB

b.	Laughlin AFB

c.	Columbus AFB

d.	Vance AFB

e.	Sheppard AFB

f.	NAS Pensacola

g.	NAS Corpus Christi

h.	NAS Whiting

i.	NAS Kingsville

j.	NAS Meridian

SCN 1

k.	Reserved

SCN 1

l.	Reserved

m.	Moody AFB

The GBTS shall accommodate any throughput up to the maximum throughput specified in Tables 3.1.2.2.1 to 3.1.2.2.3, and 3.1.2.6.  The GBTS shall recover from short term interruptions to training such as adverse weather, Operational Readiness Verifications (ORVs), etc., without affecting throughput rates.

FIGURE 3.1   GBTS Hierarchy

## 4.3.1  Verification of GBTS Description

This requirement shall be verified by analysis at design reviews.

## 3.1.1  Interface Definitions

The GBTS components shall interface with the following systems, as necessary, to meet the requirements of this System Specification:

- facilities in which it is installed.

- the JPATS aircraft and its support infrastructure.

- external computer systems.

- other Components of the GBTS, including components at other bases.

The GBTS shall include all cabling necessary to interface within itself and with other systems at each base.  For the Navy, the GBTS shall also include interconnections between buildings at each base.  In addition, the operation of the GBTS shall be consistent with AETC and CNATRA policies and procedures.

## 4.3.1.1  Verification of Interface Definitions

This requirement shall be verified by inspection, analysis, demonstration, and test.  Facilities interface requirements shall be verified by analysis at design reviews.  Facilities interface shall be verified by inspection at each site.  JPATS aircraft and its support infrastructure interface shall be verified by analysis of course material at design reviews and by demonstration during System Level Formative Evaluation (SLFE).  Interfaces to external computer systems and other GBTS components shall be verified by analysis of interfaces during design review, by test when required by this specification and by demonstration during SLFE.  The cabling interface requirement shall be verified by analysis of network design diagrams during design reviews.

## 3.1.2  GBTS General Description

The following paragraphs identify each of the Segments and Components of the GBTS.

## 4.3.1.2  Verification of GBTS General Description

Verification of this paragraph is not applicable.

## 3.1.2.1  Aircrew Training Devices

The Aircrew Training Devices (ATD), consisting of Flight Training Devices (FTD) (the Operational Flight Trainer (OFT), the Instrument Flight Trainer (IFT), and the Unit Training Device (UTD)) and Procedural Training Devices (PTD) (the Egress Procedures Trainer (EPT), the Ejection Seat Trainer (EST), and the Parachute Procedures Trainer (PPT)) shall perform all training allocated to them by the principal course syllabuses.

## 4.3.1.2.1  Verification of Aircrew Training Devices

This requirement shall be verified by analysis, and demonstration.  Analysis at design reviews shall show the correlation of each ATD design to the training tasks allocated to that ATD.  ATD performance for all training allocated to them by the principal course syllabuses shall be demonstrated using demonstration missions conducted during other ATD testing required by this specification.

## 3.1.2.2  Development Courses

Each development course (i.e., principal course, secondary course and administrative course) shall:

- Include all courseware as required herein.

- Utilize the Computer-Based Training System (CBTS) to the extent required by the JPATS development syllabuses.

- Be supported by all portions of the Modification and Update Support System (MUSS).

- Be managed by the Training Integration Management System (TIMS).

The maximum throughput shall consist of C classes per year with S students per class.  Development courses shall be deployed at all or any combination of bases as indicated in tables 3.1.2.2.1 to 3.1.2.2.3.

## 4.3.1.2.2  Verification of Development Courses

This requirement shall be verified by analysis and demonstration.  Analysis at design reviews shall verify all required courseware is provided and that CBTS is utilized to the extent required.  TIMS operation and MUSS support shall be demonstrated at SLFE using actual course material.

## 3.1.2.2.1  Principal Courses

Principal courses (Joint Primary Pilot Training (JPPT) in the JPATS aircraft) are listed in Table 3.1.2.2.1.

TABLE 3.1.2.2.1  Principal Courses

| Course | C | S | Bases |
| --- | --- | --- | --- |
| Joint Primary Pilot Training (JPPT) Student | 24 | 20 | Columbus, Vance, Sheppard, and Laughlin AFBs |
|  | 10 | 20 | Moody AFB |
|  | 24 | 30 | NAS Corpus Christi |
|  | 24 | 45 | NAS Whiting |
| JPPT Pilot Instructor Training | 10 | 10 | Randolph AFB |
|  | 12 | 9 | NAS Corpus Christi |
|  | 12 | 10 | NAS Whiting |

(C - Classes per year per base, S - Students per class)

# 4.3.1.2.2.1  Verification of Principal Courses

This requirement shall be verified by demonstration of selected elements of each course.  The student throughput requirement shall be verified by analysis at design reviews.

# 3.1.2.2.2  NFO Courses (Reserved)

TABLE 3.1.2.2.2 NFO Courses (Reserved)

| Course | C | S | Bases |
| --- | --- | --- | --- |

(C - Classes per year per base, S - Students per class)

# 4.3.1.2.2.2  Verification of NFO Courses (Reserved)

# 3.1.2.2.3  Administrative Courses

Administrative courses (i.e., "how-to" courses for GBTS operation) shall be tailored to individual duties and roles.  Administrative courses are listed in Table 3.1.2.2.3.

## 4.3.1.2.2.3  Verification of Administrative Courses.

This requirement shall be verified as per Paragraph 4.3.1.2.2.1

TABLE 3.1.2.2.3  Administrative Courses

| Course | C | S | Bases |
| --- | --- | --- | --- |
| TIMS Operation | 5 | 45 | Moody AFB |
| SCN 1 | 10 | 45 | Randolph, Columbus, Laughlin, Sheppard, and Vance AFB NAS Whiting, Pensacola, Corpus Christi, Kingsville, and Meridian |
| Computer Aided Instruction (CAI) Authoring and Curriculum Support System Operation | 4 | 10 | Randolph AFB |
|  | 4 | 10 | NAS Corpus Christi |
| JPATS ATD Instructor and Operator | 2 | 10 | Randolph and Moody AFBs |
|  | 4 | 10 | NAS Whiting and NAS Corpus Christi |
|  | 2 | 10 | NAS Pensacola |
|  | 4 | 13 | Columbus, Vance, Sheppard and Laughlin AFBs |
| New Media Academic Instructor | 4 | 25 | Randolph AFB |
| SCN 1 | 4 | 50 | Columbus, Laughlin, Sheppard, Vance and Moody AFB NAS Whiting, Pensacola, Corpus Christi, Kingsville, and Meridian |

(C - Classes per year per base, S - Students per class)

## 3.1.2.3  Standardized Courses

Standardized courses are listed in Tables 3.1.2.3 and 3.1.2.4.  Standardized Courses are modified as described in the detail listings following the respective table.  Each courses’ data files shall be supported by CSS to the extent that the modification described in the detail provides for that support.

## 4.3.1.2.3  Verification of Standardized Courses

Verification of this requirement is accomplished in the following subparagraphs.

TABLE 3.1.2.3   Standardized Courses (USAF)

| Course ID | Title | CAI Language | CAI Hrs | CAI Reuse | MIL Hrs | MIL Reuse | C | S | Location |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F-V5A-E | T-1A Pilot Instructor Training (PIT) | Authorware |  | 32.3 | 48 |  | 11 | 10 | Randolph AFB |
| N-V4R-N-2 | Airlift/ Tanker/ Maritime/ Navigator/ NFO | Authorware | 8 |  | 130 |  |  |  | Randolph AFB |
| N-V4R-N-1 | Primary Navigator/NFO | Authorware | 19.5 |  | 206 |  | 12 | 13 | Randolph AFB |
| N-V4R-N-4 | Theater Operations | Authorware | 1 |  | 76 |  | 12 | 13 | Randolph AFB |
| N-V9-IT | Navigator/NFO/EWO Instructor Training | Authorware | 1 |  | 101 |  |  |  | Randolph AFB |
| P-V4A-G** | T-1A Specialized Undergraduate Pilot Training** | Authorware | 89 |  | 72 |  | 15 | 16 | Laughlin AFB, Vance AFB, Columbus AFB |
| F-V5A-Q** | T-1A Fixed Wing Transition** | Authorware |  | 89 |  | 72 | 15 | 16 | Laughlin AFB, Vance AFB, Columbus AFB |
| N-V5-CN | Celestial Navigation Course | Authorware | 7 |  | 63 |  |  |  | Randolph AFB |
| P-V4A-A | T-37B Joint Specialized UPT | MicroTICCIT | 46 |  | 143 |  | 15 | 29 | Laughlin AFB |
|  |  |  |  |  |  |  | 15 | 29 | Vance AFB |
|  |  |  |  |  |  |  | 15 | 27 | Columbus AFB |
|  |  |  |  |  |  |  | 15 | 27 | Sheppard AFB |
| P-V4A-E (TRIM) | T-37B Aviation Leadership Program (ALP) | MicroTICCIT | 2 | 46 | 7 | 43 | 6 | 4 | Laughlin AFB |
| F-V5A-W (TRIM) | T-37B USN Pre-Qualification Course | MicroTICCIT |  | 16 |  | 77.5 |  |  | Vance AFB |
| F-V5A-Q (TRIM) | T-37B Fixed Wing Qualification | MicroTICCIT |  | 27 |  | 98 | 4 | 2 | Vance AFB |
| P-V4A-A | T-38A Joint Specialized UPT Course | Authorware | 27.5 |  | 43 | 0 | 15 | 9 | Laughlin AFB |
|  |  |  |  |  |  |  | 15 | 8 | Vance AFB |
|  |  |  |  |  |  |  | 15 | 8 | Columbus AFB |
| P-V4A-P | T-38A Advanced Training Program | Authorware | 0 | 39 |  | 46 | 4 | 2 | Laughlin AFB, Columbus AFB |
| F-V5A-Q | T-38A Fixed Wing Qualification | Authorware | 0 | 28 | 43 |  | 2 | 4 | Vance AFB |
| F-V5A-B(R) | T-38A Pilot Instructor Requalification | Authorware | 0 | 20 | 38 | 0 |  |  | Randolph AFB |
| B/FV5A-K(C) | T-38C Introduction to Fighter Fundamentals | Authorware | *TBD | *TBD | 0 | 0 | 15 | 15 | Moody AFB |
| B/FV5A-M(C) | T-38C Introduction to Fighter Fundamentals Instructor Training | Authorware | *TBD | *TBD | 0 | 0 |  |  | Randolph AFB (25 Students/ yr) |
| *TBD | T-38 A to C Transition Course at Introduction to Fighter Fundamentals | Authorware | *TBD | *TBD | 0 | 0 |  |  | Randolph AFB |

*	These TBD courses are not yet developed.

**	These are two courses that use the same syllabus.  Each course uses the same CAI hours.

TABLE 3.1.2.4   Standardized Courses (USN)

| Course ID | Title | CAI Language | CAI Hours | CAI Reuse | MIL Hrs | MIL Reuse | C | S | Location |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCN 2 1542.54K | Primary Student NFO Training Curriculum | PowerPoint | 0 | 0 | 38.5 | 0 | 24 | 24 | NAS Pensacola |
|  |  | Toolbook | 0 | 0 | 8 |  |  |  |  |
|  |  | C++ (RIOT) | 3 | 0 | 26 |  |  |  |  |
|  |  | Mandarin | 3 | 0 | 0 |  |  |  |  |
|  |  | Partest | 0 | 5 | 9 |  |  |  |  |
| 1542.32D | Primary/Intermediate NFO/NAV IUT Curriculum | PowerPoint | 0 | 0 | 46 | 37 | 12 | 3 | NAS Pensacola |
|  |  | Partest |  |  | 13.5 | 0 |  |  |  |
| 1542.131B SCN 2 | Intermediate NFO/Navigator Training Curriculum | PowerPoint | 0 | 0 | 31.5 | 0 | 24 | 17 | NAS Pensacola |
|  |  | Partest |  |  | 8 |  |  |  |  |
| 1542.134 | Intermediate NFO/Navigator T-1A IUT Curriculum | PowerPoint | 0 | 0 | 83 | 83 | 7 | 1 | NAS Pensacola |
|  |  | Partest |  |  | 4 | 0 |  |  |  |
| 1542.65H SCN 2 | Joint Advanced Multi-Engine (T-44A) | Authorware | 25 | 0 | 32 | 0 | 10 | 3 | NAS Corpus Christi |
| 1542.61H SCN 2 | Primary/Intermediate Maritime/Helo Flight Instructor Training Curriculum | Partest | 0 | 0 | 13 | 0 | 12 | 5 | NAS Whiting |
|  |  |  |  |  |  |  | 12 | 3 | NAS Corpus Christi |
| 1542.66G SCN 2 | Maritime Flight Instructor | Authorware | 25 | 25 | 29 | 29 | 15 | 20 | NAS Corpus Christi |
| 1542.73F SCN 2 | Intermediate E2/C2 Training (T-44) | Authorware | 25 | 25 | 29 | 29 | 23 | 2 | NAS Corpus Christi |
| 1542.104 SCN 2 | Naval Test Pilot Prep/Maritime Transition (T-44/TC-12) | Authorware | 25 | 25 | 17 | 17 | 15 | 1 | NAS Corpus Christi |
| 1542.138A SCN 2 | Joint Advanced Multi-Engine TC-12 Flight Training | Authorware | 30 | 4 | 19 | 0 | 10 | 11 | NAS Corpus Christi |
| 1542.140A SCN 2 | Multi-Service Pilot Training System | Mandarin | 25 | 0 | 0 | 0 | 24 | 32 | NAS Whiting |
|  |  | Toolbook | 0 |  | 8 |  |  |  |  |
|  |  | Authorware | 15.5 |  | 14 |  | 24 | 20 | NAS Corpus Christi |
|  |  | Partest | 0 |  | 4.5 |  |  |  |  |
| 1542.41D | Helicopter Transition Pilot Curriculum | Mandarin | 20 | 20 | 0 | 0 | 10 | 1 | NAS Whiting |
|  |  | Authorware | 1.5 | 1.5 | 2 | 2 |  |  |  |
| 1542.53F SCN 2 | Aerospace Medicine Specialist Indoctrination Curriculum | Mandarin | 20 | 0 | 0 | 0 | 12 | 1 | NAS Whiting |
|  |  | Authorware | 0 |  | 6 |  |  |  |  |
|  |  | Toolbook | 0 |  | 2 |  |  |  |  |
| 1542.91E | Undergraduate Helicopter Flight IUT | Mandarin | 27.5 | 27.5 | 0 | 0 | 12 | 4 | NAS Whiting |
|  |  | Authorware | 5.5 | 5.5 | 11.5 | 11.5 |  |  |  |
| 1542.93F SCN 2 | Undergraduate Helicopter Flight Training | Mandarin | 27.5 | 0 | 0 | 0 | 26 | 21 | NAS Whiting |
|  |  | Authorware | 6.5 |  | 19.5 |  |  |  |  |

TABLE 3.1.2.5  Reserved

## 3.1.2.3.1  T-37B Detail:  Convert to Authorware

This shall apply to the following T-37B courses:

- P-V4A-A: T-37B Joint Specialized UPT

- P-V4A-E (TRIM): T-37B Aviation Leadership Program (ALP)

- F-V5A-W (TRIM): T-37B USN Pre-Qualification Course

- F-V5A-Q (TRIM): T-37B Fixed Wing Qualification.

These courses shall be converted to the Authorware courseware authoring language and be capable of being supported by the CSS.  The converted courseware shall have the same text as depicted in the existing MicroTICCIT (MCT) screens.  The converted courseware shall contain the graphics files provided by the TRSS.  The converted courseware branching and navigation shall conform to the following rules:

- All learning objectives shall be stated exactly as they are stated in the corresponding MCT screens.

- The course shall be developed in a linear sequence.  The Rule, Rule Easy, and Rule Hard branching methodology will not be duplicated.

- The MCT Rule Help, Example Help, Practice, Practice Help, and Feedback functions shall be provided by the JPATS T-6A help, hyper-text, or remediation function as defined in the Lesson Design & Strategy Report (LDSR).

- Paper-based tests shall be converted to the JPATS test GUI.

- The JPATS test system shall be modified to provide support for alternate forms of a test.

- The TIMS CMI interface shall be modified to provide support for alternate forms of a test.

## 4.3.1.2.3.1  Verification of T-37B Detail:  Convert to Authorware

The requirement for conversion of courseware to Authorware shall be verified by side-by-side demonstration of selected lessons of each course.  Additionally, this requirement shall be verified using an operational CBTS TIMS environment to demonstrate the following for selected lessons and tests:  (1) the launch and termination of lessons and tests under TIMS Management, (2) TIMS Management of on-line tests CMI output data files, (3) TIMS Management of lesson completion data, (4) TIMS updates to the student grade book, (5) TIMS support of alternate test forms, and (6) on-line presentation of tests.

## 3.1.2.3.2  T-1A Detail:  TIMS CMI Interface Modification

This shall apply to the following T-1A courses:

- F-V5A-E: T-1A Pilot Instructor Training (PIT)

- P-V4A-G: T-1A Specialized Undergraduate Pilot Training

- F-V5A-Q: T-1A Fixed Wing Transition.

For these T-1A courses, the TIMS CMI interface shall be modified to:

- Launch T-1A lessons and tests and record completion information.

- Aggregate T-1A CMI data to TIMS.

- Provide the four additional academic reports currently used by the T-1A courseware.

- Import T-1A objectives into TIMS.

## 4.3.1.2.3.2  Verification of T-1A Detail:  TIMS CMI Interface Modification

This requirement shall be verified using an operational CBTS TIMS environment to demonstrate the following for selected lessons and tests:  (1) the launch and termination of lessons and tests under TIMS Management, (2) TIMS Management of CMI output data files, and (3) TIMS updates to the student gradebook.  The four additional T-1A academic reports shall be verified in a side-by-side (electronic or paper) demonstration of report content and format.  The T-1A objectives import shall be verified by a demonstration of selected on-line objectives in TIMS.

## 3.1.2.3.3  Navigator Detail:  TIMS CMI Interface and On-Line Test Modification

This shall apply to the following Navigator courses:

- N-V4R-N-1 (N-V4-CO): Student NAV/NFO

- N-V4R-N-4: (N-V4-TO): Theater Operations

- N-V4R-N-2: (N-V4-AT): Airlift/Tanker/Maritime Navigator/NFO (formerly ATM Syllabus)

- N-V5-CN: Celestial Navigation

- N-V9-IT: Navigator/NFO/EWO Instructor Training.

For these Navigator courses, the following shall be accomplished:

- The TIMS CMI interface shall be modified to Launch Navigator lessons and record completion information.

- Paper based tests, currently in Page Maker and Word, shall be converted to the JPATS test GUI.

- The JPATS test system shall be modified to provide support for alternate forms of a test.

- The TIMS CMI interface shall be modified to provide support for alternate forms of a test.

## 4.3.1.2.3.3  Verification of Navigator Detail:  TIMS CMI Interface and On-Line Test Modification

This requirement shall be verified using an operational CBTS TIMS environment to demonstrate the following for selected lessons and tests:  (1) the launch and termination of lessons and tests under TIMS Management, (2) TIMS Management of on-line tests CMI output data files, (3) TIMS Management of lesson completion data, (4) TIMS updates to the student grade book, (5) TIMS support of alternate test forms, and (6) on-line presentation of tests.

## 3.1.2.3.4  T-38A/C Detail:  TIMS Courseware Installation and On-Line Test Modification

This shall apply to the following T-38A/C courses:

- P-V4A-A:   T-38A Joint Specialized UPT Course

- P-V4A-P:   T-38A Advanced Training Program

- F-V5A-Q:  T-38A Fixed Wing Qualification

- F-V5A-B(R):  T-38A Pilot Instructor Requalification

- B/FV5A-K(C):  T-38C Introduction to Fighter Fundamentals

- B/FV5A-M(C):  T-38C Introduction to Fighter Fundamentals Instructor Training

- TBD:  T-38 A to C Transition Course at Introduction to Fighter Fundamentals

For the GFP T-38A courses (1-4) listed above, the following shall be accomplished:

- Using the JPATS/TIMS CMI launch and data collection methods, T-38A/C lesson executables will be mapped to TIMS syllabus events to launch the T-38A/C lessons and record completion information.

- Paper-based tests shall be converted to the JPATS test GUI.

- The JPATS test system shall be modified to provide support for alternate forms of a test.

- The TIMS CMI interface shall be modified to provide support for alternate forms of a test.

For the GFP T-38C courses (5-7) listed above, only item "a" above shall be accomplished.

## 4.3.1.2.3.4  Verification of T-38A/C Detail:  TIMS Courseware Installation and On-Line Test Modification

This requirement shall be verified using an operational CBTS TIMS environment to demonstrate the following for the T-38A/C courses:  (1) the launch and termination of lessons and tests under TIMS Management, (2) TIMS Management of CMI output data files, and (3) TIMS updates to the student grade book.  GFP courseware must conform to the JPATS CMI data collection method for updates to be accurate.  For T-38C courses, only item (1) applies.

## 3.1.2.3.5  Navy NAS Whiting/NAS Pensacola/NAS Corpus Christi Detail:  TIMS CMI Interface and On-Line Test Modification

This shall apply to the following Navy NAS Whiting/Pensacola/Corpus Christi courses:

1542.140A:  Multi-Service Pilot Training System

SCN 2

1542.93F:  Undergraduate Helicopter Flight Training Curriculum

SCN 2

- 1542.91E:  Undergraduate Helicopter Flight Instructor Under Training (IUT)

- 1542.53F:  Aerospace Medicine Specialist Indoctrination Curriculum

- 1542.41D:  Helicopter Transition Pilot Curriculum

1542.54K:  Primary Student NFO Training Curriculum

SCN 2

1542.65H:  Joint Advanced Multi-Engine (T-44A)

SCN 2

- 1542.138A:  Joint Advanced Multi-Engine TC-12 Flight Training

1542.73F:  Intermediate E2/C2 Training (T-44)

SCN 2

- 1542.66G:  Maritime Flight Instructor

- 1542.104:  Naval Test Pilot Prep/Maritime Transition (T-44/TC-12)

For these courses, the following shall be accomplished:

- The TIMS CMI interface shall be modified to incorporate the launch/completion mechanism of the NAS Whiting/NAS Pensacola/NAS Corpus Christi mini-CMI system.

- The TIMS CMI interface shall be modified to aggregate Whiting/Pensacola/Corpus Christi CMI data to TIMS.

- The JPATS test system shall be modified to provide support for alternate forms of a test.

- The TIMS CMI interface shall be modified to provide support for alternate forms of a test.

- Tests (paper and electronic) shall be converted to the JPATS test GUI.

## 4.3.1.2.3.5 Verification of Navy NAS Whiting/NAS Pensacola/NAS Corpus Christi Detail:  TIMS CMI Interface and On-Line Test Modification

This requirement shall be verified using an operational CBTS TIMS environment to demonstrate the following for selected lessons and tests:  (1) the launch and termination of lessons and tests under TIMS Management, (2) TIMS Management of CMI output data files, (3) TIMS updates to the student grade book, (4) TIMS support of alternate test forms, and (5) on-line presentation of tests.

## 3.1.2.3.6 Test-Only Courses Detail:  Test Modification

This shall apply to the following test only courses:

- 1542.32D: Primary/Intermediate NFO/NAV IUT Curriculum

1542.131B: Intermediate NFO/Navigator Training Curriculum

SCN 2

- 1542.134: Intermediate NFO/Navigator T-1A IUT Curriculum

1542.61H: Primary/Intermediate Maritime/Helo Flight Instructor Training Curriculum

SCN 2

For these test-only courses, the following shall be accomplished:

- Tests shall be converted to the JPATS test GUI.

- The JPATS test system shall be modified to provide support for alternate forms of a test.

- The TIMS CMI interface shall be modified to provide support for alternate forms of a test.

## 4.3.1.2.3.6 Verification of Test-Only Courses Detail:  Test Modification

This requirement shall be verified using an operational CBTS TIMS environment to demonstrate the following for selected tests:  (1) the launch and termination of tests under TIMS Management, (2) TIMS Management of CMI output data files, (3) TIMS updates to the student grade book, (4) TIMS support of alternate test forms, and (5) on-line presentation of tests.

## 3.1.2.4  Operational Support Segment

All components of the Operational Support Segment (OSS) shall be fully integrated to automate their operation and shall meet the requirements of the following subparagraphs.

## 4.3.1.2.4  Verification of Operational Support Segment

Verification of this requirement is accomplished in the following subparagraphs.

## 3.1.2.4.1  Computer Based Training System

The CBTS shall present CAI lessons to the student in both individual study and mediated interactive lectures (MIL).

## 4.3.1.2.4.1  Verification of Computer Based Training System

This requirement shall be verified by demonstration using actual course material.

## 3.1.2.4.2  Training Integration Management System

The TIMS shall integrate management, control, and administration of training to provide increased effectiveness and efficiency.  There shall be a single software version and a single (excluding the network) hardware architecture fielded for both the USAF and USN.  TIMS workstations shall be deployed at all bases listed in paragraph 3.1.

SCN 1

## 4.3.1.2.4.2  Verification of Training Integration Management System

This requirement shall be verified by analysis at design reviews, demonstration of TIMS operation during SLFE, and inspection and demonstration of TIMS hardware and software at each site.

## 3.1.2.4.3  Modification and Update Support System

One MUSS shall be located at Randolph AFB.  The other MUSS shall be located at NAS Corpus Christi.  Each MUSS shall consist of the following:  the Aircrew Training Device Support System (ATDSS), the Training Integration Management System Support System (TIMS-SS) and the CSS.  The ATDSS shall support all JPATS GBTS ATDs.  The CSS shall support Development and Standardized Courseware.  The TIMS Support System (TIMS-SS) shall support the JPATS GBTS TIMS, CBTS, and MUSS.

## 4.3.1.2.4.3  Verification of Modification and Update Support System

This requirement shall be verified by demonstration of the operation of the ATDSS, CSS, and TIMS-SS using actual course material.

## 3.1.2.5  TIMS Management Courses

TIMS Management Courses (Table 3.1.2.6) shall be managed by TIMS and supported by the CSS.

## 4.3.1.2.5  Verification of TIMS Management Courses

This requirement shall be verified by demonstration of TIMS operation using government selected course syllabus data.

TABLE 3.1.2.6  TIMS Management Courses (USAF/USN) (Page 1 of 3)

| Course ID | Title | USAF/ USN | C | S | Location |
| --- | --- | --- | --- | --- | --- |
| 1542.8H | Aviation Preflight Indoctrination Curriculum | USN | 26 | 77 | NAS Pensacola |
| 1542.16G SCN 2 | Academic Instructor Training Course | USN | 12 | 12 | NAS Pensacola |
| 1542.15E | Flight Instructor Training Course | USN | 24 | 9 | NAS Pensacola, NAS Corpus Christi, NAS Whiting |
| 1542.57C | Naval Test Pilot School Preparatory Curriculum | USN | 3 | 1 | NAS Meridian |
| 1542.117 | Aircrew Coordination Training (ACT) Instructor Curriculum | USN | 24 | 8 | NAS Pensacola |
| 1542.121C | Advanced NFO Strike Fighter Training | USN | 12 | 16 | NAS Pensacola |
| 1542.122B | Advanced NFO Strike Training Curriculum | USN | 12 | 18 | NAS Pensacola |
| 1542.123A | Advanced Military Flight Officer IUT T-39 Training | USN | 12 | 2 | NAS Pensacola |
| 1542.124 SCN 2 | Advanced NFO Instructor Pilot IUT Training Curriculum | USN | 6 | 1 | NAS Pensacola |
| 1542.56K | Intermediate Strike & Advanced E2/C2 Pilot Training Curriculum | USN | 24 | 12 | NAS Meridian |
| 1542.40D | Intermediate Strike & Advanced E2/C2 IUT training Curriculum | USN | 26 | 2 | NAS Kingsville |
| 1542.132 | Advanced NFO/Navigator Core Training Curriculum | USN | 24 | 23 | NAS Pensacola |
| 1542.54K SCN 2 | Primary Student NFO Training Curriculum | USN | 24 | 24 | NAS Pensacola |
| 1542.32D | Primary/Intermediate NFO/NAV IUT Curriculum | USN | 12 | 3 | NAS Pensacola |
| 1542.131B SCN 2 | Intermediate NFO/Navigator Training Curriculum | USN | 24 | 17 | NAS Pensacola |
| 1542.134 SCN 2 | Intermediate NFO/Navigator T-1A IUT Curriculum | USN | 7 | 1 | NAS Pensacola |
| 1542.65H | Joint Advanced Multi-Engine (T-44A) | USN | 10 | 3 | NAS Corpus Christi |
| 1542.61G | Primary/Intermediate Maritime/Helo Flight Instructor Training Curriculum | USN | 12 | 5 | NAS Whiting |
|  |  |  | 12 | 3 | NAS Corpus Christi |
| 1542.66G | Maritime Flight Instructor | USN | 15 | 20 | NAS Corpus Christi |
| 1542.73F SCN 2 | Intermediate E2/C2 Training (T-44) | USN | 23 | 2 | NAS Corpus Christi |
| 1542.104 | Naval Test Pilot Prep/Maritime Transition (T-44/TC-12) | USN | 15 | 1 | NAS Corpus Christi |
| 1542.138A | Joint Advanced Multi-Engine TC-12 Flight Training | USN | 10 | 11 | NAS Corpus Christi |
| 1542.140A SCN 2 | Multi-Service Pilot Training System | USN | 24 | 32 | NAS Whiting |
|  |  |  | 24 | 20 | NAS Corpus Christi |
| 1542.41D | Helicopter Transition Pilot Curriculum | USN | 10 | 1 | NAS Whiting |

TABLE 3.1.2.6  TIMS Management Courses (USAF/USN) (Page 2 of 3)

| Course ID | Title | USAF/ USN | C | S | Location |
| --- | --- | --- | --- | --- | --- |
| 1542.53F | Aerospace Medicine Specialist Indoctrination Curriculum | USN | 12 | 1 | NAS Whiting |
| 1542.91E SCN 2 | Undergraduate helicopter Flight IUT | USN | 12 | 4 | NAS Whiting |
| 1542.93F | Undergraduate helicopter Flight Training | USN | 26 | 21 | NAS Whiting |
| 1542.118 SCN 2 | T-45A TS Strike Designated Naval Aviator Jet Refresher/NATOPS | USN | 12 | 5 | NAS Kingsville |
| 1542.108D | T-45A TS Strike Flight Training Curriculum | USN | 12 | 15 | NAS Kingsville |
| 1542.109B | T-45A TS Strike Flight Instructor training Curriculum | USN | 13 | 2 | NAS Kingsville |
| 1542.120B | T-45A Advanced Strike Flight Training | USN | 20 | 15 | NAS Kingsville |
| 1542.127 | T-45C TS Strike Flight Instructor Training | USN | 12 | 2 | NAS Meridian |
| 1542.128A SCN 2 | T-45C TS Advanced Strike Flight Training | USN | 24 | 7 | NAS Meridian |
| 1542.135 | T-45A Advanced E2/C2 Flight Training | USN | 12 | 4 | NAS Kingsville |
| 1542.139 | T-45A Transition Strike IUT | USN | ? | ? | NAS Kingsville |
| B/F-V5A-K | Introduction to Fighter Fundamentals (T-38A/B) | USAF | 15 | 15 | Sheppard AFB, Columbus AFB |
| B/F-V5A-M | Introduction to Fighter Fundamentals Instructor Training (T-38 A/B) | USAF | * | * | Randolph AFB (25 Students/ yr) |
| F-V5A-A | T-37 Pilot Instructor Training (PIT) | USAF | 12 | 24 | Randolph AFB |
| F-V5A-A(CC) | T-37 Senior Officer Qualification | USAF |  |  | Randolph AFB |
| F-V5A-A(R) | T-37 Pilot Instructor Requalification | USAF | 10 | 1 | Randolph AFB |
| F-V5A-B | T-38A Pilot Instructor Training (PIT) | USAF | 10 | 11 | Randolph AFB |
| F-V5A-CC | T-1A Senior Officer Qualification | USAF |  |  | Randolph AFB |
| F-V5A-G | Crossflow (T-38 course) | USAF |  |  | Laughlin AFB, Vance AFB, Columbus AFB, Sheppard AFB, Randolph AFB |
| F-V5N-A/B | T-37/T-38 ENJJPT PIT w/Ch 1 | USAF | 6 | 6 | Sheppard AFB |
| P-V4A-N | T-37/T-38 ENJJPT Primary Student w/Ch1, Ch 2 | USAF | 8 | 45 | Sheppard AFB |
| V-V4-AI | T-1A Airmanship | USAF |  |  | Randolph AFB |
| F-V5A-CC (T-38) | T-38 Senior Officer Qualification | USAF |  |  | Randolph AFB |
| F-V5A-R | T-38 Transition | USAF |  |  | Randolph AFB |
| B/F-V5A-I | Basic Fighter Introduction | USAF |  |  |  |
| B/F-V5A-K w/Ch 1 | ENJJPT Intro to Fighter Fundamentals | USAF | 15 | 15 | Sheppard AFB |
| F-V5A-E(R) | T-1A Pilot Instructor Requalification | USAF |  |  |  |

TABLE 3.1.2.6  TIMS Management Courses (USAF/USN) (Page 3 of 3)

| Course ID | Title | USAF/ USN | C | S | Location |
| --- | --- | --- | --- | --- | --- |
| N-V4R-N-3 | Electronic Warfare Principles | USAF | 15 | 11 | Randolph AFB |
| B-V4E-W | Electronic Warfare Officer training | USAF | 15 | 4 | Randolph AFB |
| B-V5E-W | Electronic Warfare Upgrade | USAF | 15 | 4 | Randolph AFB |
| B-V7C-E | Electronic Warfare Fundamentals & Applications (Split out from N-EG-EW) | USAF | 8 | 22 | Randolph AFB |
| S-V8E-S | Electronic Warfare Coordinator | USAF | 4 | 12 | Randolph AFB |
| F-V5A-E | T-1A Pilot Instructor Training (PIT) | USAF | 11 | 10 | Randolph AFB |
| N-V4R-N-2 | Airlift/Tanker/Maritime/Navigator/ NFO | USAF |  |  | Randolph AFB |
| N-V4R-N-1 | Primary Navigator/NFO | USAF | 12 | 13 | Randolph AFB |
| N-V4R-N-4 | Theater Operations | USAF | 12 | 13 | Randolph AFB |
| N-V9-IT | Navigator/NFO/EWO Instructor Training | USAF |  |  | Randolph AFB |
| P-V4A-G | T-1A Specialized Undergraduate Pilot Training | USAF | 15 | 16 | Laughlin AFB, Vance AFB, Columbus AFB |
| F-V5A-Q | T-1A Fixed Wing Transition | USAF | 15 | 16 | Laughlin AFB, Vance AFB, Columbus AFB |
| S-V8E-A/B | ## T-37 Medical/Physiological Officer Flight Familiarization Training | USAF | ## * | ## * | ## Randolph AFB (7 Students/ yr) |
| N-V5-CN | Celestial Navigation Course | USAF |  |  | Randolph AFB |
| P-V4A-A | T-37B Joint Specialized UPT | USAF | 15 | 29 | Laughlin AFB, Vance AFB |
|  |  |  | 15 | 27 | Columbus AFB, Sheppard AFB |
| P-V4A-E (TRIM) | T-37B Aviation Leadership Program (ALP) | USAF | 6 | 4 | Laughlin AFB |
| F-V5A-W (TRIM) | T-37B USN Pre-Qualification Course | USAF |  |  | Vance AFB |
| F-V5A-Q (TRIM) | T-37B Fixed Wing Qualification | USAF | 4 | 2 | Vance AFB |
| P-V4A-A | T-38A Joint Specialized UPT Course | USAF | 15 | 9 | Laughlin AFB |
|  |  |  | 15 | 8 | Vance AFB |
|  |  |  | 15 | 8 | Columbus AFB |
| P-V4A-P | T-38A Advanced Training Program | USAF | 4 | 2 | Laughlin AFB, Columbus AFB |
| F-V5A-Q | T-38A Fixed Wing Qualification | USAF | 2 | 4 | Vance AFB |
| F-V5A-B(R) | T-38A Pilot Instructor Requalification | USAF |  |  | Randolph AFB |
| B/FV5A-K(C) | T-38C Introduction to Fighter Fundamentals | USAF | 15 | 15 | Moody AFB |
| B/FV5A-M(C) | T-38C Introduction to Fighter Fundamentals Instructor Training | USAF |  |  | Randolph AFB (25 Students/ yr) |
| TBD | T-38 A to C Transition Course at Introduction to Fighter Fundamentals | USAF |  |  | Randolph AFB |

* See location column for student throughput.

# 3.2  System-Level Requirements for Development Courses

For each development course, the GBTS shall integrate all GBTS Segments, Components, and Elements as well as existing non-specific course media (e.g., altitude chamber) into comprehensive and effective courses of instruction.  The courses shall progressively train the objectives in a building block approach allowing each new area of instruction to build upon the previous areas.  The GBTS shall require students to demonstrate their comprehension, retention, and task proficiency.  Training shall be tailored to the specific site at which it is conducted.  There shall be a consistent presentation format, style and philosophy in the use of training media across all courses.  Unless otherwise specified, all courses will be conducted by government supplied personnel.

## 4.3.2  Verification of System Level Requirements for Development Courses

This requirement shall be verified by demonstration of actual course material.

## 3.2.1  Syllabus

For each course, a syllabus shall prescribe the overall training strategy and the approximate amount of instruction necessary for a student having the entry prerequisites to attain the course goals and to graduate.  The syllabus shall be objective based.  The syllabus shall allocate objectives to blocks of instruction and lessons.  Lessons shall be allocated to specific media.  Training events shall maximize retention and minimize loss of proficiency, especially when mastery has not been confirmed or when transitioning from simulated flight to actual flight in the aircraft.  The syllabus shall prescribe remediation for unsatisfactory progress and as required by operational training regulations.  The syllabus shall contain course flow, lesson duration, and course training standards for all areas of training.  Procedures unique to specific training bases shall be contained in as few lessons as possible.  In these cases there shall be a unique version of the lesson for each training base.

## 4.3.2.1  Verification of Syllabus

This requirement shall be verified by analysis at design reviews, SME review of the Lesson Design and Strategy Report (LDSR) and lesson content, and satisfactory completion of the Individual Tryouts (ITOs) and Small Group Tryouts (SGTOs).

## 3.2.2  Qualification Requirements

The qualification levels for each course shall meet the required levels of performance defined in the syllabus.

## 4.3.2.2  Verification of Qualification Requirements

This requirement shall be verified by analysis of the courses at design reviews and inspection of student performance reports and data using actual course material.

## 3.2.2.1  Performance Evaluation

Each course shall evaluate and critique student performance against criterion specified in each training objective.  The criterion shall be quantitative.  Evaluation materials shall identify, diagnose, and explain student strengths, weaknesses, and errors.

## 4.3.2.2.1  Verification of Performance Evaluation

This requirement shall be verified by analysis and demonstration.  Analysis at design reviews shall verify performance measurement criteria for each course.  These criteria shall be demonstrated for the JPPT Student Course during SLFE.

SCN 1

3.2.3  Principal Courses (Flight Training)

SCN 1

Each development course shall qualify students in all Instructional Systems Development (ISD) derived primary aircraft operating tasks (knowledge, skills, and attitudes).  Each course shall integrate three areas of training: academics, simulated flight training, and flight training.  Comprehension, retention, and task proficiency shall be demonstrated in controlled, realistic practice situations for normal, abnormal, and emergency conditions.  Courses shall teach each subject area in academic instruction, then in lower level training devices, then high level training devices, and finally in the aircraft.  Those tasks which cannot be safely performed or evaluated in the JPATS aircraft shall be trained in ATDs.  All tasks which can be safely performed in the JPATS aircraft shall be evaluated and result in student qualification in the aircraft.  If full simulated conditions are not possible, performance measurement of subordinate competencies or of some performance known to correlate with the objective shall be substituted.  Each syllabus shall provide scheduled time for student briefings prior to, and debriefings after, each session in the aircraft and ATDs.

4.3.2.3  Verification of Principal Courses (Flight Training)

SCN 1

This requirement shall be verified by analysis at design reviews and demonstration of flight training using actual courseware.

## 3.2.3.1  Simulated Flight Environment

Except for low level navigation routes, FTD lessons shall occur in a simulated environment representing the local area.  For each course, the simulated environment shall replicate the environments and areas where the actual flight training for the course occurs.  The simulated environment shall represent the visual, navigation aid, and radio communication characteristics of that environment.

## 4.3.2.3.1  Verification of Simulated Flight Environment

This requirement shall be verified by analysis, and demonstration.  Analysis at design reviews shall correlate FTD visual, navigation aid, and radio communication characteristics to the FTD lessons for each course.  The FTD simulated flight environment for the local area shall be demonstrated using missions conducted during other FTD testing required by this specification.

## 3.2.3.2  Local Area Procedures

Local area procedures (i.e., those which must be trained in the context of a specific training base) shall be trained in the GBTS.

## 4.3.2.3.2  Verification of Local Area Procedures

This requirement shall be verified by analysis, and demonstration.  Analysis at design reviews shall verify inclusion of correct local area procedures for each course.  Local area procedures shall be demonstrated during SLFE and other testing required by this specification.

## 3.2.4  Training Traceability

Each training objective shall be traceable as required by paragraph 3.8.4.3.2.1.

## 4.3.2.4  Verification of Training Traceability

This requirement shall be verified by a demonstration which traces selected objectives for each course.

## 3.2.5  Course Constraints

## 4.3.2.5  Verification of Course Constraints

Verification of this paragraph is not applicable.

## 3.2.5.1  Flying Hours

The number of aircraft flight hours per course per student shall not be changed from those prescribed by the government.

## 4.3.2.5.1  Verification of Flying Hours

This requirement shall be verified by analysis at design reviews.

## 3.2.5.2  Duty Day

A normal student week shall be five days.  Maximum academic training shall be seven hours per day per student.  The average number of hours a student can perform training activities shall be between 8 to 10 hours per day; the maximum shall be 12 hours per day.  Minimum crew rest (not performing job related duties) for students, trainees, and instructors is 12 hours.

## 4.3.2.5.2  Verification of Duty Day

This requirement shall be verified by analysis at design reviews.

# 3.3  Reserved

Reserved

## 4.3.3  Reserved

# 3.4  System Design and Construction

Each GBTS Component shall meet the requirements of the following subparagraphs.

## 4.3.4  Verification of System Design and Construction

Verification of this paragraph is not applicable.

## 3.4.1  Operating Life

Each ATD shall operate for a minimum of 105,600 hours, or 24 years (whichever occurs first) given normal recurring maintenance.

## 4.3.4.1  Verification of Operating Life

This requirement shall be verified by analysis.  Analysis at design reviews shall be based on comparison of ATD predicted component and system life-cycle performance with historic operational life-cycle performance data from comparable systems.

## 3.4.2  Environmental Conditions

All GBTS Components shall fully operate within the ambient air environmental conditions listed below:

- Environmental control equipment shall be incorporated as part of each GBTS Component if necessary to meet the operational environment conditions.

- OFT and IFT:  AETC simulator bay environment (59 Deg F to 89 Deg F; 20% to 80% non-condensing relative humidity).

- UTD, EST, and EPT:  AETC/CNATRA flight room environment (32 Deg F to 95 Deg F; 20% to 80% non-condensing relative humidity).

- TIMS and CBTS:  AETC/CNATRA flight room environment (No facility air conditioning available).

- CSS, ATDSS, TIMS-SS:  standard business office environment.

- All GBTS Components shall operate in accordance with this specification after storage in the following environment:

- Temperature:  -9°F to +140°F (degrees Fahrenheit).

- Relative Humidity:  up to 95% (Non-condensing).

- Barometric Pressure:  27.0 to 31.35 inches of Hg.

- Personnel Enclosures:  The temperature and relative humidity inside any enclosed instructor or student compartment in the GBTS Components shall be controlled and adjustable to maintain a summer comfort zone.  All personnel enclosures shall be ventilated.

## 4.3.4.2  Verification of Environmental Conditions

This requirement shall be verified by analysis and demonstration.  The analysis at design reviews shall confirm compliance with environmental requirements for operation and storage.  Personnel enclosure environmental conditions shall be demonstrated during other testing required by this specification.

## 3.4.3  Human Engineering

The GBTS work environment shall foster effective procedures and work patterns and minimize factors which degrade human performance or increase error.  The requirements of this section shall apply except as necessary to match aircraft design and performance.

## 4.3.4.3  Verification of Human Engineering

This requirement shall be verified by analysis and demonstration.  Analysis at design reviews shall correlate GBTS component design to required human engineering characteristics.  Demonstrations of system operation under normal operating conditions shall be conducted during other testing required by this specification to verify compliance with human engineering requirements.

## 3.4.3.1  Human Design and Operation

The GBTS Components shall use as a guide MIL-STD-1472, sections 4.1 through 4.9, and 5.2 through 5.5, for the human design and operation requirements and equipment selection and design.  The selection, design and layout of computer terminal workstations shall minimize the risk of cumulative trauma disorders.  ANSI/HFS 100 shall be used as a guide for computer terminals.

## 4.3.4.3.1  Verification of Human Design and Operation

This requirement shall be verified by analysis at design reviews.

## 3.4.3.2  Controls and Displays

Display systems, other than those used to simulate the aircraft cockpit, shall respond to or acknowledge control inputs as defined within this paragraph.  Except as otherwise necessary to meet the requirements of this specification, the time between a control action and its associated feedback response (e.g., the time between an event occurring and when a display is changed) shall not exceed 200 milliseconds.  When an activity initiated by a control precludes other actions, an appropriate cue shall be provided on the display.  Unless otherwise specified, all controls which are integrated into a display system shall be available within 5 seconds time whenever required by a display operator.

## 4.3.4.3.2  Verification of Controls and Displays

This requirement shall be verified by analysis and demonstration.  Analysis at design reviews shall correlate GBTS component design and parts selection to requirements.  The analysis shall include data from vendors of component parts of the GBTS to confirm compliance with control and display system requirements.  Demonstrations to verify control and display compliance with these requirements shall be conducted during other testing required by this specification.

## 3.4.3.3  Human-Computer Interface

Human-computer interfaces shall be windowed, graphics based and easily comprehended by operators (e.g., point and click, drag and drop).  Interfaces shall be tailored for each class of operator.  The interface shall allow easy reconfiguration of displays, labels, etc., by the various users without recompilation of software.  These interfaces shall check for validity of all inputs.  The system shall tolerate invalid inputs.  Upon detection of an invalid input, the operator shall be immediately signaled to correct the input.  Correction of an error shall not require re-entering data (or portion thereof) that was correctly input originally (for typed entries editing the data shall be allowed).  The interface shall also provide context-sensitive on-line help and on-line user operating manuals.  Information shall be presented in a consistent format.  The human-computer interface shall use native application interfaces.  Neither emulation nor simulation shall be used.

## 4.3.4.3.3  Verification of Human-Computer Interface

This requirement shall be verified by analysis at design review and demonstration during other verification activities required by this specification.

## 3.4.3.4  Ambient Noise

Each GBTS Component's acoustic environment shall neither cause fatigue nor interfere with voice or other communications.

## 4.3.4.3.4  Verification of Ambient Noise

This requirement shall be verified by demonstration.  Ambient noise performance under normal operating conditions shall be demonstrated using Demonstration Missions conducted during other GBTS testing required by this specification.

## 3.4.4  Power System

Each GBTS Component shall neither require nor use any form of power other than electrical.  Only electrical power shall be required from the facility.  Any backup power associated with the GBTS shall have a switch to permit shut-off in the event of planned power shutdown.  All other forms of power shall be generated within the GBTS.

## 4.3.4.4  Verification of Power System

This requirement shall be verified by analysis and inspection.  Analysis at design review shall verify the use of only electrical power in the GBTS design.  Inspection shall verify the shut-off switch for any backup power source.

## 3.4.4.1  Electromagnetic Interference and Electromagnetic Compatibility

The GBTS shall not electromagnetically interfere with itself or any other electronic equipment at the same base.  The GBTS shall resist electromagnetic interference from other electronic equipment.

## 4.3.4.4.1  Verification of Electromagnetic Interference and Electromagnetic Compatibility

This requirement shall be verified by analysis and demonstration.  Analysis at design reviews and demonstrations of component operation in the installed environment for all GBTS components shall show no adverse effects due to the electromagnetic environment.

## 3.4.4.2  Power Interruption and Transients

Power spikes, transients, and power interruptions shall not damage the GBTS.  In the event of an unplanned power interruption, except those initiated by Emergency Power-Off, the GBTS shall provide sufficient power to automatically shutdown each Component and save all data.  All GBTS components, except ATDs, shall remain fully operational for a minimum of 5 minutes before shutdown.  The GBTS shall remain fully operational during the following degraded power conditions:

- An over-voltage transient of three and one-half (3.5) times normal peak voltage, for impulses up to one cycle in duration.

- An under-voltage of less than one-tenth (0.1) of normal RMS voltage but of a minimum of 1 cycle duration.

## 4.3.4.4.2  Verification of Power Interruption and Transients

This requirement shall be verified by analysis at design reviews.

## 3.4.5  Workmanship

No dirt or blemishes shall be visible on the GBTS.

## 4.3.4.5  Verification of Workmanship

This requirement shall be verified by inspection of GBTS components during acceptance.

## 3.4.5.1  Protective Coatings

Surfaces subject to wear, abrasion, or accidental liquid spills (e.g.,  students' beverages) shall be laminated with a protective coating to prevent equipment damage.

## 4.3.4.5.1  Verification of Protective Coatings

This requirement shall be verified by analysis during design reviews and inspection during acceptance.

## 3.4.6  Reserved

## 4.3.4.6  Reserved

## 3.4.7  Safety

The GBTS shall conform to the health and safety requirements of the following documents to minimize and control hazards to personnel and equipment during use and maintenance of the GBTS:

- AFOSH 127-118/T except for paragraph 6.a.(5)(a)

- AFOSH 127-66

- NFPA-70

## 4.3.4.7  Verification of Safety

This requirement shall be verified by analysis.  The analysis shall correlate GBTS component design and parts selection to this requirement.  The analysis shall include data from vendors of component parts of the GBTS to confirm compliance with safety requirements.

## 3.4.7.1  Hazardous Materials

Hazardous material is a material which, because of the material's physical, chemical, or biological characteristics, quantity, or concentration may, during the GBTS life cycle (development, testing, manufacture, operation, maintenance, and disposal):

- cause or contribute to adverse effects in organisms or offspring, or

- pose a substantial present or future danger to the environment, or

- result in damage or loss of equipment or property.

## 4.3.4.7.1  Verification of Hazardous Materials

Verification of this paragraph is not applicable.

## 3.4.7.1.1  Restricted Hazardous Materials

The following hazardous materials shall not be contained in the GBTS, including commercial off-the-shelf items, and shall not be used in the operation or support of the GBTS:

- Asbestos

- Carbon Tetrachloride

- Tetrachloromethane

- CFCs -11, 12, 13, 111, 112, 113, 114, 115, 211, 212, 213, 214, 215, 216, and 217

- Halons - 1011, 1202, 1211, 1301, 2402

- Hydrazine

- Methyl Bromide

- Methyl Chloroform

- Polychlorinated Biphenyls (PCBs Trichloroethane)

## 4.3.4.7.1.1  Verification of Restricted Hazardous Materials

This requirement shall be verified by analysis.  The analysis shall correlate GBTS component design and parts selection to this requirement.  The analysis shall include data from vendors of component parts of the GBTS to confirm compliance with hazardous material requirements.

## 3.4.7.1.2  Targeted Hazardous Materials

The following hazardous materials shall be reduced, minimized, or eliminated in the GBTS, including commercial off-the-shelf items, and in the operation and support of the GBTS:

- Benzene

- Cadmium and compounds

- Chloroform

- Chromium and compounds

- Cyanides (CN)

- Dichloromethane (DCM)

- Formaldehyde

- Isocyanates (all isomers)

- Lead and compounds

- Mercury and compounds

- Methylene Chloride

- Methylene Dianiline (MDA)

- Methyl Ethyl Ketone (MEK)

- Methyl Isobutyl Ketone (MIBK)

- Nickel and compounds

- PD-680

- Perchloroethylene

- Phenols

- Tetrachloroethylene

- Toluene

- Trichloroethylene (TCE)

- Xylene(s)

## 4.3.4.7.1.2  Verification of Targeted Hazardous Materials

This requirement shall be verified by analysis.  The analysis shall correlate GBTS component design and parts selection to this requirement.  The analysis shall include data from vendors of component parts of the GBTS to confirm compliance with hazardous material requirements.

## 3.4.8  Nameplates and Product Marking

- A durable nameplate shall be securely fastened to each GBTS Component.  The nameplate shall indicate the manufacturer, nomenclature, serial number, and be labeled "Property of U.S. Government".

- All safety hazards shall be provided with adequate and conspicuous warning signs.

- Modified or rejected JPATS aircraft equipment used in the GBTS shall be durably marked with the following precautionary marking:  "WARNING:  FOR GROUND TRAINING USE ONLY".  The marking shall be in a plainly visible position.

## 4.3.4.8  Verification of Nameplates and Product Marking

This requirement shall be verified by inspection.

## 3.4.9  System Security and Training Integrity

Sensitive, but unclassified, information shall be protected in accordance with applicable DOD security guidelines.  The GBTS shall have user configurable access codes, keys, or locks to prevent unauthorized access to training equipment and data records.  The GBTS shall provide controls to limit or prevent introduction of unauthorized files and software based on user privileges.  Commercial Off the Shelf (COTS) virus-detection software shall be installed to detect computer viruses.

## 4.3.4.9  Verification of System Security and Training Integrity

This requirement shall be verified by analysis and demonstration.  Analysis at design reviews shall verify GBTS security and data integrity requirements during system operation.  Security system operation shall be demonstrated during other testing required by this specification.

## 3.4.10  Computation Systems

The computation systems (i.e., all computer hardware, software, and communication services required to operate and support the GBTS) shall use an open third party expandable architecture.

## 4.3.4.10  Verification of Computation Systems

This requirement shall be verified by analysis at design reviews.  The analysis shall include data from vendors of component parts to confirm compliance.

## 3.4.10.1  Computation System Hardware

COTS computer hardware system(s) shall be used in the GBTS.

## 4.3.4.10.1 Verification of Computation System Hardware

This requirement shall be verified by analysis at design reviews.  The analysis shall include data from vendors of component parts to confirm compliance.

## 3.4.10.1.1  PC Energy Efficiency

All COTS personal computers (PC), PC monitors, and printers shall be compliant with EPA Energy Star requirements.

## 4.3.4.10.1.1  Verification of PC Energy Efficiency

The requirement shall be verified by inspection of documentation from COTS vendors.

## 3.4.10.2  Computation System Software

Operating system software shall be COTS.

## 4.3.4.10.2  Verification of Computation System Software

This requirement shall be verified by analysis at design reviews.  The analysis shall include data from vendors of component parts to confirm compliance.

## 3.4.10.2.1  Computer Programming Language

Use of COTS Software shall be maximized.  Use of Structured Query Language (SQL) shall be in accordance with ANSI/STD.X3.135-1992.  All software shall comply with specification requirements.

## 4.3.4.10.2.1  Verification of Computer Programming Language

This requirement shall be verified by analysis at design reviews.

## 3.4.10.2.2  Design and Coding Constraints

Software shall be robust and fault tolerant.  All development software shall be object oriented.  The software shall be modular with minimal dependence of the operation of one module on the internal details of other modules.  Items subject to frequent change shall be modifiable without source code changes or recompilation.  COTS software shall not be modified.

## 4.3.4.10.2.2  Verification of Design and Coding Constraints

This requirement shall be verified by analysis at design reviews.

## 3.4.10.2.3  Software Portability

With the exception of the image generator software, all software shall function correctly on two different-vendor hardware platforms without any modifications to source code for the TIMS, CBTS, MUSS, and the FTDs.  Recompilation of the unmodified source code for the new target processors will be allowed.  All OSS functions shall be available from any GBTS workstation on the network provided the required hardware is installed and subject to access permissions.

## 4.3.4.10.2.3  Verification of Software

This requirement shall be verified by analysis at design reviews and, for TIMS only, test of two different-vendor hardware platforms.

## 3.4.10.3  Computation System Spare and Growth

Office automation functions shall utilize spare except for the space required to install the office automation software.

- The system shall permit all courses listed in Tables 3.1.2.3, 3.1.2.4, and 3.1.2.6 to be expanded to fully objective-based syllabuses and be fully supported by the OSS without adding any additional hardware.

- All spare shall be useable without modification to software (i.e., the amount of data stored shall be limited only by available storage and not by the software architecture).

- All growth shall be useable without recompilation of software.

- The OSS shall allow a doubling of the managed information beyond the maximum required by this specification without adding hardware or changing the architecture.

- Except as otherwise specified a nominal spare of 50% is required without adding hardware.  TIMS workstations shall be provided with a nominal hard disk space spare of 55%.

## 4.3.4.10.3 Verification of Computation System Spare and Growth

This requirement shall be verified by analysis, test, and demonstration.  Analysis at design reviews shall correlate GBTS component design and parts selection to computational system spare and growth requirements.  The analysis shall include data from vendors of component parts of the GBTS to confirm compliance with these requirements.  Tests shall measure FTD computational system spare under normal operating conditions to verify compliance with computational system spare requirements.  Analysis shall verify compliance with OSS computational system and network spare requirements.  The use of spare without modification of software shall be verified by demonstration during other testing required by this specification.

## 3.4.10.4  Interoperability

The TIMS and MUSS shall comply with the JPATS GBTS Joint Interoperability Profile.

## 4.3.4.10.4  Verification of Interoperability

This requirement shall be verified by analysis at design reviews.

## 3.4.11  Reserved

## 4.3.4.11  Reserved

## 3.4.12  Mission Effectiveness

## 4.3.4.12  Verification of Mission Effectiveness

The verification of this paragraph is not applicable.

## 3.4.12.1  ATD Mission Effectiveness

ATD mission effectiveness shall be 95% per month for each ATD type at each base when the ATD(s) are used up to 16 hours per day, six days per week, 52 weeks per year.  ATD mission effectiveness shall be computed as follows:

Mission Effectiveness % = 		Completed Missions X 100_____

Scheduled Missions - Censored Missions

Censored missions shall be defined as scheduled missions that are not used or that are lost due to reasons outside contractor’s control as mutually agreed to by the contractor and the Government.  Adjustment for censored time shall be made for missions lost due to power outages, facility environmental problems, weather alerts, or other conditions.  This shall include mission cancellations and no shows that are not reflected on the schedule.  Determination of whether a scheduled mission has been completed shall be at the discretion of the instructor.

## 4.3.4.12.1  Verification of ATD Mission Effectiveness

This requirement shall be verified by analysis and inspection.  The Mission Effectiveness Analysis at design reviews shall correlate ATD component design and parts selection to this requirement.  The analysis shall include data from vendors of component parts of the GBTS to develop reliability, maintainability, and resultant availability performance assessment for the ATDs.  Inspection of records of ATD training mission performance during SLFE, and analysis using the required mission effectiveness formula shall verify compliance with this requirement.

## 3.4.12.2  CBTS Mission Effectiveness

CBTS mission effectiveness shall be 98% per month at each base when the CBTS is used up to 24 hours per day, seven days per week, 52 weeks per year.  CBTS mission effectiveness shall be computed as follows:

Mission Effectiveness  % = 	                 Completed Lessons X 100

Total Lessons (Completed and lost) - Censored Lessons

Censored lessons shall be defined as lessons lost due to reasons outside the contractor's control as mutually agreed to by the contractor and the Government.  Adjustment for censored lessons shall be made for lessons not completed due to power outages, facility environmental problems, weather alerts, or other conditions outside the contractor's control.

Completed lessons shall be defined as  (1) Lessons scheduled through the TIMS and (2) Unscheduled lessons initiated by the student during normal operating hours that are completed by the student in accordance with the syllabus requirements.  Lost lessons shall be defined as:  (1) Scheduled and unscheduled lessons not available due to system or equipment failure and (2) Scheduled and unscheduled lessons interrupted for a period of 15 minutes or longer due to system or equipment failure.

## 4.3.4.12.2  Verification of CBTS Mission Effectiveness

This requirement shall be verified by analysis and inspection.  The Mission Effectiveness Analysis at design reviews shall correlate CBTS component design and parts selection to this requirement.  Inspection of records of CBTS availability data during SLFE and analysis using the required mission effectiveness formula shall verify compliance with this requirement.

## 3.4.12.3  TIMS Mission Effectiveness

TIMS mission effectiveness shall be 98%, with 99% for data critical to daily flying operations and safety of flight, per month at each base when the TIMS is used up to 24 hours per day, seven days per week, 52 weeks per year.  TIMS mission effectiveness shall be computed as follows:

Mission Effectiveness % = 	   Operational Hours X 100___

Total Hours - Censored Hours

Operational hours shall be defined as the total hours (number of TIMS workstations times 24 hours per day times the number of days in each month) minus the monthly number of non-operational workstation hours (measured in 15 minute increments).  Censored hours shall be workstation hours lost due to reasons outside the contractor's control as mutually agreed to by the contractor and the Government.  Adjustment for censored workstation hours shall be made for operational down time due to power outages, facility environmental problems, weather alerts, hosted software, or other conditions outside the contractor’s control.

## 4.3.4.12.3  Verification of TIMS Mission Effectiveness

This requirement shall be verified by analysis and inspection.  The Mission Effectiveness Analysis at design reviews shall correlate TIMS component design and parts selection to this requirement.  Inspection of records of TIMS availability data during SLFE and analysis using the required mission effectiveness formula shall verify compliance with this requirement.

## 3.4.13  Facilities

Training, operation, and maintenance of the system shall be accomplished through maximum use of existing Navy and Air Force facilities.

## 4.3.4.13  Verification of Facilities

This requirement shall be verified by analysis at design reviews.

## 3.4.14  Daily Readiness Test

A daily readiness test for each GBTS hardware Component shall determine if the Component is fully operational and provide indications for corrective maintenance.  The daily readiness test shall not require more than 10 minutes to complete.

## 4.3.4.14  Verification of Daily Readiness Test

This requirement shall be verified by demonstration.  The demonstration shall cover all normal operational modes of each GBTS hardware component.  A Daily Readiness Test for each GBTS hardware component shall be demonstrated during other testing required by this specification.

# 3.5  Documentation and Data Repository

The GBTS shall contain a master technical data library and GBTS on-site libraries for all technical and design documentation and data.

## 4.3.5  Verification of Documentation and Data Repository

This requirement shall be verified by inspection.

## 3.5.1  Master Technical Data Library

As a minimum, the library shall contain the following data:

- Operating Procedures for GBTS components

- User's Manuals

- GBTS drawings, development and product specifications

- GBTS design documentation

- GBTS software and firmware on magnetic media

- GBTS facilities drawings

- COTS documentation

- Relevant MIL and DOD standards

- Relevant USAF and USN Regulations

- Relevant AETC and CNATRA Regulations

- Relevant (both JPATS aircraft and GBTS) technical manuals

- Developed GBTS databases

- Relevant JPATS aircraft data, drawings and specifications (i.e., all data used for GBTS development)

- Development Courseware

- Standardized Courseware

- GBTS Technical Data Package

- GBTS Indentured Data List

- GBTS Technical Publications

- Support Equipment List

- Spare Parts List

## 4.3.5.1  Verification of Master Technical Data Library

This requirement shall be verified by inspection of the Master Technical Data Library at both the USAF and USN MUSS’s .

## 3.5.2  GBTS On-Site Libraries

Each base shall have a GBTS on-site library.  Each library shall contain all technical data required for the operation, maintenance, management, and support at that base.

## 4.3.5.2  Verification of GBTS On Site Libraries

This requirement shall be verified by inspection of the GBTS On-Site Libraries at each base.

# 3.6  Manpower, Personnel and Training

The amount of manpower required to operate the GBTS as a whole and for each course of instruction shall not exceed the manpower required to operate the existing systems and courses that the GBTS is replacing.

## 4.3.6  Verification of Manpower, Personnel and Training

This requirement shall be verified by analysis at design reviews.

# 3.7  Logistics

The GBTS shall have logistics support sufficient to achieve and maintain all requirements specified herein.  The GBTS shall be maintained by total contractor logistics support.  Support equipment shall not require the inclusion of special tools or test equipment.

## 4.3.7  Verification of Logistics

This requirement shall be verified by inspection.  Inspection of mission effectiveness records of the GBTS during System Level Formative Evaluation shall verify that logistic support is sufficient to meet the stated requirement.

# 3.8  Segment and Component Detailed Characteristics

## 4.3.8  Verification of Segment and Component Detailed Characteristics

Verification of this paragraph is not applicable.

## 3.8.1  Aircrew Training Device Segment

## 4.3.8.1  Verification of Aircrew Training Device Segment

Verification of this paragraph is not applicable.

## 3.8.1.1  Flight Training Devices

The OFT, IFT, and UTD shall be assembled from elements shown in Table 3.8.2.1.  Each FTD shall electronically interface with the Operational Support Segment to effectively automate the conduct, administration, and support of FTD training.

TABLE 3.8.2.1   FTD Element Definition

| Elements | Components |  |  |
| --- | --- | --- | --- |
|  | OFT | IFT | UTD |
| Central Training Complex | X | X | X |
| Instructor Operator Station | X | X | X |
| Remote Instructor Station | X | X | X |
| Canopy Assembly | X | X |  |
| Image Generator | X | X |  |
| Wide FOV Visual | X |  |  |
| Narrow FOV Visual |  | X |  |

## 4.3.8.1.1  Verification of Flight Training Devices

This requirement shall be verified by inspection and demonstration.  Inspection of the OFT, IFT, and UTD shall be conducted to verify that each device includes the required FTD elements.  The operational interfaces between FTDs and the Operational Support Segment shall be demonstrated during other testing required by this specification.

## 3.8.1.1.1  FTD Components

## 4.3.8.1.1.1 Verification of FTD Components

Verification of this paragraph is not applicable.

## 3.8.1.1.1.1  Operational Flight Trainer

The OFT shall train in visual flight for areas such as taxi, take off, landing, circling overhead patterns, formation, visual navigation, and visual emergencies.  The OFT shall support circling approaches and closed patterns.  The OFT shall present day and night visual scenes within weather, above weather, and below weather.  Simulation of formation flight shall include wing takeoff, wing fingertip and route positions; formation cross under; close and extended trail positions; and rejoins.  The OFT shall display detailed cross country navigation scenes at low and intermediate altitudes.  This includes airfields and areas representative of the local training area.  The OFT shall support practice of visually oriented contact flight emergency procedures.  These include flameouts, departing prepared surface, low altitude engine failure, low altitude airstart, asymmetrical and no-flap approach and landing, and engine out approach and landing.  At the users’ selection, the OFT shall be used as an IFT.  At the user's selection, the OFT shall automatically reconfigure and function as a UTD.

## 4.3.8.1.1.1.1  Verification of Operational Flight Trainer

This requirement shall be verified by analysis and by demonstration.  Analysis shall consist of design review presentations which show the correlation of OFT design to these training requirements.  Demonstrations shall be used to exercise the device characteristics necessary for performing the training tasks required for the OFT.  Demonstrations shall consist of demonstration missions which cover all training tasks allocated to the OFT, IFT, and UTD.

## 3.8.1.1.1.2  Instrument Flight Trainer

The IFT shall train instrument procedures, instrument takeoffs, instrument cross-check, precision and non-precision approaches.  The IFT shall also train high altitude navigation, in-flight planning, in-flight communications, strange field procedures, transition from instrument to visual flight conditions and then transition to land.  The IFT shall present day and night visual scenes within weather, above weather, and below weather.  It shall also train all aspects of emergency procedures.  The IFT shall display detailed cross country navigation scenes.  This includes airfields and areas representative of the local training area.  At the users selection, the IFT shall automatically reconfigure and function as a UTD.

## 4.3.8.1.1.1.2 Verification of Instrument Flight Trainer

This requirement shall be verified by analysis and by demonstration.  Analysis shall consist of design review presentations which show the correlation of IFT design to these training requirements.  Demonstrations shall be used to exercise the device characteristics necessary for performing the training tasks required for the IFT.  Demonstrations shall consist of demonstration missions which cover all training tasks allocated to the IFT and UTD.

## 3.8.1.1.1.3  Unit Training Device

The UTD shall train normal and emergency (ground and in-flight) procedures associated with operating the aircraft.

## 4.3.8.1.1.1.3  Verification of Unit Training Device

This requirement shall be verified by analysis and by demonstration.  Analysis shall consist of design review presentations which show the correlation of UTD design to this requirement.  Demonstrations shall be used to exercise the device characteristics necessary for performing the training tasks required for the UTD.  Demonstrations shall consist of demonstration missions which cover all training tasks allocated to the UTD.

## 3.8.1.1.2  FTD Elements

## 4.3.8.1.1.2  Verification of FTD Elements

Verification of this paragraph is not applicable.

## 3.8.1.1.2.1  Central Training Complex

The Central Training Complex (CTC) shall be a full fidelity simulation of the performance of the JPATS aircraft.  It shall fully replicate all capabilities of the aircraft throughout the aircraft envelope.  The CTC shall include a replication of the JPATS aircraft front cockpit and all necessary computation systems and other equipment, and sub-systems necessary to accomplish the simulation.

## 4.3.8.1.1.2.1  Verification of Central Training Complex

This requirement shall be verified by inspection and test.  Tests shall be conducted to measure simulation performance versus the JPATS aircraft flight performance and systems performance data to verify full fidelity simulation.  Inspection shall verify the physical replication of the aircraft cockpit.

## 3.8.1.1.2.2  Instructor Operator Station

The Instructor Operator Station (IOS) shall initiate, control and monitor the training session and provide the information necessary to enable an instructor to evaluate a student's performance.

## 4.3.8.1.1.2.2  Verification of Instructor Operator Station

This requirement shall be verified by test and demonstration.  A detailed test procedure shall be written and executed to verify the performance of the IOS.  Demonstration Missions shall be conducted to verify the IOS enables an instructor to control, monitor, and evaluate training.

## 3.8.1.1.2.3  Remote Instructor Station

The Remote Instructor Station (RIS) shall function primarily as a brief and debrief station.  As a briefing station the RIS shall access student records, review the training scenario and precondition the CTC for the training session.  As a debriefing station, the RIS shall access information stored during the training session and update student records.

## 4.3.8.1.1.2.3  Verification of Remote Instructor Station

This requirement shall be verified by test and demonstration.  A detailed test procedure shall be written and executed to verify the performance of the RIS and its interfaces to FTD CTC and TIMS.  RIS operation shall be demonstrated for both briefing and debriefing in conjunction with FTD Demonstration Missions during other testing required by this specification.

## 3.8.1.1.2.4  Canopy

The canopy shall be installed in the OFT and IFT.

## 4.3.8.1.1.2.4  Verification of Canopy

This requirement shall be verified by inspection of the OFT and IFT.

## 3.8.1.1.2.5  Image Generator

The Image Generator (IG) shall process the information required to accomplish all visual training.

## 4.3.8.1.1.2.5  Verification of Image Generator

This requirement shall be verified by analysis and test.  Analysis at design reviews shall show the correlation of Visual Image Generator design to this requirement.  A detailed test procedure shall be written and executed to verify the performance of the image generator.

## 3.8.1.1.2.6  Wide Field of View Visual Display

The Wide Field of View (WFOV) Visual Display shall have a field of view and performance characteristics necessary to accomplish all required OFT training.

## 4.3.8.1.1.2.6  Verification of Wide Field of View Visual Display

This requirement shall be verified by analysis and demonstration.  Analysis at design reviews shall show the correlation of the WFOV Visual Display design to this requirement.  Visual display performance shall be demonstrated in conjunction with OFT Demonstration Missions during other testing required by this specification.

## 3.8.1.1.2.7  Narrow Field of View Visual Display

The Narrow Field of View (NFOV) Visual Display shall have a field of view and performance characteristics necessary to accomplish all required IFT training.

## 4.3.8.1.1.2.7  Verification of Narrow Field of View Visual Display

This requirement shall be verified by analysis and demonstration.  Analysis at design reviews shall show the correlation of the NFOV Visual Display design to this requirement.  Visual display performance shall be demonstrated in conjunction with IFT Demonstration Missions during other testing required by this specification.

## 3.8.1.2  Procedural Training Devices

## 4.3.8.1.2  Verification of Procedural Training Devices

Verification of this paragraph is not applicable.

## 3.8.1.2.1  Egress Procedures Trainer

The EPT shall train normal and emergency procedures for ground and airborne ingress and egress.  The EPT shall be a portable, low fidelity, ruggedized device that simulates the aircraft front cockpit, including all obstructions to egress.  The canopy system shall include a clear glass or Plexiglas shell and be fully operational.  The fire warning, master warning and master caution lights shall be functional and under instructor control.  The transponder segment of the Radio Management Unit shall be capable of displaying the 7700 emergency code when the field select key is activated.  An indication of the activation shall be presented to the instructor.  Other aircraft main instrument panel displays shall be 2 dimensional representations.  All other aircraft controls and displays shall be represented, but inoperative.  All internal and external equipment, controls, and warning devices associated with the seat and canopy systems shall function (including actuation forces) and respond as they do in the aircraft, except that the canopy shall not jettison or shatter, and the ejection seat initiators shall be inert.  Specifically to be included as part of the seat are ejection handle, lapbelt, shoulder harness, parachute risers, ejection seat safety pin and leg restraint leg line release lever and personal survivor pack (PSP) to torso harness connectors.  The seat and rudder pedals shall be adjustable throughout their normal range.  The canopy shall be made of a clear material.  An intercom shall be provided between the instructor and the student.  The G-suit, oxygen and intercom, radio connections, Emergency Firewall Shutoff handle, Parking Brake Handle,  Power Control Lever with Fuel Cutoff Stop Lever and the Canopy Fracture Handle shall be functional.

## 4.3.8.1.2.1  Verification of Egress Procedures Trainer

This requirement shall be verified by inspection and test.  A detailed test procedure shall verify the required performance of the EPT.  Inspection shall be performed to verify the required non-functional capabilities of the EPT.

## 3.8.1.2.2  Ejection Seat Trainer

The EST shall teach the student how to engage and disengage all connections and equipment associated with the ejection seat.  When ejection is properly initiated, the EST shall provide an aural indication to the student.  The EST shall be movable from room-to-room by one person, and shall fit through a standard single door classroom entrance (30" opening).  The EST shall include an ejection seat and restraining system which shall replicate the JPATS aircraft seat and restraining system.  The ejection seat shall be accessible and viewable from all sides.  The EST shall rapidly reconfigure for repetitive use.

## 4.3.8.1.2.2  Verification of Ejection Seat Trainer

This requirement shall be verified by inspection, test and demonstration.  A detailed test procedure shall verify the required performance of the EST.  Inspection shall be performed to verify the physical fidelity, viewability, and accessibility of the EST.  Demonstration shall be performed to verify the reconfigurability and transportability of the EST.

## 3.8.1.2.3  Parachute Procedures Trainer

The PPT shall train parachute procedures in a variety of ground training equipment.  The PPT shall consist of a JPATS harness and the adapters necessary to connect them to: the parachute swing landing trainer, parachute landing fall trainers, para-sailing, classroom demonstrators, and water-survival "pool drag" training.  The adapters shall allow user-selectable use of either existing harness or the JPATS harness.

## 4.3.8.1.2.3  Verification of Parachute Procedures Trainer

This requirement shall be verified by demonstration.

## 3.8.2  Development Courseware Elements

The courseware for each development course shall consist of the elements identified in the following subparagraphs.  All student and instructor materials not used in the aircraft shall be electronically based.  Training materials for use in the aircraft shall be hard copy and similar in size, format, and shape of the current in-flight materials.

SCN 1

## 4.3.8.2  Verification of Development Courseware Elements

This requirement shall be verified by inspection.  Training materials and student performance reports and records shall be inspected to verify compliance with requirements.

## 3.8.2.1  ISD Materials

The instructional tools and methods in support of the course design are the ISD materials.  The ISD materials consist of items such as the Training System Requirements Analysis (TSRA) documentation, syllabus, and lesson strategy and design reports.

## 4.3.8.2.1  Verification of ISD Materials

This requirement shall be verified by an inspection of ISD materials for each course.

## 3.8.2.2  Instructor Materials

The courseware shall include instructor guides and instructional materials used by an instructor to impart, guide, and assess instruction.

## 4.3.8.2.2  Verification of Instructor Materials

This requirement shall be verified by an inspection of the instructor materials for each course.

## 3.8.2.2.1  Abbreviated Syllabus

The courseware shall include an abbreviated syllabus for instructor daily use styled after the existing flight-line syllabus used for T-37 Primary Flight Training.

## 4.3.8.2.2.1  Verification of Abbreviated Syllabus

This requirement shall be verified by inspection of the abbreviated syllabus for each course.

SCN 1

## 3.8.2.3  Course Assessment

Materials shall assess course effectiveness and student retention.  Tests shall:  (1) measure student attainment of the performance objectives, and (2) measure the effectiveness of the instruction.  Criterion tests shall reflect AETC and CNATRA testing procedures and evaluation methods.

## 4.3.8.2.3  Verification of Course Assessment

This requirement shall be verified by analysis, demonstration, and inspection.  Analysis at design reviews shall verify assessment requirements and compliance with AETC and CNATRA procedures and methods.  Test measurement and materials assessment of the JPPT Student Course shall be demonstrated during SLFE.  Student performance reports and records shall be inspected to verify test measurement criteria.  All other development courses shall be demonstrated.

SCN 1

SCN 1

## 3.8.2.4  Student Materials

The courseware shall include materials required for each student to meet study requirements.  Student materials shall include all reference materials needed, including the aircraft Dash-1 and applicable regulations.  For each lesson requiring independent study, there shall be a student guide (SG).  These SGs shall include the lesson objective and the material necessary for the student's comprehension of the objective and associated references.

SCN 1

## 4.3.8.2.4  Verification of Student Materials

This requirement shall be verified by an inspection of the student materials for each course.

## 3.8.2.5  Computer-Aided Instruction

Computer-Aided Instruction (CAI) shall provide self-paced, independent instruction and performance evaluation.  CAI lessons shall use CAI interactive presentation levels as defined in the Lesson Design and Strategy Report.  The highest level CAI (Level III) provides a high degree of interactivity, extensive branching, maximum remediation, real time event simulation (to include three dimensional and animated graphics), interface with other output devices, and thorough computer managed instruction.  The real-time event simulation shall respond to student input as appropriate.  The CAI shall permit students to suspend a lesson and re-start it later at or before the point of suspension.  Performance evaluation shall include intermediate lesson progression checks (quizzes or branching decisions) during CAI lessons.

## 4.3.8.2.5  Verification of Computer Aided Instruction

This requirement shall be verified by analysis and demonstration.  Analysis at design reviews shall verify Computer-Aided Instruction design meets required functional characteristics.  CAI operation shall be demonstrated during SLFE.

## 3.8.2.6  Training Aids and Classroom Equipment

The courseware shall include training aids (i.e., electronic or physical models necessary for instruction) and all presentation devices necessary for classroom instruction.

## 4.3.8.2.6  Verification of Training Aids and Classroom Equipment

This requirement shall be verified by an inspection of the training aids and classroom equipment for each course.  Training aids and classroom equipment shall be demonstrated during SLFE.

## 3.8.2.7  FTD Lesson Sortie Files

Each FTD lesson shall have a corresponding electronic Sortie File.  Each FTD sortie file shall contain training scenarios to set-up, condition, and operate the FTD to train and practice the skills consistent with the lesson objectives.  FTD Lesson Sortie Files shall be in an electronic format that the TIMS can automatically load for use in each FTD.

## 4.3.8.2.7  Verification of FTD Lesson Sortie Files

This requirement shall be verified by analysis and demonstration.  Analysis at design reviews shall verify FTD Lesson Sortie File design meets this requirement.  FTD Lesson Sortie File operation shall be demonstrated during other testing required by this specification.

## 3.8.2.7.1  Simulated Flight Data Bases

FTD lessons shall occur in a simulated environment representing the geographic area used for the training.  These simulated flight databases shall represent the visual, navigation aid, and radio communication characteristics of the environment as necessary to accomplish syllabus training requirements.  The data base shall provide geospecific terrain.  The data base shall provide uninterrupted terrain around the main operating base (MOB) and each outlying and auxiliary field as is required to teach visual traffic pattern procedures, departures, recoveries, and main operating area (MOA) procedures.  The data base shall also provide uninterrupted terrain for all low level navigation routes.  All navigation aids shall be accurately represented.  The simulated flight data bases shall be consistent with the World Geodetic System 84.

## 4.3.8.2.7.1  Verification of Simulated Flight Data Base

This requirement shall be verified by analysis and demonstration.  Analysis at design reviews shall verify Simulated Flight Data Base design meets this requirement.  Simulated Flight Data Base operation shall be demonstrated during other testing required by this specification.

## 3.8.2.8  Electronic Management Files

For each course there shall be a separate set of electronically based TIMS management file(s).  For each course, there shall be a fully populated TIMS database that allows for complete automated management of the course to the full extent provided by the TIMS; and there shall be a fully populated CSS database that allows for complete automated course support, control, and currency to the full extent provided by the CSS.

## 4.3.8.2.8  Verification of Electronic Management Files

This requirement shall be verified by inspection of the electronic management files, and demonstration of representative TIMS and CSS operation using fully populated databases for each course.

## 3.8.3  Standardized Courseware Elements

Courseware elements for standardized courses shall comply with the following subparagraphs.

## 4.3.8.3  Verification of Standardized Courseware Elements

Verification of this paragraph is not applicable.

## 3.8.3.1  Standardized Courseware

All courses listed in tables 3.1.2.3 and 3.1.2.4 shall be standardized for use on the new CBTS to the extent specified in the applicable detail paragraphs following each table.  Standardized CAI presented on the new CBTS shall appear no different to the student than the original CAI running on the original CBTS except as specified in the detail listing.  Minor changes to format and style are permissible only if the new CBTS cannot duplicate the features of the original CBTS systems.  Each course shall include fully populated electronic management files in accordance with paragraph 3.8.2.8.

## 4.3.8.3.1  Verification of Standardized Courseware

This requirement shall be verified by analysis, demonstration, and inspection.  Analysis shall specify the extent to which existing CAI lessons are to be standardized and what style and formats changes are permissible.  The operation of representative standardized lessons shall be demonstrated by running new CBTS operations that comply with SS paragraph 4.3.1.2.3.  Inspection shall verify all designated standardized courses and the fully populated electronic management files.

## 3.8.3.2  TIMS Management Courses

Each course shall include electronic management files populated to enable management of the course on the TIMS.

## 4.3.8.3.2  Verification of TIMS Management Courses

This requirement shall be verified by inspection of the electronic management files and demonstration of TIMS operation using actual course material.

## 3.8.4  Operational Support Segment

The Operational Support Segment (OSS) consists of the TIMS, the CBTS, and the MUSS.  The TIMS, CBTS, ATDSS, CSS and TIMS-SS shall be integrated into a distributed, fault tolerant, data base client-server system as a seamless network.  Seamless is defined to be the ability for an OSS user to log onto and access data (subject to access permissions) within a base to perform an OSS function without needing to know specifically where the data resides. The OSS shall effectively integrate and automate the conduct, administration, and support of Undergraduate Flight Training.  The OSS shall utilize existing Navy fiber optic cabling where available, and as agreed to by the service decision authority, to install a network for between-building connectivity.  The OSS shall utilize Air Force Base Networks or fiber for between-building connectivity.  The OSS shall utilize a government-provided network for between-base connectivity.

SCN 1

SCN 1

- The OSS shall permit integrated or independent operation of Navy and Air Force portions of the systems.  The OSS shall also permit integrated or independent operation of the various components.  When operating in the integrated mode, all data on either Services’ system shall be accessible to the other Service, subject to access permissions.  The OSS shall permit only USAF, only USN, or both services to have access to read, write, copy, or delete data.

- There shall be no single failure point in which a Flight or Squadron is unable to continue management of training using the TIMS and CBTS.  If there is a failure, the OSS shall operate even if in a degraded mode.  In the event of failure, all data, except that local to a workstation, shall remain available without operator intervention.  It shall not be necessary to shut down an entire computer network to repair a failure or perform preventive maintenance.  The OSS shall remain operational even during periods where lightning is in the immediate area.

- Each base shall maintain its data independent of the other bases.  If part of the system goes down, that data shall be retained until the interface link is re-established and the data is exchanged.

- Peripheral equipment shall be accessible to all stations on the OSS network within a base, subject to access permissions.

- Data shall be entered only once into any part of the OSS.

- All hardware used on the network shall be certified interoperable.

## 4.3.8.4  Verification of Operational Support Segment

This requirement shall be verified by analysis, demonstration, and test.  Analysis of OSS system architecture at design reviews shall verify in detail the required characteristics.  OSS operation shall be demonstrated at SLFE.  Comprehensive tests shall verify: network operation, data integrity, hardware interoperability, and failure mode recovery.

## 3.8.4.1  Computer-Based Training System

The CBTS shall provide all instructional features required herein.  The CBTS shall present CAI as defined in paragraph 3.8.2.5.  The CBTS system shall provide an interactive digital multimedia presentation of still and motion video including full and partial screen combinations.  The CBTS shall manipulate screen presentations to zoom, enlarge, rotate, and select different view angles under student control.  The CBTS shall present three dimensional and animated graphics.  The CBTS shall present electronic models of training aids with fidelity, operability, and dynamics that fully aid the instruction as would a physical training aid.  The CBTS shall be networked.  The network shall support transmission of lesson data from a library of CAI and MIL materials to the student or instructor within one minute under all student loads.  It shall not take longer than 1 minute to enter lesson completion data.  The CBTS shall generate a printout of student performance or MIL lesson materials within 2 minutes of any request.  Once a print job is initiated, the requesting workstation shall be free to perform other functions.  The CBTS shall electronically exchange student performance data with the TIMS.  The activation of JPATS CBTS shall not impact the ongoing operation of existing CBT systems.  The CBTS shall playback recorded FTD missions when requested by the student.

## 4.3.8.4.1  Verification of Computer-Based Training System

This requirement shall be verified by analysis and demonstration.  Analysis of CBTS system architecture at design reviews shall verify in detail the required characteristics.  CBTS operation shall be demonstrated at SLFE.

## 3.8.4.1.1  CAI Laboratory

The design quantity of CAI laboratories at each base shall be as shown in Table 3.8.4.1-1.  Each CAI laboratory shall consist of student stations, printers and other items required to meet the requirements of this specification.

- The CAI Laboratory shall allow an instructor to access student grades and status, compile summary data and statistics, administer tests, and analyze results.  It shall also allow students to communicate with the instructors to ask questions or get help.

- Failures of CBTS equipment in a single classroom shall not impact the use of any other classroom.

- No failure of a single student station shall impact use of any other student station.   No maintenance on any student station shall impact any other student station.

TABLE 3.8.4.1-1  CAI Laboratories by Base

| Base | CAI Labs | Base | CAI Labs |
| --- | --- | --- | --- |
| Randolph AFB | 2 | NAS Corpus Christi | 3 |
| Laughlin AFB | 5 | NAS Whiting | 5 |
| Columbus AFB | 5 | NAS Pensacola | 4 |
| Vance AFB | 5 | NAS Kingsville | 2 |
| Moody AFB | 2 | NAS Meridian | 2 |
| Sheppard AFB | 2 |  |  |
| Hondo | 0 |  |  |
| USAFA | 0 |  |  |

## 4.3.8.4.1.1  Verification of CAI Laboratory

This requirement shall be verified by analysis, demonstration, and inspection.  Analysis of CAI laboratory system architecture at design reviews shall verify in detail the required characteristics.  CAI laboratory operation shall be demonstrated at SLFE.  Inspection at each site shall verify the number of CAI laboratories.

## 3.8.4.1.1.1  Student Stations

Each student station shall operate with or without the presence of an instructor.  All stations shall consist of a student carrel with stereo headsets.  The students shall be permitted to access their own CAI training lessons from any student station on that student’s base.  The equipment at each station shall be housed in a carrel at the proper ergonomic position for the student; and includes a flat, laminated, uncluttered work surface.  The carrels shall be of sufficient size, space and strength to hold the CBTS workstation computer, keyboard, mouse, display, uninterruptible power supply plus a work surface large enough for using a 3-ring notebook, calculator, and writing pad.  Carrel work surface size shall be reduced where necessary to fit the required number of carrels in the individual CAI Labs.  The station shall also include storage space for materials used with CAI lessons (e.g., charts and look-up tables).  In addition there shall be carrel lighting provided for locations where carrel design causes room ambient lighting to be insufficient.  Power distribution control for the workstation, headphone volume control, and a switch for carrel lighting (where implemented) shall be easily accessible to the student.  Partitioning between carrels shall minimize disturbances from the surrounding environment.  Each learning carrel shall also include an adjustable office-type chair. When disassembled, the carrels shall fit through standard 30 inch doorways.

SCN 1

## 4.3.8.4.1.1.1  Verification of Student Stations

This requirement shall be verified by analysis, inspection, and demonstration.  Analysis during design reviews shall verify student station configuration and arrangement.  Analysis shall verify the carrel work surface size. Each station shall be inspected during acceptance.  Student Station operation shall be demonstrated at SLFE.

## 3.8.4.1.2  MIL Classroom

The MIL classroom design quantities for each base is listed in Table 3.8.4.1-2.  For MIL, the CBTS shall be used for large group presentation of academic materials and instructor-lead group interactive learning.  Each MIL classroom shall have a large screen projection system that is easily viewable by all occupants in the classroom.  The projection system shall display standard television video from external sources.  The classroom shall include an electronic podium, a tailored CBTS workstation that shall operate under instructor control.  There shall be a wireless hand-held means for the instructor to control presentation of MIL.  A "mouse-like" pointing and highlighting device for interaction with the MIL is required.  The MIL classrooms shall be networked with a CAI laboratory, other MIL classrooms, or in any other configuration consistent with the requirements herein.

TABLE 3.8.4.1-2  MIL Classrooms by Base

| Base | MIL Classrooms | Base | MIL Classrooms |
| --- | --- | --- | --- |
| Randolph AFB | 4 | NAS Corpus Christi | 4 |
| Laughlin AFB | 4 | NAS Whiting | 4 |
| Columbus AFB | 4 | NAS Kingsville | 4 |
| Vance AFB | 4 | NAS Meridian | 4 |
| Moody AFB | 4 | NAS Pensacola | 4 |
| Sheppard AFB | 4 |  |  |
| Hondo | 0 |  |  |
| USAFA | 0 |  |  |

## 4.3.8.4.1.2  Verification of MIL Classroom

This requirement shall be verified by analysis, inspection and demonstration.  Analysis of MIL classroom architecture at design reviews shall verify in detail the required characteristics.  Inspection at each site shall verify the number of MIL classrooms, the equipment included and networking.  Operation of MIL classrooms shall be demonstrated during SLFE.

## 3.8.4.1.3  Reserved

## 4.3.8.4.1.3  Reserved

## 3.8.4.1.4  CBTS Computers

Each student station and MIL classroom shall have state of the art COTS equipment.  The computer system shall be compatible with the other systems and CBT standards in current use at the time of delivery.  The system shall include appropriate display controls.  Each station shall provide real-time, full motion video and associated stereo audio playback for lessons of at least 50 minutes duration.

## 4.3.8.4.1.4  Verification of CBTS Computers

This requirement shall be verified by analysis during design reviews and demonstration using actual courseware.

## 3.8.4.1.4.1  Real-Time Imagery

The CBTS shall provide real time student interaction with computer generated imagery as required by training objectives.  The image generation shall not produce any distracting effects such as breakup, dropout, streaking, flashing, noise, or discontinuities of any part of the visual scene.

## 4.3.8.4.1.4.1 Verification of Real-Time Imagery

This requirement shall be verified by demonstration using actual courseware.

## 3.8.4.2  Training Integration Management System

The TIMS shall manage AETC and CNATRA training as specified in Tables 3.1.2.2.1, 3.1.2.2.2, 3.1.2.2.3, and 3.1.2.6.  The TIMS shall operate under maximum student loading conditions in all courses it manages simultaneously.  The TIMS shall:

- Perform autonomous scheduling for the flight, squadron, group, wing, and command level creating daily, weekly, class, and forecast schedules in accordance with paragraph 3.8.4.2.1.  Provide schedule displays compatible with AETC and CNET(CNATRA as lead agent for CNET) operating procedures.

- Control the creation and modification of all courseware elements for each course managed by the TIMS.

- Manage all resources used by courses managed by the TIMS including availability and status of instructors, students, aircraft, airspace, facilities, and media.  Update the data as changes are made.  Ensure compatibility of resources with lessons and sorties.

- Allow data to be collected and analyzed at various AETC and CNATRA command levels on all courses managed by the TIMS.

- Interface with bookstores at all bases to effectively distribute training materials.

- Manage logistics support of the GBTS.

- There shall be no wait to enter data into the system.  Data collection and inputs of grading information shall take no longer than the TRIM "bubble sheet" system.

- Autonomously monitor and perform all routine computer system administration activities (e.g., backing up disk drives, archiving, self test, data transfers to external systems.) for the entire OSS.

- Assure failures are contained to affect at most one workgroup, such as an Air Force Flight or Navy Squadron.

- Provide for command-wide management of courses as specified in Tables 3.1.2.2.1, 3.1.2.2.2, 3.1.2.2.3, and 3.1.2.6 for each command; provide for joint-command integrated management of joint courses and be expandable to potentially manage all AETC and CNATRA flying training.  The TIMS shall retrieve data and provide reports within 15 minutes of request on any single student or group of students or resources managed by the TIMS.

- Interface with AETC and CNATRA command levels.

- Perform the necessary functions of the existing undergraduate flying training management systems.

- Assist in management of the GBTS configuration.  Maintain comprehensive records of change history.

- Provide a variety of performance measurements customized to the plan of instruction and standards established for each course.  Provide for weighted merit ranking of students in a flight, class, squadron, or access any user selected aggregate of students.

- Provide a library of standard reports types, as well as a system to produce user defined queries and reports.

- Provide the following general office automation functions for all TIMS users:  word processing, spreadsheet, database management system, presentation graphics (generation and display), file compression utility and Portable Document Format (PDF) file reader.

- Provide E-Mail.

## 4.3.8.4.2  Verification of Training Integration Management System

This requirement shall be verified by inspection, analysis, demonstration, and test.  Inclusion of office automation functions shall be verified by inspection, analysis, and demonstration during TIMS operation.  Analysis of system architecture at design reviews shall verify in detail the required characteristics.  The TIMS operation shall be demonstrated at SLFE.  Comprehensive tests shall verify: scheduling at the flight, squadron, group, wing and command level, modification and control of courseware elements, data collection, interfaces with other computer systems, performance measurement, configuration management, report generation, resource management, system administration, curriculum management and logistics support.  All other verification by analysis, demonstration, and test of the TIMS required by this specification shall exclude office automation functions.

## 3.8.4.2.1  Scheduling

The TIMS shall prepare schedules current to the minute.  The TIMS shall also prepare flying period daily, weekly, and quarterly forecast schedule plans for each course, and aggregates of courses.  There shall be two scheduling modes:  1) a scheduling assistant that validates and makes suggestions to human scheduler scheduling actions via an electronic grease-board; and 2) an autonomous scheduler that mimics the thought process of a human scheduler, and automatically applies all rules, regulations and constraints and rules of thumb to achieve a validated executable training schedule.  The TIMS scheduling function shall:

- Automatically generate a valid daily event schedule for a Flight within 2.5 minutes after activation of the automatic schedule generation function.

- Have independent flight operations so that each flight (and squadron) can generate schedules independent of the other flights (and squadrons).

- Automatically generate a valid daily event schedule for a Squadron within 5 minutes after activation of the automatic schedule generation function.

- Create and store one primary and up to four alternate schedules.

## 4.3.8.4.2.1  Verification of Scheduling

This requirement shall be verified by analysis, demonstration, and test.  Analysis of system architecture at design reviews shall verify in detail the required characteristics.  The TIMS scheduling operation shall be demonstrated at SLFE.  Comprehensive tests shall verify scheduling modes and forecast schedule plans.

## 3.8.4.3  Modification and Update Support System

The MUSS shall contain copies of all software (both source and executable code) developed for the GBTS.  All GBTS Components shall be completely supported by the MUSS.  The MUSS shall completely generate all developed and modified executable software from source code.

## 4.3.8.4.3  Verification of Modification and Update Support System

This requirement shall be verified by demonstration and inspection.  The MUSS operation shall be demonstrated at SLFE.  Inspection shall verify GBTS software (source and executable).

## 3.8.4.3.1  Aircrew Training Device Support System (ATDSS)

The ATDSS shall provide configuration management, modification and total support of the ATDs.  The ATDSS shall have stand-alone computer(s) and associated peripheral devices independent from the ATD computation systems, and support documentation to enable GBTS ATD support.  The ATDSS shall generate new visual and navigation databases.  The ATDSS shall maintain ATD documentation.

## 4.3.8.4.3.1  Verification of Aircrew Training Device Support System (ATDSS)

This requirement shall be verified by analysis, inspection, and demonstration.  Analysis of the ATDSS system architecture at design reviews shall verify in detail the required characteristics.  ATDSS documentation for ATD support shall be verified by inspection.  Operation of the ATDSS for configuration management, software modification, documentation maintenance, and database generation shall be verified by demonstration.

## 3.8.4.3.1.1  Traceability

The ATDSS shall automatically relate systems and characteristics of the JPATS aircraft to ATD systems and characteristics.  The ATDSS shall also relate System Specification requirements where applicable to the ATD specifications.

## 4.3.8.4.3.1.1  Verification of Traceability

This requirement shall be verified by analysis and inspection.  Analysis of the ATDSS system architecture at design reviews shall verify the capability to provide traceability of ATD characteristics to the JPATS aircraft.  This requirement shall be further verified by inspection of ATDSS documentation products for specification requirements traceability.

## 3.8.4.3.1.2  Computer Aided Software Engineering Tools

The ATDSS shall have Computer-Aided Software Engineering (CASE) tools which include all tools and utilities necessary for ATD software updates and maintenance.  The CASE tools shall be a COTS product.

## 4.3.8.4.3.1.2  Verification of Computer Aided Software Engineering Tools

This requirement shall be verified by analysis and inspection.  Analysis of the ATDSS system architecture at design reviews shall verify in detail the required characteristics.  Inspection of the ATDSS software documentation shall verify the requirement that the CASE tools are incorporated in the ATDSS.

## 3.8.4.3.1.3  ATDSS Usage

The ATDSS shall permit 2 persons  to perform modifications simultaneously.

## 4.3.8.4.3.1.3  Verification of ATDSS Usage

This requirement shall be verified by demonstration.  Simultaneous operation of the ATDSS by 2 persons shall be verified by demonstration.

## 3.8.4.3.2  Curriculum Support System (CSS)

The CSS shall provide configuration management, modification, and support for the courseware as required by this System Specification.  The CSS shall also perform course effectiveness evaluations.  The CSS shall include a single courseware authoring system which supports all GBTS courseware plus all hardware, software, and documentation necessary to use it effectively.  The CSS shall have automated equipment, ISD tools, and procedures to perform development, revision, and production of all courseware elements.  The CSS shall generate and modify CBTS real-time imagery.  The CSS shall include a document management system allowing rapid, computer based document review and change identification.  The document management system shall allow rapid search and global replacement of all data on any topic. The CSS shall produce a print-ready, electronic version of each course printed material; duplication will be performed by AETC and CNATRA.

SCN 1

## 4.3.8.4.3.2  Verification of Curriculum Support System

This requirement shall be verified by analysis, demonstration, and inspection.  Analysis of system architecture at design reviews shall verify in detail the required characteristics.  CSS documentation for Courseware support shall be verified by inspection.  The CSS operation shall be demonstrated at SLFE.

## 3.8.4.3.2.1  Traceability of Objectives

The CSS shall automatically relate the following items:

- The course objective hierarchy.

- Specific lesson requirements.

- Methods of performance evaluation and its associated criterion tests.

- The ATDs, where and when applicable.

- The aircraft used in the course, where applicable.

- System specification requirements.

- Courseware specification requirements.

The system shall provide similar tracking for activity-based syllabuses.  The CSS shall maintain this tracking for each course.

## 4.3.8.4.3.2.1  Verification of Traceability of Objectives

This requirement shall be verified by analysis at design reviews and demonstration during SLFE.

## 3.8.4.3.2.2  CSS Usage

The CSS shall permit multiple training development teams of up to seven persons each to perform modifications and update activities simultaneously.  The Air Force CSS shall have three independent training development teams housed at one location on Randolph AFB, and two independent training development teams housed at one location within a 25 mile radius of Randolph AFB.  The Navy CSS shall have two teams housed at one location on NAS Corpus Christi.

## 4.3.8.4.3.2.2  Verification of CSS Usage

This requirement shall be verified by demonstration.

## 3.8.4.3.2.3  Call In Support

The CSS shall provide customer call-in help for courseware support.

## 4.3.8.4.3.2.3  Verification of Call In Support

This requirement shall be verified by demonstration.

## 3.8.4.3.3  TIMS Support System (TIMS-SS)

The TIMS-SS shall provide configuration management, modification and support for the TIMS, CBTS, MUSS and associated documentation.  The TIMS-SS shall control external computer system access to the OSS.

## 4.3.8.4.3.3  Verification of TIMS Support System (TIMS-SS)

This requirement shall be verified by analysis, inspection, and demonstration.  Analysis of system architecture at design reviews shall verify in detail the required characteristics.  TIMS documentation for TIMS operation shall be verified by inspection.  The TIMS-SS operation shall be demonstrated at SLFE.

## 3.8.4.3.3.1  Requirements Traceability

The TIMS-SS shall relate System Specification requirements where applicable to the TIMS specifications.

## 4.3.8.4.3.3.1  Verification of Requirements Traceability

This requirement shall be verified by analysis at design reviews and demonstration during SLFE.

## 3.8.4.3.3.2  Computer Program Support Environment

The TIMS-SS shall have CASE tools which include all tools and utilities necessary for TIMS software updates and maintenance.  The CASE tools shall be a COTS product.

## 4.3.8.4.3.3.2  Verification of Computer Program Support Environment

This requirement shall be verified by analysis and inspection.  Analysis of the TIMS-SS system architecture at design reviews shall verify in detail the required characteristics.  Inspection of the TIMS-SS shall verify the requirement that the CASE tools are incorporated in the TIMS-SS.

## 3.8.4.3.3.3  TIMS-SS Usage

The TIMS-SS shall permit 2 persons to perform modifications and support activities simultaneously.

## 4.3.8.4.3.3.3  Verification of TIMS-SS Usage

This requirement shall be verified by demonstration.

## 3.8.4.3.3.4  Call In Support

The TIMS-SS shall provide customer call-in help for the TIMS and CBTS operations at each base.  The TIMS-SS shall remotely access computers to facilitate rapid problem resolution.

## 4.3.8.4.3.3.4  Verification of Call In Support

This requirement shall be verified by demonstration.

# 4.0  Quality Assurance Provisions

# 4.1  General

The basic objective of quality assurance is to verify that all the requirements of Section 3 have been met.  This activity shall be accomplished with the completion of Functional Configuration Audit (FCA).  The test program shall include all activities required by the SOW and herein.  In addition to the specific requirements herein, the GBTS shall be exercised in all appropriate phases of System Level Formative Evaluation.

## 4.1.1  Responsibility for Tests

In accordance with the SOW.

# 4.2  Specific Verification Provisions

Verification of the Section 3 requirements shall be accomplished by inspection, analysis, demonstration, test, or a combination of these methods as defined in the subsequent Section 4.3 paragraphs.  The numbering of paragraphs in Section 4.3 is consistent with Section 3 paragraph numbering.  For example 4.3.3.7.7.1 coincides with 3.3.7.7.1.  The definitions of each of these verification methods is contained in paragraphs 4.2.1 through 4.2.4.  For each Section 3 paragraph there is a corresponding Section 4.3 paragraph.

## 4.2.1  Inspection

Verification shall be by visually examining the item, reviewing descriptive documentation, and comparing the appropriate characteristics with predetermined standards to determine conformance to requirements without the use of special laboratory equipment or procedures.  When specifically stated in Section 4.3, inspection requirements may be fulfilled by a letter certifying that the inspection was performed and that it successfully verified the requirement.

## 4.2.2  Analysis

Verification shall be by technical or mathematical models or simulation, algorithms, charts, graphs, or circuit diagrams, and representative data.  When specifically stated in Section 4.3, analysis requirements may be fulfilled by a letter certifying that the analysis was performed and that it successfully verified the requirement.

## 4.2.3  Demonstration

Verification shall be by operation, adjustment, or reconfiguration of items performing their designed functions under specific scenarios.

## 4.2.4  Test

Verification shall be by use of equipment under controlled conditions in conjunction with a detailed test procedure to assure a requirement has been met.

# 4.3  Verification of Requirements

[NOTE:  The specific section 4.3 verification methods corresponding to the section 3 requirements have been included with the applicable section 3 paragraphs.]

# 5.0  Reserved

# 6.0  Notes

# 6.1  Definitions

This paragraph contains definitions of key words and groupings of words and clarifies their use in the document.

| Key Words | Definitions |
| --- | --- |
| Academics | Instruction delivered in a classroom, also referred to as ground school. |
| Authoring System | Prepackaged computer-aided instruction (CAI) templates or menu-driven editors which allows developers to create courseware on a host computer without requiring elaborate programming skills. Uses a courseware development approach that does not require computer code generation as part of the instructional development process. The authoring system supports multiple instructional strategies and a variety of course content. |
| Base | Any AETC or CNATRA training location at which a GBTS Component is installed. It includes all the locations listed in paragraph 3.1. |
| Block of Instruction | A group of related instructional lessons covering a major subject area. For example, the Aerodynamics Block of Academic Instruction or the Navigation block of flying instruction. |
| Branch | A departure from the normal sequential line-by-line flow of a program. To jump from one sequence in a program to another. An interactive courseware segment selected on the basis of user response. A branch may be either conditional (based upon the outcome of a test) of unconditional (absolute). |
| Class | Collection of students all taking a similar portion of the curriculum. |
| Commercial Off the Shelf (COTS) Item | An item that has not been modified (except as intended by the vendor) from a vendor's commercial version. The design configuration of COTS items is controlled by commercial markets and independent contractor and vendor actions rather than by the government. Modification of an item by the original vendor using non-GBTS contract funds and established as part of the vendor's commercial product line prior to incorporation into the GBTS is considered COTS. |
| Component of GBTS | A major portion or section of the GBTS. These include all ATDs, courses, TIMS, CBTS, CSS, ATDSS, and TIMS-SS. |

| Key Words | Definitions |
| --- | --- |
| Computer-Assisted Instruction (CAI) | An instructional presentation method whereby students interact (usually) individually with instruction presented through the CBTS media. The student can respond to the stimuli in a variety of ways, such as with a keyboard or pointing device, and is immediately informed about the accuracy of the response. The computer uses algorithms to diagnose the response to determine the probable reason for errors, and to present additional instruction in accordance with the diagnosis. CAI includes audio capabilities. |
| Course | Logically grouped units or blocks of instruction designed to teach and achieve specific learning objectives. A course consists of an ordered arrangement of lessons (academics, simulated flying and flying) designed to achieve pre-defined areas of specialization or a specific qualification level for a crew position (e.g., pilot initial qualification course). |
| Courseware | Encompasses all instructional materials which support academic or hands-on training. Courseware includes training device mission guides, flight training guides, lesson plans, Computer Aided Instruction (CAI) lessons, Student Guides, Instructor Guides, classroom media, audio-visual media (e.g., textual material, audio tapes, slides, and video), and associated training materials (charts, calculators, "wiz" wheels, tables, etc.) |
| Criterion | A parameter which provides a basis for judging proficiency. |
| Criterion-Reference Test | Test to determine if behavior as specified in objectives has been acquired. During instructional system development the criterion-reference test can be used to measure the effectiveness of the instructional system. |
| Curriculum | A set of courses constituting an area of specialization or a particular course of study. It includes all courses outlined into specific topics required to satisfy specific training objectives, graduated with a recognized level of education/training, with a certified proficiency. For example: SUPT is a curriculum, whereas JPPT SP is a course. |

| Key Words | Definitions |
| --- | --- |
| Hosted Software | Additional software loaded onto the TIMS workstation above and beyond that which is necessary for operation of the TIMS functions. (Note that Office Automation and e-mail software is part of the Administrative Management (AM) Function and therefore is not Hosted Software). |
| Joint Undergraduate Navigator Training (JUNT) | All JUNT student navigators will train in a common primary track prior to advancing into one of several specialized advanced tracks. |
| Knowledges | Thoughts manifested (inferred) through associated overt activities. Knowledges are not directly observable. They involve the use of mental processes which enable a person to recall facts, identify concepts, apply rules or principles, solve problems, think creatively, and use symbols. |
| Media | The training presentation equipment or style used to provide instruction. Included are the aircraft, all ATDs, the CBTS presentation platform and authoring system, platform instruction training aids, classroom equipment, video players, projection displays, books, handouts, maps, student materials, instructor guides, grade sheets, slides, photographs, sound recordings, film, video tapes, etc. |
| Mock-Ups | Models that can be used to simulate actual equipment such as an instrument panel or a specific set of instruments. |
| Objectives | Objectives specify precisely what behavior is to be exhibited, the condition under which the behavior will be accomplished, and the minimum standard of acceptable performance. |
| Phase | A logical sub-division of a curriculum. |
| Proficiency Level | The graduated, relative ability (or inability) to perform a task. |
| Segment | A part of the GBTS system. |

| Key Words | Definitions |
| --- | --- |
| Skills | Skills involve physical or manipulative activities. They often require knowledge for their execution. All skills are actions having special requirements for speed, accuracy, or coordination. (See also Knowledges.) |
| Sortie | One training session in the aircraft or an FTD. |
| Specialized Undergraduate Pilot Training (SUPT) | All USAF student pilots will train in a common primary track prior to advancing into one of two specialized advanced tracks--the Bomber-Fighter track employing the T-38, and the Tanker-Transport track, employing the T-1A. The T-37B and its associated GBTS are currently used for primary training. |
| Standard | The value of a criterion which defines proficiency. The standard defines the criteria for acceptable performance by the student. |
| SG SCN 4 | Student Guide |
| Training Aid | A chart, graphic, or cutaway used by an instructor usually for group or classroom instruction. Training aids include both operational and non-operational equipment. Any item which is developed or procured with the primary intent that it shall assist in training and the process of learning. |
| Undergraduate Flying Training (UFT) | Undergraduate flying training encompasses JPPT, SUPT, JUNT, UNPT, UNFOT, IP, IFF, IBF, EFS, etc., introductory lead-in training, continuation training, USN undergraduate helicopter training, all course derivatives, and all specialty course training. This includes all tracks and phases of training, for all T-37, T-38, T-1, AT-38, JPATS, T-34, T-3, T-2, TA-4, T-45, TH-57B/C, and T-44 curricula. |