NETWORK SYSTEMS REQUIREMENTS DOCUMENT

- AIR FORCE

- JOINT PRIMARY AIRCRAFT TRAINING SYSTEM

- GROUND BASED TRAINING SYSTEM

- OPERATIONAL SUPPORT SEGMENT

- TRAINING INTEGRATION MANAGEMENT SYSTEM

CONTRACT #KTFSSC797

SDRL G038

30 April 2002

Revision C

Prepared by:

FlightSafety Services Corporation

3333 S. Bannock, #100

Englewood, CO  80110-2432

Prepared for:

Raytheon Aircraft Company

Trainer Systems Division

9709 E. Central Avenue, Bldg. B-90

Wichita, KS  67201

DISTRIBUTION STATEMENT “D”:  Distribution authorized to Department of Defense, RAC, and DOD subcontractors only to protect technical or operational data or information from automatic dissemination under the International Exchange Program or by other means.  This protection covers information required solely for administrative or operational purposes, date of document as shown hereon, 12 June 2001 ASC/YTK.

WARNING:  This document contains technical data whose export is restricted by the Arms Export Control Act (Title 22, U.S.C. 2751 et seq.) or Executive Order 12470.  Violation of these export control laws is subject to severe criminal penalties.  Dissemination of this document is controlled under DOD Directive 5230.25

| 3333 S. Bannock St. Suite 100 Englewood, CO 80110 CAGE CODE: 7Y637 (FSSC) |  | DOCUMENT CHANGE LOG (DCL) | Project: JPATS Page 1 of 1 |
| --- | --- | --- | --- |
| JPATS GBTS TIMS Network Systems Requirements Document |  |  |  |
| Revision No/ Release Date | Reason for Revision |  | Pages/Sections Affected |
| 3 September 1999 | Per Draft |  | All |
| 20 September 1999 | Per SDRL |  | All |
| Revision A 28 January 2000 | This document incorporates government comments provided as part of the 8 Dec 99 memorandum for record. |  | All |
| Revision B 12 June 2001 | This document incorporates changes caused after installations at Randolph AFB and Moody AFB and changes to Vance AFB and Sheppard AFB networks. |  | All |
| Revision C 30 April 2002 | This document incorporates changes caused after TIMS installation at Randolph AFB. |  | All |

- TABLE OF CONTENTS

Section	Title	Page

1.  	OVERVIEW, REQUIREMENTS, METHODOLOGIES, GENERAL ASSIGNMENTS	1

1.1  	Project Title and Scope	1

1.2  	Document Organization	1

1.3  	Overall Network System Requirements	1

1.3.1  	Salient Network Requirements	1

1.3.2  	Deleted	1

1.4  	Deleted	1

1.5  	Analysis Methodologies Used	2

1.6  	Deleted	2

1.7  	Network Design	2

1.8  	Organizations Contacted and Date of Actual Survey	2

1.9  	Network Implementation Assignments	2

1.9.1  	Telephone Requirements	2

1.9.2  	WAN - NIPRNET Requirements	2

1.9.3  	BAN for the Air Force	3

1.9.3.1  Contractor BAN Responsibilities	3

1.9.3.2  Government BAN Responsibilities	3

1.9.4  	Inside Building LAN for the Air Force	3

1.9.5  	Telecommunications Closets	4

1.9.6  	Computer Equipment Rooms	4

1.9.7  	End-user Workstations	4

1.10  	Scaled Drawings and Floor Layouts	4

1.11  	Power and Heat Loads	4

1.12  	Deleted	5

1.12.1  	Deleted	5

2.  	ACRONYMS AND ABBREVIATIONS	6

Appendix A

10.  	RANDOLPH AFB	A-2

10.1  	Organizations Contacted and Date of Actual Survey	A-2

10.2  	Scaled Drawings and Floor Layouts	A-2

10.3  	Power and Heat Loads	A-2

10.4  	Randolph AFB Planned Architecture for TIMS	A-2

10.5  	Server Functions and Locations	A-4

10.6  	Telecommunication Closet Rack	A-5

10.7  	Between Building Edge Device Assignments	A-5

10.7.1  	Government	A-5

10.7.2  	Contractor	A-6

Appendix B

20.  	MOODY AFB	B-2

20.1  	Organizations Contacted and Date of Actual Survey	B-2

20.2  	Scaled Drawings and Floor Layouts	B-2

- TABLE OF CONTENTS (cont'd)

Section		Title	Page

20.3  	Power and Heat Loads	B-2

20.4  	Moody AFB Planned Architecture for TIMS	B-2

20.5  	Telecommunication Closet Rack	B-3

20.6  	Between Building Edge Device Assignments	B-4

20.6.1  	Government	B-4

20.6.2  	Contractor	B-4

Appendix C

30.  	COLUMBUS AFB	C-2

30.1  	Persons Contacted and Date of Actual Survey	C-2

30.2  	Scaled Drawings and Floor Layouts	C-2

30.3  	Power and Heat Loads	C-2

30.4  	Columbus AFB Planned Architecture for TIMS	C-2

30.5  	Server Functions and Locations	C-4

30.6  	Telecommunication Closet Rack	C-4

30.7  	Between Building Edge Device Assignments	C-5

30.7.1  	Government	C-5

30.7.2  	Contractor	C-5

Appendix D

40.  	LAUGHLIN AFB	D-2

40.1  	Organizations Contacted and Date of Actual Survey	D-2

40.2  	Scaled Drawings and Floor Layouts	D-2

40.3  	Power and Heat Loads	D-2

40.4  	Laughlin AFB Planned Architecture for TIMS	D-2

40.5  	Server Functions and Locations	D-3

40.6  	Telecommunication Closet Rack	D-4

40.7  	Between Building Bandwidth Assignments	D-4

40.7.1  	Deleted	D-4

40.7.2  	Deleted	D-4

Appendix E

50.  	VANCE AFB	E-2

50.1  	Organizations Contacted  and Date of Actual Survey	E-2

50.2  	Scaled Drawings and Floor Layouts	E-2

50.3  	Power and Heat Loads	E-2

50.4  	Vance AFB Planned Architecture for TIMS	E-2

50.5  	Server Functions and Locations	E-4

50.6  	Telecommunication Closet Rack	E-4

50.7 	Between Building Edge Device Assignments	E-5

50.7.1  	Government	E-5

50.7.2  	Contractor	E-5

- TABLE OF CONTENTS (cont'd)

Section		Title	Page

Appendix F

60.  	SHEPPARD AFB	F-2

60.1  	Organizations Contacted and Date of Actual Survey	F-2

60.2  	Scaled Drawings and Floor Layouts	F-2

60.3  	Power and Heat Loads	F-2

60.4  	Sheppard AFB Planned Architecture for TIMS	F-2

60.5  	Server Functions and Locations	F-4

60.6  	Telecommunication Closet Rack	F-4

60.7  	Between Building Edge Device Assignments	F-4

60.7.1  	Government	F-4

60.7.2  	Contractor	F-5

Appendix G

70.  	HONDO AIR FIELD	G-2

Appendix H

80.  	UNITED STATES AIR FORCE ACADEMY (USAFA)	H-2

- LIST OF FIGURES

Figure	Title	Page

A-1.  	Randolph AFB OSS Network Logical Layout	A-3

A-2.  	Randolph AFB OSS Physical Server Layout	A-4

B-1.  	Moody AFB OSS Network Logical Layout	B-2

B-2.  	Moody Physical Server Layout Server Functions and Locations	B-3

C-1.  	Columbus AFB OSS Network Logical Layout	C-3

C-2.  	Columbus AFB Physical Server Layout	C-4

D-1.  	Laughlin AFB OSS Network Logical Layout	D-2

D-2.  	Laughlin AFB Physical Server Layout	D-3

E-1.  	Vance AFB OSS Network Logical Layout	E-3

E-2.  	Vance Physical Server Layout	E-4

F-1.  	Sheppard AFB OSS Network Logical Layout	F-3

F-2.  	Sheppard AFB Physical Server Layout	F-3

- LIST OF TABLES

Table	Title	Page

1. 		Expected GBTS WAN Traffic (TCP Port 137 thru 139)	3

2.  		TIMS Equipment Specifications	5

A-1.  	Randolph AFB OSS Estimated Power and Heat Building Requirements	A-2

A-2		Randolph AFB Telecommunication Closets and Equipment Rooms Electrical Circuit

Requirements	A-2

A-3. 	Randolph AFB Server Types and Locations	A-5

A-4.  	Building Rack Requirements for Server Equipment	A-5

A-5.  	Government Provided Between Building Connections and Edge Devices	A-6

A-6.  	Preliminary RAC Provided Building Connections and Edge Devices	A-7

B-1.  	Moody AFB Power and HVAC Requirements Per Building and Floor	B-2

B-2.  	Moody AFB Telecommunication Closets and Equipment Rooms Electrical Circuit

Requirements	B-2

B-3.  	Moody AFB Server Types and Locations	B-3

B-4.  	Building Rack Requirements for Server Equipment	B-4

B-5.  	Preliminary Government Provided Between Building Connections and Edge Devices	B-4

B-6.  	Preliminary RAC Provided Building Connections and Edge Devices	B-4

C-1.  	Columbus AFB Power and HVAC Requirements Per Building	C-2

C-2.  	Columbus AFB Telecommunication Closets and EquipmentRooms Electrical Circuit

Requirements	C-2

C-3.  	Columbus AFB Server Types and Locations	C-4

C-4.  	Building Rack Requirements for Server Equipment	C-5

C-5.  	Preliminary Government Provide Between Building Edge Devices	C-5

C-6.  	Preliminary RAC Provide Building Edge Devices	C-5

D-1.  	Laughlin AFB Power and HVAC Requirements for Servers	D-2

D-2.  	Laughlin AFB Telecommunication Closets and EquipmentRooms Electrical Circuit

Requirements	D-2

D-3.  	Laughlin AFB Server Types and Locations	D-3

D-4.  	Building Rack Requirements for Server Equipment	D-4

D-5.  	Between Building Bandwidth Requirements	D-4

E-1.  	Vance AFB Power and HVAC Requirements Per Building and Floor	E-2

E-2.  	Vance AFB Telecommunication Closets and Equipment Rooms Electrical Circuit

Requirements	E-2

E-3.  	Vance AFB Server Types and Locations	E-4

E-4.  	Building Rack Requirements for Server Equipment	E-5

F-1.  	Sheppard AFB Power and HVAC Requirements Per Building and Floor	F-2

F-2.  	Sheppard AFB Telecommunication Closets and Equipment Rooms Electrical Circuit

Requirements	F-2

F-3.  	Sheppard AFB Server Types and Locations	F-4

F-4.  	Unit Rack Space Requirements for Network Equipment	F-4

F-5.  	Building Rack Space Requirements for Network Equipment	F-4

F-6.  	Building Rack Requirements for Server Equipment	F-4

F-7.  	Preliminary Government Provided Between Building Edge Devices	F-4

F-8.  	Preliminary RAC Provided Building Edge Devices	F-5

# 1.  OVERVIEW, REQUIREMENTS, METHODOLOGIES, GENERAL ASSIGNMENTS

# 1.1  Project Title and Scope

This Network Systems Requirements Document (NSRD) is prepared in accordance with DI-FACR-80149/T (Program Support Letter (PSL) Addendum).  The NSRD covers Ground Based Training System (GBTS) Operational Support Segment (OSS) and Training Integration Management System (TIMS) network requirements.  The NSRD identifies base-to-base network requirements to support GBTS OSS and TIMS for the United States Air Force, currently under contract for design based on parameters identified in contract documents up through Engineering Change Proposal (ECP)-21 and Attachment 2 (NSRD changes) contained in the memorandum of record dated 8 December 1999.  This document is In Accordance With (IAW) 5010.4.1.3 of the Unilateral Change Order Modification 001 dated 5 May 1999 and Contract No. JPATS-SUB-96-002 SDRL G038-1.

# 1.2 Document Organization

This document is a living document and will be updated as information is developed and modified throughout the course of the contract.  The document is organized into two sections and eight appendices (A-H).   Section one provides a general overview, description of the document, overall network system requirements, general assumptions, analysis methodologies used, and general assignments.  Section two is a listing of acronyms and abbreviations used in this document.  Appendices A-H cover Randolph Air Force Base (AFB), Moody AFB, Columbus AFB, Laughlin AFB, Vance AFB, Sheppard AFB, Hondo Air Field, and the United States Air Force Academy (USAFA), respectively.

# 1.3 Overall Network System Requirements

The system specifications and Program Unique Specification Document (PUSD) identify network system requirements.

# 1.3.1 Salient Network Requirements

While there are many requirements for the network system, the following three requirement areas are the largest driving factors in establishing the network design:

- Fault tolerance.  While failures occur, network impacts must be minimized and alternate connectivity provided to critical users and resources.

- Power Protection.  Since most network problems and failures occur as a direct result of power problems, the OSS must include equipment necessary to take proactive measures against losses resulting from power problems.  All OSS hardware will be protected against power surges and sudden power loss.

- Performance.  This is a combination of network speed-bandwidth, segmentation, processing power, locations of clients - servers, and end user perception.  Listed below are the network requirements for performance as identified in the PUSD dated 10 September 1999.

| Network data | Time Required |
| --- | --- |
| Access of data, which resides on a given TIMS workstation. | 3 seconds |
| Access to the local organization's and base's data. | 5 seconds |
| Transfer of a 500 kilobyte (KB) file from any TIMS base to any other TIMS base from the start of transfer at the requesting workstation until the transfer is complete (99% reliability). | 16 seconds design goal |

# 1.3.2 Deleted

# 1.4 Deleted

# 1.5 Analysis Methodologies Used

This NSRD and the results contained herein use a number of methodologies.  They are as follows:

- Review governing documents.

- Gather existing and relevant data.

- Make preliminary design rules of thumb based on a. and b. above.

- Visit each base to gather additional data and identify any problems with c. above.

- Update designs for each base using the data gathered and resolve any design issues.

- Document the results.

# 1.6 Deleted

# 1.7 Network Design

The network design for the Air Force is as follows:

- The Wide Area Network (WAN) is implemented by using the Non-Classified Internet Protocol Routed Network (NIPRNET).  It is required between all GBTS bases and is provided by the Government using Defense Information Systems Agency (DISA) resources.  The Government is responsible for providing the WAN for between-base connectivity.  See also paragraph 1.9.2 below.

- Asynchronous Transfer Mode (ATM) base backbone going between major node buildings called Information Transfer Nodes (ITN) with alternate paths between ITNs such that it creates a ring, partial mesh, or complete mesh between ITNs usually at OC-12 (622 Megabits (Mbit) per second (MBPS)) rates.

- Minor node buildings are connected to ITNs either as a sub-ITN or as an end node.  A sub-ITN may have other buildings connected to it.  Sub-ITNs are connected to ITNs using ATM at OC-12 or OC-3 (155 MBPS) rates.

- End node buildings may be connected to ITNs or sub-ITNs using ATM at OC-3 data rates, 100 MBPS fiber extended Ethernet (100Fx), T-1 (1.544 MBPS), or Digital Subscriber Line (DSL) technologies.

# 1.8 Organizations Contacted and Date of Actual Survey

The Organizations Contacted and Date of Actual Survey are shown in each individual base appendix.

# 1.9 Network Implementation Assignments

Generic WAN, Base Area Network (BAN), and Local Area Network (LAN) are listed below.  Refer to each base appendix for assignments specific to a particular base.  Assignments in this section apply to all bases.

# 1.9.1 Telephone Requirements

TIMS requires Remote Access Service (RAS) (dial-in service) at each base to support up to 16 simultaneous users.  RAS service ports are required to support TIMS training by allowing access to the TIMS network from off-site locations. Ports should be 28.8 kbps or greater.

# 1.9.2 WAN - NIPRNET Requirements

TIMS requires WAN connections to all GBTS bases.  The following data flows can be expected from the TIMS OSS:

- TIMS data upward replication to Navy and Air Force Modification Update Support Systems (MUSS) at NAS Corpus Christi and Randolph AFB respectively.  This replication rate is still to be determined but is at least daily and may be hourly.

New Student Replication		400 megabits per day per base
Daily Student Changes			200 megabits per day per base

- MUSS updates to GBTS components require significant bandwidth for dissemination to the 13 bases and MUSS-to-MUSS transfers.  Analysis of available Base provided WAN connections shows only a four-hour window of opportunity each night to send system updates.

MUSS updates to each base		500 to 2000 megabits per day per base

- Daily readiness testing of each GBTS-TIMS component is monitored for successful completion and requires upwards of 0.25 megabits information transfer per network device.  There are over 7,000 network nodes in the 13 base GBTS system.  Remote monitoring and over-the-shoulder user help from the remote Contractor Logistics Support (CLS) personnel at Corpus and Randolph put an ever-growing load on the WAN links.

- Logistic traffic between MUSS and each base of 10 megabits per day.

- Assume 10% of the total bandwidth for TIMS users to gain access to resources on remote TIMS bases. 500 megabits per day per base.

- System management traffic for maintaining status of network via SNMP is estimated at 250 to 500 megabits per day per base.

- WAN traffic is generated with the creation of Trust relationships between Navy and Air Force bases.  Table 1 below lists the expected WAN traffic associated with the Microsoft NT network used for GBTS communications.  The traffic in table 1 utilizes TCP ports 137 thru 139.

Table 1. Expected GBTS WAN Traffic (TCP Port 137 thru 139)

| Activity | Traffic Per Hour (KB) | Traffic Per Day (KB) |
| --- | --- | --- |
| Browser – four announcements every 12 minutes; three BDCs exchange browse lists every 12 minutes with PDC | 65 | 1560 |
| Directory Services – two changes per hour | 40 | 960 |
| WINS Replication – three record changes per hour | 10.5 | 252 |
| Trust Relationships – 10 pass-through authentication’s per day | 6.5 | 156 |
| Directory Replication – two file changes per day | 5 | 120 |

# 1.9.3 BAN for the Air Force

# 1.9.3.1 Contractor BAN Responsibilities

The Raytheon Aircraft Company (RAC) is responsible for:

- Assessing the capability of the existing Air Force BAN infrastructure to support the required GBTS bandwidths using commercial practices.

- Defining between-building bandwidth requirements for the Government-provided network to support the GBTS.

- Defining the interface requirements between the Government-provided BAN and the RAC-provided networks and hardware.

The assessments and assignments are shown in each base appendix.

# 1.9.3.2 Government BAN Responsibilities

The Government is providing all fiber optic cable between buildings at Air Force bases with OSS components.  Additionally, the Government is providing network edge devices at the ITNs of Air Force bases with OSS components.  These edge devices are part of the ITNs and provide the connection via fiber optic cables to each of the non-ITN buildings to the base network backbone.  Edge device assignments are show in each base appendix.

# 1.9.4 Inside Building LAN for the Air Force

The LAN architecture for each building is driven by the total number of network users in a building and the type of network traffic they generate.  The LANs for each building are segmented into Ethernet collision domains by use of switched network ports in the building's network hubs.  OSS users are on fully switched network ports that are their own collision domain.  In the case of the Air Force design, most buildings will have an existing LAN connected to the rest of the base network by fiber optic cable. If the building is an ITN and already on the BAN backbone then the existing building network will be undisturbed by the addition of hubs being provided by the contractor for that building.  Buildings that are not ITNs or Runway Service Units (RSU)/Runway Control Stations (RCS), and have an existing network connected to the BAN using fiber optic cable, the contractor provided hub(s) will be connected back to the BAN using the fiber that had been going to the existing building network.  This will require a "cut-over" in conjunction with the base network managers, the contractor installation team, and the building network users.  The existing building network will then be daisy-chained off the new contractor provided OSS network hub.  The existing building network users connection to the BAN will thus be restored through the daisy chain.   For many cases where the existing building connection to the BAN is 100Fx, the ITN connection for that building will have to be upgraded to OC-3 or greater by the government.  RSU/RCS will be connected to the base backbone by the government with copper cable using at least T-1 data rates and a device converting the T-1 to 10 or 100 MBPS ethernet.  Based on discussions during the week of December 6, 1999, the government will provide connections to the RSU/RSC with data modems or other devices deemed acceptable in terms of cost and performance.  RAC is then not responsible for performance at these locations extending beyond normal operations for a RSU/RCS.

# 1.9.5 Telecommunications Closets

There should be at least one Telecommunications Closet or lockable cabinet located in each building so that the Category 5 cabling distance will not exceed 90 meters.  Equipment residing in Telecommunication Closets will have normal operating conditions ranging between 32 °F to 100 °F and 10% to 85% humidity, (noncondensing).  Grounding within the Telecommunications Closets shall follow American National Standards Institute (ANSI)/National Fire Protection Association (NFPA) 70 commonly known as the National Electric Code (NEC) and lightning protection shall follow ANSI/NFPA 780 the Lightning Protection Code.  Detailed space requirements are provided in the respective appendices.

# 1.9.6 Computer Equipment Rooms

A computer equipment room is a special-purpose room that provides space and maintains a suitable operating environment for large communications and/or computer equipment.  Computer equipment rooms also include workspace for telecommunications personnel.  Computer equipment rooms should meet the same grounding and lightning protection as the telecommunications closets as identified in paragraph 1.7.5.  Computer equipment located in these rooms operates within the following ranges, 50°F to 95°F, and 8% to 80% (noncondensing).

# 1.9.7 End-user Workstations

TIMS requires power receptacles within 6 feet of workstation locations.  Receptacles should be located in such a manner to minimize the presence of power cables in aisles or walkways.

# 1.10 Scaled Drawings and Floor Layouts

The scaled drawings and floor layouts will be provided in each base appendix upon completion of the final site survey.  These drawings will be updated at completion of installation and as-built drawings will be delivered to the MUSS who will maintain configuration control. Updates to the drawings will be addressed by CLS.

# 1.11 Power and Heat Loads

The power and heat loads are shown in each base appendix.  Unit loads for voltage, power and heat for each type of equipment are listed in Table 2.

Table 2.  TIMS Equipment Specifications

| Equipment | Input Voltage Volts Alternating Current (VAC) | Peak Input Current (Amps) | Power (Watts) | Heat Dissipation British Thermal Units (BTU) |
| --- | --- | --- | --- | --- |
| Catalyst 29xx | 110-120 | 2 | 90 | 307 |
| Catalyst 35xx | 110-120 | 2 | 100 | 600 |
| Catalyst 4xxx | 110-120 | 3.8 | 465 | 1582 |
| Catalyst 55xx | 110-120 | 16 | 1230 | 4198 |
| Catalyst 65xx | 110-120 | 17.6 | 1300 | 6140 |
| Server | 110-120 |  | 700 | 2389 |
| RAID | 110-120 |  | 600 | 2047 |
| DLT Tape Backup | 110-120 |  | 70 | 239 |
| SmartUPS 700 | 110-120 | 15 | 450 | 42 |
| SmartUPS 1000 | 110-120 | 15 | 670 | 89 |
| SmartUPS 1400 | 110-120 | 15 | 950 | 171 |
| SmartUPS 2200 | 110-120 | 20 | 1600 | 275 |

# 1.12 Deleted

# 1.12.1 Deleted

# 2.  ACRONYMS AND ABBREVIATIONS

AETC	Air Education and Training Command

AFB	Air Force Base

ATD	Aircrew Training Device

ATM	Asynchronous Transfer Mode

BAN	Base Area Network

BDC	Backup Domain Controller

BTU	British Thermal Units

CLS	Contractor Logistics Support

DISA	Defense Information Systems Agency

DSL	Digital Subscriber Link

ECP	Engineering Change Proposal

FSSC	FlightSafety Services Corporation

GBTS	Ground Based Training System

HVAC	Heating, Ventilation and Air Conditioning

IAW	In Accordance With

ITN	Information Transfer Nodes

KB	Kilobyte

LAN	Local Area Network

Mbit	Megabit

MBPS	Megabit Per Second

Mbyte	Megabyte

MUSS	Maintenance and Update Support System

NAS	Naval Air Station

NEC	National Electric Code

NFPA	National Fire Protection Association

NIPRNET	Non-Classified Internet Protocol Routed Network

NSRD	Network Systems Requirements Document

OSS	Operational Support Segment

PDC	Primary Domain Controller

PSL	Program Support Letter

PUSD	Program Unique Specification Document

RAC	Raytheon Aircraft Company

RAID	Redundant Array of Inexpensive Disks

RAS	Remove Access Server

RCS	Remote Control Station

RIS	Remote Instructor Station

RSU	Runway Supervisory Unit

SPO	System Program Office

TCP	Transmission Control Protocol

TIMS	Training Integrated Management System

UPS	Uninterruptible Power Supply

USAFA	United States Air Force Academy

VAC	Volts Alternating Current

WAN	Wide Area Network

WINS	Windows Internet Name Service

APPENDIX A

RANDOLPH AFB, TX

# 10.  RANDOLPH AFB

# 10.1 Organizations Contacted and Date of Actual Survey

The Randolph Final Site Survey was conducted 19-23 July 1999. Organizations represented were AETC/XP, AETC/SC, Systems Project Office (SPO), RAC, FSSC, and Logicon.

# 10.2 Scaled Drawings and Floor Layouts

See Randolph AFB workstation locations.

# 10.3 Power and Heat Loads

Table A-1 identifies estimated power and heat loads associated with each building receiving TIMS servers.  These values are based on information collected during intermediate site surveys, final site survey, and final installation.  Table A-2 identifies the electrical requirements associated with locations receiving servers and lists the electrical plug used by the equipment and the minimum amperage requirement for the electrical circuit. All electrical circuits need to be separately switched.

Table A-.  Randolph AFB OSS Estimated Power and Heat Building Requirements

|  |  | Totals |  |
| --- | --- | --- | --- |
| Service for Building | Function or User | Watts | BTU |
| 990 | Base Communications | 12000 | 37812 |
| 740 | OSS | 5870 | 21398 |
| 738 Floor 2 | Simulators/Academics | 33100 | 233200 |
| H-6 Floor 1 | AETC/TRSS/CSS | 2800 | 9556 |

Table A-2 Randolph AFB Telecommunication Closets and Equipment Rooms Electrical Circuit Requirements

|  |  | Power Circuits |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Service for Building | Room | Quantity | Plug | Voltage | Amps |
| H-6 | SVR/COMM Rm | 2 | NEMA 5-20P | 120 | 20 |
| 738 | 205 | 4 | NEMA 5-20P | 120 | 20 |
| 740 | N/A | 4 | NEMA 5-20P | 120 | 20 |
| 990 | 119 | 6 | NEMA 5-20P | 120 | 20 |

# 10.4 Randolph AFB Planned Architecture for TIMS

Figures A-1 and A-2 represent the TIMS architecture being deployed at Randolph AFB.

Wing

RAN PDC

Base SQL

Backup

Network Mgmt

SMS

File Server 1

BDC

ClusterServer

Cluster

Server

Academic

Content (2)

T-37/T-6

T-38

T-1

T-43

SQL

SQL

Cluster

Server

SQL

Cluster

Server

SQL

SIM

F1

9 WS

F1

10W

F7

4WS

F1

10 WS

F1

15 WS

F2

9 WS

F6

9 WS

F2

10W

F2

10 WS

F3

9 WS

F5

9 WS

F3

10W

F3

10 WS

MUSS Servers

F4

9 WS

F8

15 WS

HQ

File Svr 2

BDC

CSS

(5)

TIMS-SS

SQL

Cluster

Server

MUSS

PDC

Figure A-.  Randolph AFB OSS Network Logical Layout

Figure A-.  Randolph AFB OSS Physical Server Layout

# 10.5  Server Functions and Locations

Table A-3 identifies the type and location of each server to be installed at Randolph AFB in support of JPATS TIMS.

Table A-3.  Randolph AFB Server Types and Locations

| Building Number | Server Function Type | Location |
| --- | --- | --- |
| 990 | TIMS PDC, DHCP, WINS TIMS SMS T-38 Resource Cluster T-6/T-37 Resource Cluster T-1 Resource Cluster NAV Resource Cluster Base File Server 1 Base File Server 2 | Base Server Room |
| 740 | Academic Content 1 Academic Content 2 HQ SQL Server Cluster T-6 CSS MUSS PDC TIMS Backup Server Network Management Server | MUSS Communications Closet |
| Hangar 6 | T-37 CSS T-38 CSS T-1 CSS NAV CSS | 1st Floor Server Room |
| 738 | Simulator Server TIMS-SS Server | 2nd Floor COMM Room |

# 10.6 Telecommunication Closet Rack

Network equipment will be housed in government provided racks.  Network augmentation meetings held in March 2000 identified the equipment that RAC will provide to augment the existing network to support TIMS.  RAC will provide equipment racks for the TIMS servers being installed at the base.  Table A-4 identifies by building the rack requirements for GBTS TIMS servers.  Servers will be housed in enclosed 19-inch cabinets with a 30” wide by 33” depth footprint and will be 7 feet tall.  Racks for servers will be supplied by RAC unless otherwise stated.

Table A-4.  Building Rack Requirements for Server Equipment

| Building | Number of Servers | Rack Quantities |
| --- | --- | --- |
| 990 | 13 | 4 |
| 740 | 8 | 2 |
| 738 | 2 | 2 |
| H-6 | 4 | 1 |

# 10.7 Between Building Edge Device Assignments

# 10.7.1 Government

The Government is to provide the following bandwidth and edge devices for between building communications to allow GBTS network equipment to connect to the base backbone.  Table A-5 identifies by building the required connections for Randolph AFB.

Table A-5.  Government Provided Between Building Connections and Edge Devices

| Buildings | Assumption |
| --- | --- |
| 3 | One OC-3 SMF connection to base backbone |
| 4 | One 100FX SMF connection to base backbone |
| 5 | One 100FX SMF connection to base backbone |
| 6 | One OC-3 MMF connection to base backbone |
| 7 | One 100FX MMF connection to base backbone |
| 8 | One 100FX MMF connection to base backbone |
| 12 | Two OC-3 MMF connections to the base backbone |
| 16 | One 100FX MMF connection to base backbone |
| 28 | One 100FX MMF connection to base backbone |
| 40 | One 100FX SMF connection to base backbone |
| 41 | One 100FX SMF connection to base backbone |
| 63 | One OC-3 MMF connection to base backbone |
| 64 | Two OC-3 MMF connections to the base backbone |
| 66 | One 100FX MMF connection to base backbone |
| 100 | 100FX MMF port at ITN in basement |
| 148 | One 100FX SMF connection to base backbone |
| 241 | One 100FX SMF connection to base backbone |
| 379 | Provide 2 ATM OC-3 MMF ports or one module slot in LightStream chassis at this ITN |
| 399 | Provide one module slot in 5xxx series switch for one 24 port 10/100 switch module |
| 661 | One OC-3 MMF backbone connection to one (Cisco) ATM switch |
| 663 | Provide one module slot in 5xxx series switch for one 24 port 10/100 switch module |
| 738 | One OC-3 MMF connection to base backbone |
| 740 | Two OC-12 SMF backbone connections to two locations |
| 743 | One OC-3 MMF connection to base backbone |
| 745 | One OC-3 MMF connection to base backbone |
| 747 | One 100FX MMF connection to base backbone |
| 902 | One OC-3 SMF connection to base backbone |
| 905 | Provide one module slot in 5xxx series switch for one 24 port 10/100 switch module |
| 990 | Two OC-12 SMF backbone connections to two locations |
| 1040 | One 100FX MMF connection to Sub-ITN switch in building |
| AUX Field | Provide wide area communications with 10/100 UTP connection |
| Off-base CSS | Provide wide area communications with 10/100 UPT connection |

# 10.7.2 Contractor

RAC will provide network edge devices and communications switches required for TIMS users.  Table A-6 identifies proposed communication switches and backbone connection requirements required for TIMS users to gain access Randolph AFB base backbone.  RAC will provide the required network equipment in a building (not including CAT 5 or Fiber optic cabling) required for TIMS users.  Communications switches will be equipped with the appropriate number of 10/100 Mbps CAT 5 ports, ATM server ports, and uplink modules required for TIMS users and to gain access to the base backbone.

Table A-6.  Preliminary RAC Provided Building Connections and Edge Devices

| Buildings | Assumption |
| --- | --- |
| AUX Field | Cisco Catalyst 2924M-XL-EN |
| 4, 5, 7, 8, 28, 40, 41, 66, 100, 148, 241, 747, 1040 | Cisco Catalyst 2924M-XL-EN with 2 port 100FX up-link module |
| 661 | Cisco Catalyst 2924M-XL-EN with OC-3 ATM up-link module |
| 3 | Cisco Catalyst 5505 with 2 port OC-3 SMF module |
| 16 | Cisco Catalyst 5505 with 2 port OC-3 MMF module |
| 902 | Cisco Catalyst 5500 with 2 port OC-3 SMF module |
| 6, 12, 63, 64, 743, 745 | Cisco Catalyst 5500 with 2 port OC-3 MMF module |
| 738 | Cisco Catalyst 5500 with 4 port OC-3 MMF module |
| 990, 740 | Cisco Catalyst 5500 with two OC-12 SMF modules |
| 379 | Cisco 4 port OC-3 MMF module for LightStream 1010 |
| 399, 663, 905 | Cisco 24 port 10/100 switch module for Catalyst 55xx |

APPENDIX B

MOODY AFB, TX

# 20.  MOODY AFB

# 20.1 Organizations Contacted and Date of Actual Survey

The Moody Final Site Survey was conducted 18-20 October 2000.  Organizations represented were HQAETC/XPRI; HQAETC/XPRI; 347 WG/XP; MSgt. 347 SPTG/CES; HQ AETC/SCXP; 347 CS; RAC, FSSC and Logicon.

# 20.2 Scaled Drawings and Floor Layouts

See Moody AFB Workstation Locations.

# 20.3 Power and Heat Loads

Table B-1 identifies estimated power and heat loads associated with each building receiving TIMS computer, server, and network equipment.  These values are based on information collected during intermediate site surveys and the final site survey.  Table B-2 identifies the electrical requirements associated with locations receiving servers lists the electrical plug used by the equipment and the minimum amperage requirement for the electrical circuit. All electrical circuits need to be separately switched.

Table B-.  Moody AFB Power and HVAC Requirements Per Building and Floor

|  |  | Totals |  |
| --- | --- | --- | --- |
| Service for Building | Function or User | Watts | BTU |
| 709 | Academics | 1400 | 4778 |
| 711 | T-6A Simulator | 3500 | 11945 |
| 792 | T-6A Train. Sqdn | 4140 | 14128 |

Table B-.  Moody AFB Telecommunication Closets and Equipment Rooms Electrical Circuit Requirements

|  |  | Power Circuits |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Service for Building | Room | Quantity | Plug | Voltage | Amps |
| 709 | N/A | 1 | NEMA 5-20P | 120 | 20 |
| 711 | N/A | 3 | NEMA 5-20P | 120 | 20 |
| 792 | A09 | 2 | NEMA 5-20P | 120 | 20 |

# 20.4 Moody AFB Planned Architecture for TIMS

Figures B-1 and B-2 represent the TIMS architecture being deployed at Moody AFB.

Figure B-.  Moody AFB OSS Network Logical Layout

Figure B-.  Moody Physical Server Layout Server Functions and Locations

Table B-3 identifies the type and location of each server to be installed at Moody AFB in support of JPATS TIMS.

Table B-.  Moody AFB Server Types and Locations

| Building Number | Server Function Type | Location |
| --- | --- | --- |
| 709 | Academic Content 1 Academic Content 2 | 1st Floor |
| 711 | TIMS PDC, DHCP, WINS TIMS E-mail TIMS SMS Simulator Server TIMS Backup Server TIMS BBS/File Server | CLS Maintenance Location |
| 792 | T-38 Resource Cluster T-6/T-37 Resource Cluster | Communications Room |

# 20.5 Telecommunication Closet Rack

Network equipment will be housed in government provided racks.  Network augmentation meetings held in March 2000 identified the equipment that RAC will provide to augment the existing network to support TIMS.  Rack will provide equipment racks for the TIMS servers being installed at the base. Table B-4 identifies by building the rack requirements required for GBTS TIMS servers.  Servers will be housed in enclosed 19-inch cabinets with a 24” wide by 36” depth footprint and will be 7 feet tall.

Table B-4.  Building Rack Requirements for Server Equipment

| Building | Number of Servers | Rack Quantities |
| --- | --- | --- |
| 711 | 6 | 3 |
| 792 | 4 | 2 |
| 709 | 2 | 1 |

# 20.6 Between Building Edge Device Assignments

# 20.6.1 Government

The Government is to provide the following bandwidth and edge devices for between building communications to allow GBTS network equipment to connect to the base backbone.  Table B-5 identifies by building the required connections for Moody AFB.

Table B-5.  Preliminary Government Provided Between Building Connections and Edge Devices

| Buildings | Assumption |
| --- | --- |
| 590, 701, 706, 770, 772, 775, 899, 1300, 1301, and RSU/RSC | One 100FX MMF connection to the base backbone for each building specified |
| 103, 328, 622, 707, 739 | One OC-3 MMF backbone connection to one (Cisco) ATM switch |
| 711,771 | One OC-3 MMF backbone connection to one (Cisco) ATM switch |
| 704, 709 and 792 | Two OC-12 SMF backbone connections to two separate (Cisco) ATM switches |

# 20.6.2 Contractor

RAC will provide network edge devices and communications switches required for TIMS users.  Table B-6 identifies proposed communication switches and backbone connection requirements required for TIMS users to gain access Moody AFB base backbone.  RAC will provide the required network equipment in a building (not including CAT 5 or Fiber optic cabling) required for TIMS users.  Communications switches will be equipped with the appropriate number of 10/100 Mbps CAT 5 ports, ATM server ports, and uplink modules required for TIMS users and to gain access to the base backbone.

Table B-6.  Preliminary RAC Provided Building Connections and Edge Devices

| Buildings | Assumption |
| --- | --- |
| 590, 701, 706, 770, 772, 775, 899, 1300, 1301,and RSU/RSC | Cisco Catalyst 2924M-XL-EN with 2 port 100FX uplink module |
| 103, 328, 622, 707, 739 | Cisco Catalyst 2924M-XL-EN with OC-3 ATM uplink module |
| 711,771 | Cisco Catalyst 5500 with 4 port OC-3 module |
| 704, 709 and 792 | Cisco Catalyst 5500 with two OC-12 modules |

APPENDIX C

COLUMBUS AFB, MS

# 30.  COLUMBUS AFB

# 30.1 Persons Contacted and Date of Actual Survey

The Columbus AFB Telecommunications Site Survey was conducted 16-17 July 1999.  Among many representing Air Education and Training Command were Major Thomas Mowle (XP), Micheal Wasson (XP), Mr. Hutchinson, Mr. Larry Gore, and Capt. Bula.  Representing the SPO were Steve Poehl and MSgt. John Miller.

# 30.2 Scaled Drawings and Floor Layouts

See Columbus AFB workstation locations.

# 30.3 Power and Heat Loads

Table C-1 identifies estimated power and heat loads associated with each building receiving TIMS servers.  These values are based on information collected during intermediate site surveys and updated workstation spreadsheet counts.  TableC-2 identifies the electrical requirements associated with locations receiving servers lists the electrical plug used by the equipment and the minimum amperage requirement for the electrical circuit. All electrical circuits need to be separately switched.

Table C-.  Columbus AFB Power and HVAC Requirements Per Building

|  |  | Totals |  |
| --- | --- | --- | --- |
| Service for Building | Function or User | Watts | BTU |
| 230 | Ops Gp ,Academics, T-1 Sqd (Total) | 10410 | 35526 |
| 385 | Backup Location | 700 | 2389 |

Table C-.  Columbus AFB Telecommunication Closets and Equipment Rooms Electrical Circuit Requirements

|  |  | Power Circuits |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Service for Building | Room | Quantity | Plug | Voltage | Amps |
| 230 | TBD | 8 | NEMA 5-20P | 120 | 20 |
| 385 | TBD | 1 | NEMA 5-20P | 120 | 20 |

# 30.4 Columbus AFB Planned Architecture for TIMS

Figures C-1 and C-2 represent the TIMS architecture being deployed at Columbus AFB.

Figure C-.  Columbus AFB OSS Network Logical Layout

Figure C-.  Columbus AFB Physical Server Layout

# 30.5 Server Functions and Locations

Table C-3 identifies the type and location of each server to be installed at Columbus AFB in support of JPATS TIMS.

Table C-.  Columbus AFB Server Types and Locations

| Building Number | Server Function Type | Location |
| --- | --- | --- |
| 385 | TIMS Backup Server | 1st Floor |
| 230 | TIMS PDC, DHCP, WINS TIMS SMS TIMS BBS/File Server T-38 Resource Cluster T-6/T-37 Resource Cluster T-1 Resource Cluster 3 Academic Content | 1st Floor |

# 30.6 Telecommunication Closet Rack

Network equipment will be housed in government provided racks.  Network augmentation meetings held in March 2000 identified the equipment that RAC will provide to augment the existing network to support TIMS.  Rack will provide equipment racks for the TIMS servers being installed at the base. Table C-4 identifies by building the rack requirements for GBTS TIMS servers.  Servers will be housed in enclosed 19-inch cabinets with a 24” wide by 36” depth footprint and will be 7 feet tall.

Table C-4.  Building Rack Requirements for Server Equipment

| Building | Number of Servers | Rack Quantities |
| --- | --- | --- |
| 230 | 12 | 4 |
| 385 | 1 | 1 |

# 30.7 Between Building Edge Device Assignments

# 30.7.1 Government

The Government is to provide the following bandwidth and edge devices for between building communications to allow GBTS network equipment to connect to the base backbone.  Table C-5 identifies by building the required connections for Columbus AFB.

Table C-5.  Preliminary Government Provide Between Building Edge Devices

| Buildings | Assumption |
| --- | --- |
| 240,450, 454, 456, 624, 626, 640, 724, 847, 850, 926, 989, 1100, 1046, RSU/RCS, and AUX Fields | One 100FX MMF connection to the base backbone for each building specified |
| 724, 847 | One OC-3 MMF backbone connection to one (Cisco) ATM switch |
| 216, 385 | Two OC-3 MMF backbone connections to two separate (Cisco) ATM switches located in two separate buildings |
| 230, 236, and 268 | Two OC-12 SMF backbone connections to two separate (Cisco) ATM switches |

# 30.7.2 Contractor

RAC will provide network edge devices and communications switches required for TIMS users.  Table C-6 identifies proposed communication switches and backbone connection requirements required for TIMS users to gain access Columbus AFB base backbone.  RAC will provide the required network equipment in a building (not including CAT 5 or Fiber optic cabling) required for TIMS users.  Communications switches will be equipped with the appropriate number of 10/100 Mbps CAT 5 ports, ATM server ports, and uplink modules required for TIMS users and to gain access to the base backbone.

Table C-6.  Preliminary RAC Provide Building Edge Devices

| Buildings | Assumption |
| --- | --- |
| 240, 450, 454, 456, 624, 626, 640, 850, 926, 989, 1046, 1100, RSU/RCS, and AUX Fields | Cisco Catalyst 2924M-XL-EN with 2 port 100FX uplink module |
| 724, 847 | Cisco Catalyst 2924M-XL-EN with OC-3 ATM uplink module |
| 216 | Cisco Catalyst 5500 with 4 port OC-3 module |
| 230, 236, and 268 | Cisco Catalyst 5500 with two OC-12 modules |

APPENDIX D

LAUGHLIN AFB, TX

# 40.  LAUGHLIN AFB

# 40.1 Organizations Contacted and Date of Actual Survey

The Laughlin Final Site Survey was conducted 02-03 May 2000.  Survey participants included representatives from AETC/XPRI, AETC/SCXX, ASC/YTJG, ASC/DET4, 47CS/SCX, 47CS/SCMP, 47CS/SCBB, DCMC, 47OG/SA, 38EIG/GV, and Logicon.

# 40.2 Scaled Drawings and Floor Layouts

See Laughlin AFB Workstation Locations.

# 40.3 Power and Heat Loads

Table D-1 identifies estimated power and heat loads associated with each building receiving TIMS servers.  These values are based on information collected during the final site survey on May 2-3, 2000.  Table D-2 identifies the electrical requirements associated with locations receiving servers and network equipment and lists the electrical plug used by the equipment and the minimum amperage requirement for the electrical circuit. All electrical circuits need to be separately switched.

Table D-.  Laughlin AFB Power and HVAC Requirements for Servers

|  |  | Totals |  |
| --- | --- | --- | --- |
| Service for Building | Room/Location | Watts | BTU |
| 320 | 1183 | 8310 | 28359 |
| 328 | LAB | 4200 | 14334 |

Table D-.  Laughlin AFB Telecommunication Closets and Equipment Rooms Electrical Circuit Requirements

|  |  | Power Circuits |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Service for Building | Room | Quantity | Plug | Voltage | Amps |
| 320 | 1183 | 6 | NEMA 5-20P | 120 | 20 |
| 328 | LAB | 5 | NEMA 5-20P | 120 | 20 |

# 40.4 Laughlin AFB Planned Architecture for TIMS

Figures D-1 and D-2 represent the TIMS architecture being deployed at Laughlin AFB.

Figure D-.  Laughlin AFB OSS Network Logical Layout

Figure D-.  Laughlin AFB Physical Server Layout

# 40.5 Server Functions and Locations

Table D-3 identifies the type and location of each server to be installed at Laughlin AFB in support of JPATS TIMS.

Table D-.  Laughlin AFB Server Types and Locations

| Building Number | Server Function Type | Location |
| --- | --- | --- |
| 320 | T-38 Resource Cluster T-6/T-37 Resource Cluster T-1 Resource Cluster TIMS BBS/File Server 2 Academic Content | 1183 |
| 328 | TIMS PDC, DHCP, WINS TIMS SMS Simulator Server TIMS Backup Server 2 Academic Content | LAB |

# 40.6 Telecommunication Closet Rack

Network equipment will be housed in government provided racks.  Network augmentation meetings held in March 2000 identified the equipment that RAC will provide to augment the existing network to support TIMS.  Rack will provide equipment racks for the TIMS servers being installed at the base. Table D-4 identifies by building the rack requirements for GBTS TIMS servers.  Servers will be housed in enclosed 19-inch cabinets with a 24” wide by 36” depth footprint and will be 7 feet tall.

Table D-4.  Building Rack Requirements for Server Equipment

| Building | Number of Servers | Rack Quantities |
| --- | --- | --- |
| 320 | 9 | 3 |
| 328 | 6 | 2 |

# 40.7 Between Building Bandwidth Assignments

The Government is to provide the following bandwidth capability for between building communications to allow GBTS network equipment to connect to the base backbone as determined during network augmentation teleconferences.  Table D-5 identifies by building the required connections for Laughlin AFB.

Table D-5.  Between Building Bandwidth Requirements

| Buildings | Bandwidth To Backbone |
| --- | --- |
| 77 | OC-12 |
| 195 | 100FX |
| 204 | 100FX |
| 210 | 100FX |
| 215 | OC-3 |
| 306 | 100FX |
| 310 | N/A |
| 320 | OC-12 |
| 328 | OC-12 |
| 339 | OC-12 |
| 401 | 100FX |
| 404 | OC-3 |
| 414 | 100FX |
| 508 | 100FX |
| 612 | 56K dial-up or greater |
| 670 | 56K dial-up or greater |
| 1700 | 56K dial-up or greater |

# 40.7.1 Deleted

# 40.7.2 Deleted

APPENDIX E

VANCE AFB, OK

# 50.  VANCE AFB

# 50.1 Organizations Contacted and Date of Actual Survey

The Vance Final Site Survey was conducted 04-08 December 2000.  Organizations represented were 71 OSS/OSTQ, 71 OSS/OSTQ, AETC/XPRF, ASC/DET4, NW-SC.FSSC and Logicon.

# 50.2 Scaled Drawings and Floor Layouts

See Vance AFB workstation locations.

# 50.3 Power and Heat Loads

Table E-1 identifies estimated power and heat loads associated with each building receiving TIMS server equipment.  These values are based on information collected during intermediate site surveys and final site surveys. Table E-2 identifies the electrical requirements associated with locations receiving servers and network equipment and lists the electrical plug used by the equipment and the minimum amperage requirement for the electrical circuit. All electrical circuits need to be separately switched.

Table E-.  Vance AFB Power and HVAC Requirements Per Building and Floor

|  |  | Totals |  |
| --- | --- | --- | --- |
| Service for Building | Function or User | Watts | BTU |
| 500 | Wing HQ (Total) | 1400 | 4788 |
| 672 | Simulator | 9040 | 30851 |

Table E-.  Vance AFB Telecommunication Closets and Equipment Rooms Electrical Circuit Requirements

|  |  | Power Circuits |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Service for Building | Room | Quantity | Plug | Voltage | Amps |
| 500 | TBD | 1 | NEMA 5-20P | 120 | 20 |
| 672 | TBD | 8 | NEMA 5-20P | 120 | 20 |

# 50.4 Vance AFB Planned Architecture for TIMS

Figures E-1 and E-2 represent the TIMS architecture being deployed at Vance AFB.

Figure E-.  Vance AFB OSS Network Logical Layout

Figure E-.  Vance Physical Server Layout

# 50.5 Server Functions and Locations

Table E-3 identifies the type and location of each server to be installed at Vance AFB in support of JPATS TIMS.

Table E-.  Vance AFB Server Types and Locations

| Building Number | Server Function Type | Location |
| --- | --- | --- |
| 500 | TIMS Backup Server TIMS BBS/File Server |  |
| 672 | TIMS PDC, DHCP, WINS TIMS SMS T-38 Resource Cluster T-6/T-37 Resource Cluster 4 Academic Content Simulator Server |  |

# 50.6 Telecommunication Closet Rack

Network equipment will be housed in government provided racks.  No network equipment will be provided by RAC.  RAC will provide equipment racks for the TIMS servers being installed at the base. Table E-4 identifies by building the rack requirements for GBTS TIMS servers.  Servers will be housed in enclosed 19-inch cabinets with a 24” wide by 36” depth footprint and will be 7 feet tall.

Table E-4.  Building Rack Requirements for Server Equipment

| Building | Number of Servers | Rack Quantities |
| --- | --- | --- |
| 672 | 11 | 4 |
| 500 | 2 | 1 |

# 50.7 Between Building Edge Device Assignments

# 50.7.1 Government

The Government is to provide all backbone connectivity for TIMS and ensure that TIMS servers are located have redundant connections to the base backbone.  And, provide 1 Gbps multi-mode fiber (MMF) connections to TIMS servers.

# 50.7.2 Contractor

RAC will provide no network edge devices or communications switches required for TIMS users.  RAC will provide equipment racks for the TIMS servers being installed at the base.

APPENDIX F

SHEPPARD AFB, OK

# 60.  SHEPPARD AFB

# 60.1 Organizations Contacted and Date of Actual Survey

The Sheppard Final Site Survey was conducted 17-18 January 2001. Organizations represented were AETC/XPRI, 80 OSS/DOOC, 82CS/SCXS, 82CS/SCXS, 82 CS/SCB, 82CS/SCBB, HQ AETC/SCXX, OGT FSSC and Logicon.

# 60.2 Scaled Drawings and Floor Layouts

See Sheppard AFB workstation locations

# 60.3 Power and Heat Loads

Table F-1 identifies estimated power and heat loads associated with each building receiving TIMS server equipment.  These values are based on information collected during intermediate site surveys and final site surveys.  Table F-2 identifies the electrical requirements associated with locations receiving servers and network equipment and lists the electrical plug used by the equipment and the minimum amperage requirement for the electrical circuit. All electrical circuits need to be separately switched.

Table F-.  Sheppard AFB Power and HVAC Requirements Per Building and Floor

|  |  | Totals |  |
| --- | --- | --- | --- |
| Service for Building | Function or User | Watts | BTU |
| 2320 | ENJJPT | 7240 | 26173 |
| 2330 | Wing HQ (Total) | 1400 | 4778 |

Table F-.  Sheppard AFB Telecommunication Closets and Equipment Rooms Electrical Circuit Requirements

|  |  | Power Circuits |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Service for Building | Room | Quantity | Plug | Voltage | Amps |
| 2320 | TBD | 6 | NEMA 5-20P | 120 | 20 |
| 2330 | TBD | 1 | NEMA 5-20P | 120 | 20 |

# 60.4 Sheppard AFB Planned Architecture for TIMS

Figures F-1 and F-2 represent the TIMS architecture being deployed at Sheppard AFB.

Figure F-.  Sheppard AFB OSS Network Logical Layout

Figure F-.  Sheppard AFB Physical Server Layout

# 60.5 Server Functions and Locations

Table F-3 identifies the type and location of each server to be installed at Sheppard AFB in support of JPATS TIMS.

Table F-.  Sheppard AFB Server Types and Locations

| Building Number | Server Function Type | Location |
| --- | --- | --- |
| 2330 | Simulator Server Base Backup Server | Finalize at Final Site Survey |
| 2320 | TIMS PDC, DHCP, WINS TIMS SMS TIMS BBS/File Server T-38 Resource Cluster T-6/T-37 Resource Cluster 2 Academic Content | Finalize at Final Site Survey |

# 60.6 Telecommunication Closet Rack

Network equipment will be housed in government provided racks.  No network equipment will be provided by RAC.  RAC will provide equipment racks for the TIMS servers being installed at the base. Table F-6 identifies by building the rack requirements for GBTS TIMS servers.  Servers will be housed in enclosed 19-inch cabinets with a 24” wide by 36” depth footprint and will be 7 feet tall.  RAC will provide cabinets for GBTS TIMS servers.

Table F-.  Unit Rack Space Requirements for Network Equipment

Deleted

Table F-5.  Building Rack Space Requirements for Network Equipment

Deleted

Table F-6.  Building Rack Requirements for Server Equipment

| Building | Number of Servers | Rack Quantities |
| --- | --- | --- |
| 2330 | 2 | 1 |
| 2320 | 9 | 3 |

# 60.7 Between Building Edge Device Assignments

# 60.7.1 Government

The Government is to provide all backbone connectivity for TIMS, provide 1 Gbps multi-mode fiber (MMF) connections to TIMS servers, and ensure that TIMS servers have redundant connections to the base backbone.

Table F-7.  Preliminary Government Provided Between Building Edge Devices

| Buildings | Assumption |
| --- | --- |
| 2320 | Two OC-12 |
| 1360, 2333, 2322, 2330 | One OC-3 |
| 430, 1207, 2404, 2408, 2529, 2558, RSU/RCSs, Tower | One 100Fx |

# 60.7.2 Contractor

RAC will provide no network edge devices or communications switches required for TIMS users. RAC will provide the required network equipment in a building (not including CAT 5 or Fiber optic cabling) required for TIMS users.  Communications switches will be equipped with the appropriate number of 10/100 Mbps CAT 5 ports, ATM server ports, and uplink modules required for TIMS users and to gain access to the base backbone.

Table F-8.  Preliminary RAC Provided Building Edge Devices

Deleted

APPENDIX G

HONDO AF, TX

# 70.  HONDO AIR FIELD

Deleted

APPENDIX H

USAFA, CO

# 80.  UNITED STATES AIR FORCE ACADEMY (USAFA)

Deleted