SOFTWARE PRODUCT SPECIFICATION

FOR THE

FLIGHT TRAINING DEVICE

COMPONENT OF THE

JOINT PRIMARY AIRCRAFT TRAINING SYSTEM

GROUND BASED TRAINING SYSTEM

CONTRACT #KTFSSC797

SDRL G019

11 September 2001

Prepared By:

FlightSafety Services Corporation

3333 S. Bannock St., # 100

Englewood, CO 80110

Prepared For:

Raytheon Aircraft Company

Trainer Systems Division

9709 E. Central Avenue, Bldg. 90

Wichita, KS 67201

DISTRIBUTION STATEMENT “D”: Distribution authorized to Department of Defense, RAC, and DOD subcontractors only to protect technical or operational data or information from automatic dissemination under the International Exchange Program or by other means.  This protection covers information required solely for administrative or operational purposes, date of document as shown hereon 30 August 2001, ASC/YTK.

WARNING:  This document contains technical data whose export is restricted by the Arms Export Control Act (Title 22, U. S. C. 2751 et seq.) or Executive Order 12470.  Violation of these export control laws is subject to severe criminal penalties.  Dissemination of this document is controlled under DOD Directive 5230.25.

LIST OF EFFECTIVE PAGES

Insert latest change page, dispose of superseded pages.  Date(s) of issue for original and change pages are listed below.

| Rev. | Date | Reason for Revision |
| --- | --- | --- |
| Draft | 7 May 2001 |  |
| Final | 30 Aug 2001 |  |
| Rev-A | 11 Sep 2001 | Revise in accordance with SPO telecon of 11 Sep 2001. |

| Page # | Rev. # | Page # | Rev. # | Page # | Rev. # | Page # | Rev. # |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Title | A |  |  |  |  |  |  |
| i | A |  |  |  |  |  |  |
| 4 | A |  |  |  |  |  |  |

TABLE OF CONTENTS

1	Scope	1

1.1	Identification	1

1.2	System Overview	2

1.3	Document Overview	3

2	Referenced Documents	4

3	Requirements	5

3.1	Executable Software	6

3.1.1	HST Computer	6

3.1.2	IOS Computer	7

3.1.3	ECLDS Computer	8

3.1.4	FDK Computer	9

3.1.5	ACS Computer	9

3.1.6	VCC Computer	9

3.2	Source Files	10

3.2.1	HST Computer	10

3.3	Packaging Requirements	10

4	Qualification Provisions	10

5	Software Support Information	11

5.1	As-built Software Design	11

5.1.1	Requirements Engineering Analysis	12

5.1.2	Use Cases	13

5.1.3	Simulation Model Design Paradigm	14

5.1.4	Category Diagrams	15

5.1.5	Software Source File Coding	15

5.2	Compilation and Build Procedures	16

5.3	Modification Procedures	16

5.4	Computer Hardware Resource Utilization	16

6	Requirements Traceability	16

7	Notes	16

7.1	Acronyms	17

7.2	Software Integrity Testing	19

8	Appendices	19

LIST OF FIGURES

Figure 1: GBTS End Item Hierarchy	1

Figure 2: OFT System Block Diagram	2

Figure 3: System Context Diagram	12

Figure 4: Simulation Design Paradigm	15

Figure 5: Host Computer Chassis	17

LIST OF TABLES

Table 1: FTD Software Component Categories	3

Table 2: FTD Operating Locations	3

# Scope

This document is the Software Product Specification (SPS) prepared in accordance with SDRL 019, with reference to Data Item Description (DID) DI-IPSC-81441, for the system identified here.

## Identification

This document applies to the Joint Primary Aircraft Training System (JPATS), Ground Based Training System (GBTS), Aircrew Training Devices (ATD), Flight Training Device (FTD) components: the Operational Flight Trainer (OFT), the Instrument Flight Trainer (IFT) and the Unit Training Device (UTD). Figure 1 illustrates the GBTS End Item Hierarchy and the relationship between the Aircrew Training Devices (ATD) and the FTDs.

Figure 1: GBTS End Item Hierarchy

These devices will be used in the JPATS GBTS to support the provision of primary phase Undergraduate Pilot Training (UPT) to U.S. Air Force and U.S. Navy personnel, and Pilot Instructor Training (PIT).

## System Overview

Figure 2: OFT System Block Diagram

Development Item (DI) and Non-Development Item (NDI) software components reside in the sub-systems identified here as part of the Flight Training Devices (FTD).

- The JPATS acquisition program paid for the DI products.

- Non-Development Items are software products developed at the expense of FlightSafety International – Simulation Systems Division (FSI-SSD) or purchased as Commercial-Off-The-Shelf (COTS) components.

Figure 2 is a system block diagram of the Operational Flight Trainer (OFT) showing the general arrangement of the computational sub-systems; the IFT and UTD are sub-sets of the OFT configuration.

Refer to Figure 1; note that the OFT, Instrument Flight Trainer (IFT), and Unit Training Device (UTD) are classified as Flight Training Devices (FTD).

Table 1 shows the predominant software development category of each computational sub-system.

- Within each computational sub-system there is a mixture of development, non-development, and COTS software items.

- This document identifies each software component by its location on the accompanying distribution media.

Table 1: FTD Software Component Categories

| Software Component | NDI | DI | COTS |
| --- | --- | --- | --- |
| Host Computer (HST) |  | X |  |
| Instructor Operating Station (IOS) | X |  |  |
| Electric Control Loading & Dynamic Seat (ECLDS) | X |  |  |
| Flight Deck Input-Output (FDK) | X |  |  |
| Aural Cueing System (ACS) |  |  | X |
| Visual Control Computer (VCC) |  |  | X |

Table 2 lists the operating locations for the FTDs; at each location there will be a combination of OFT, IFT, and UTDs.

Table 2: FTD Operating Locations

| Randolph AFB | Moody AFB | Laughlin AFB |
| --- | --- | --- |
| Columbus AFB | Vance AFB | Sheppard AFB |
| NAS Corpus Christi | NAS Whiting | NAS Kingsville |
| NAS Meridian | NAS Pensacola |  |

## Document Overview

This document describes the various software components stored on the accompanying disk storage media for each sub-system.

The baseline configuration at the time of delivery of the JPATS software is stored on the disks that are part of this document.

This document describes where the software is stored on the accompanying disks.

The only logical security or privacy considerations associated with the use of this software is the security of student gradebook information transmitted between the Training Integrated Management System (TIMS) and the FTD Instructor Operating Station (IOS).

- Level C3 security is provided by the Windows 2000 operating system running on the IOS.

- The TIMS is the authentication server for the FTD IOS.

- Student gradebook information exchange occurs between the TIMS and the IOS only.

Integrity of the software delivered as part of this document depends upon the magnetic and optical storage media used to store the software code.

- Stray magnetic fields can corrupt data stored on magnetic media.

- Excessive heat can corrupt data stored on both magnetic and optical media.

- Ultra-violet light can corrupt data stored on optical media.

- Dirt can corrupt data read from both magnetic and optical storage media.

- Mechanical damage to the surfaces of magnetic and optical media will corrupt data read from the storage medium.

- Defective reading equipment will corrupt data read from both magnetic and optical media.

- At least two additional copies should be made of the data stored on the disks delivered as part of this document.

- Data integrity of the WinZip archives should be verified in accordance with paragraph 7.2 before making backup copies.

- Fresh backup copies should be made each year.

- Data integrity of the WinZip archives should be tested every quarter, using the test mechanism incorporated in the WinZip product, as described in paragraph 7.2.

- Backup copies of the data archives should be stored in separate locations.

No compilation or linking of source software is done on any of the computing sub-systems described in this document.

- Maintenance of the source software for the Computer Software Configuration Item (CSCI) is performed on the Aircrew Training Device Support System (ATD-SS) component of the Modification and Update Support System (MUSS).

# Referenced Documents

- JPATS-FTD-SUM – Software User Manual (SUM) for the Joint Primary Aircraft Training System (JPATS), Ground Based Training System (GBTS), Flight Training Devices (FTD).

- JPATS-SUM-MUSS – Software User Manual (SUM) for the Joint Primary Aircraft Training System (JPATS), Modification and Update Support Segment (MUSS).

- JPATS-DTP-FTD – Development Test Procedures (DTP) for the Joint Primary Aircraft Training System (JPATS), Ground Based Training System (GBTS), Flight Training Devices (FTD).

- SimPhonics Incorporated, “JPATS Audio System, Documentation and Vendor Documentation Binder Sets”.

- “Use Cases Combined with Booch, OMT, UML: Process and Products” by Putnam P. Texel and Charles B. Williams, (Prentice-Hall, 1997), ISBN: 0-13-727405-X.

- “JPATS T-6A Texan II: FTD Software Preliminary Design Review”, 10 June 1998.

- “JPATS T-6A Texan II: FTD Software Critical Design Review”, 8 April 1999.

- JPATS-SDP-GBTS – Software Development Plan (SDP) for the Joint Primary Aircraft Training System (JPATS), Ground Based Training System (GBTS).

# Requirements

Executable and source software described in this section is placed under configuration control by the organization responsible for post delivery maintenance.

The product baseline is stored on the disks found in the appendices to this Software Product Specification.

The JPATS GBTS ATD FTD is treated as a single Computer Software Configuration Item (CSCI).

Top-level directory structure on the ATD CD-ROM found in Appendix-A is as defined here:

- flight-training-devices

- vxworks

- hst-computer

- ios-computer

- eclds-computer

- fdk-computer

- acs-computer

Each of the three FTD hardware configurations (OFT, IFT, UTD) uses a copy of the same CSCI.

## Executable Software

This paragraph describes, by reference to the enclosed CD-ROMs listed in Section 8 and included as appendices, the executable software for the CSCI, including any batch files, command files, data files, or other software files needed to install and operate the software on each target computer.

COTS software required to support operation of the CSCI will be described by reference to the COTS CD-ROMs listed in Section 8 and included in the appendices to this document.

Executable software is provided for the computers listed here, described in the following sub-paragraphs.

- HST computer,

- IOS computer,

- ECLDS computer,

- FDK computer,

- ACS computer,

- VCC computer.

The FTD Software User Manual (SUM) [Ref-2a] gives instructions for installing and running the software on each of the computer sub-systems listed above.

### HST Computer

Host (HST) computer executable software is in two parts as listed here; see the WinZip archive files in Appendix-A [Ref-8a], CD-ROM labeled: JPATS-ATD-SPS-A.

- Wind River Systems VxWorks Real-Time Operating System (RTOS) file:
\flight-training-devices\VxWorks\VxWorks.zip.

- Flight Training Device (FTD) HST computer simulation system:
\flight-training-devices\hst-computer\jpats_host_load-yyyymmdd.zip;
where yyyy is the numeric year, mm is the numeric month, and dd is the numeric day.

Within the HST computer, VxWorks supports the execution in real-time of the FTD simulation system.

VxWorks must be present before the simulation system software can be loaded and run.

The FTD Software User Manual (SUM) [Ref-2a] gives instructions for installing and running the HST executable software.

### IOS Computer

Instructor Operating Station (IOS) executable software is in several parts:

Microsoft Windows-2000 Operating System with Service Pack-2 and Windows Services for UNIX; see the appendices listed here:

- CD-ROM Appendix-B to JPATS-FTD-SPS [Ref-8b], labeled:
JPATS-FTD-SPS-B.

- CD-ROM Appendix-C to JPATS-FTD-SPS [Ref-8c], labeled:
JPATS-FTD-SPS-C.

- CD-ROM Appendix-D to JPATS-FTD-SPS [Ref-8d], labeled:
JPATS-FTD-SPS-D.

Plotting utilities for the Automatic Test Guide (ATG) use Adobe Systems Incorporated Acrobat to convert the PostScript output of the ATG to files that can be read and printed.

- See CD-ROM Appendix-E to JPATS-FTD-SPS [Ref-8d], labeled:
JPATS-FTD-SPS-E for Adobe Acrobat.

- Original development of the ATG was done with Version 3.01 of Adobe Acrobat.

- Version 4.0 of Adobe Acrobat failed to convert the PostScript files produced by the ATG.

- Each FTD is provided with a license for a later version of Adobe Acrobat than the installed version 3.01.

- The ATG is part of the IOS.

- Use of the ATG is described in the Volume-1, Simulator Performance, of the FTD Development Test Procedures (DTP) [Ref-2c].

- Use of the plotting utilities is described in the Volume-1, Simulator Performance, of the FTD Development Test Procedures (DTP) [Ref-2c].

Driver software for the 3D Labs video interface card must be installed as part of the Windows-2000 environment.

- See Appendix-A to JPATS-FTD-SPS [Ref-8a], labeled: JPATS-FTD-SPS-A file: \flight-training-device\ios-computer\w2k2150614.zip

Archiving software must be installed as part of the Windows-2000 environment.

- See Appendix-A to JPATS-FTD-SPS [Ref-8a], labeled: JPATS-FTD-SPS-A file: \flight-training-device\ios-computer\winzip.zip

Flight Training Device (FTD) IOS Operating System Software.

- See Appendix-A to JPATS-FTD-SPS [Ref-8a], labeled: JPATS-FTD-SPS-A file: \aircrew-training-device\ios-computer\jpats-ios.zip

Znyx Windows 2000 driver fix program.

- See Appendix-A to JPATS-FTD-SPS [Ref-8a], labeled: JPATS-FTD-SPS-A file: \aircrew-training-device\ios-computer\znyx-diagnostics.zip

The FTD Software User Manual (SUM) [Ref-2a] gives instructions for installing and running the software IOS executable software.

### ECLDS Computer

Electric Control Loading and Dynamic Seat (ECLDS) computer executable software is in two parts as listed here; see the WinZip archive files in Appendix-A [Ref-8a], CD-ROM labeled: JPATS-FTD-SPS-A.

- Wind River Systems VxWorks Real-Time Operating System (RTOS) file:
\flight-training-devices\VxWorks\VxWorks.zip.

- ECLDS simulation system; see Appendix-A to JPATS-FTD-SPS file:
\flight-training-devices\eclds-computer\jpats-eclds-20010413.zip

Within the ECLDS computer, VxWorks supports the execution in real-time of the FTD simulation system.

VxWorks must be present before the simulation system software can be loaded and run.

The FTD Software User Manual (SUM) [Ref-2a] gives instructions for installing and running the software ECLDS executable software.

### FDK Computer

Executable software for the Flight Deck I/O (FDK) computer is in two parts as listed here:

- Windows 98 DOS mode – see the Flight Deck I/O Windows-98, DOS-mode, Boot-disk in Appendix-H to JPATS-FTD-SPS [Ref-8a], floppy disk labeled: JPATS-FTD-SPS-H.

- FDK operating software – see the Flight Deck I/O Operating Software in Appendix-I to JPATS-FTD-SPS-I [Ref-8a], floppy disk labeled: JPATS-FTD-SPS-I.

The FTD Software User Manual (SUM) [Ref-2a] gives instructions for installing and running the software FDK executable software.

### ACS Computer

Aural cueing simulation is a Commercial-Of-The-Shelf product provided by SimPhonics Incorporated.

Aural cueing software is in two parts:

- The operating environment as delivered by SimPhonics Incorporated, described in the JPATS Audio System, Documentation and Vendor Documentation Binder Sets  [Ref-2d] delivered with the equipment.

- JPATS Aural Cueing System model – see Appendix-A to JPATS-FTD-SPS [Ref-8a], CD-ROM labeled JPATS-FTD-SPS-A, file: \flight-training-devices\acs-computer\jpats

SimPhonics Inc. V+ requires a specific PC hardware configuration to run under the Windows-98 operating system environment.

### VCC Computer

The Visual Control Computer (VCC) is a part of the Vital VIII visual system, a Commercial-Of-The-Shelf (COTS) product supplied by FlightSafety International – Visual Simulation Systems (FSI-VSS).

Refer to the Vital VIII documentation for information about the VCC computer.

## Source Files

This paragraph describes, by reference, the source software for the CSCI, including any batch files, command files, data files, or other software files needed to regenerate the executable software on each target computer.

Source software is provided for the computers listed here, described in the following sub-paragraphs.

- Host computer.

### HST Computer

Ada source code for the host computer is located in the ClearCase repository on the ATD Support System (ATD-SS).

The MUSS Software User Manual (SUM) [Ref-2b] describes the process to extract the Latest View containing most current software checked into the repository, and the commands required to invoke the build script.

## Packaging Requirements

The magnetic and optical media delivered in the appendices to this document are each marked with this document number, JPATS-FTD-SPS, plus a single letter suffix to designate to which appendix the medium belongs.

- For example: JPATS-FTD-SPS-A identifies Appendix-A to document JPATS-FTD-SPS-A [Ref-8a].

# Qualification Provisions

These qualification provisions apply to the Development Item (DI) software only.

- NDI and COTS software is supplied by the vendors “as is”.

Consistency in compilation of source software is ensured through the use of four aspects of the ClearCase Computer Aided Software Engineering (CASE) tool:

- ClearCase source code repository contains and manages the configuration of the source software,

- The ClearCase Version Object Base (VOB) controls what view of the repository is available.

- The industry standard Makefile in the repository directs which source files to compile from the view,

- A control variable specifies whether to produce a debug-load or a release-load.

In addition, architecture of the Ada software will cause a fatal compilation error when a module is missing from the build.

Validation of the executable software is accomplished by testing in accordance with the Development Test Plan [Ref-2c].

# Software Support Information

This section describes information needed to support the CSCI it is divided into four main sections:

- As-built software design,

- Compilation and build procedures,

- Modification procedures,

- Computer hardware resource utilization.

## As-built Software Design

Design of the as-built software design is based upon Object-Oriented Analysis (OOA) and Object-Oriented Design (OOD) using modifications to the Texel-Williams Process (TWP) [Ref-2e] as described in the Software Preliminary Design Review (PDR) [Ref-2f] and Software Critical Design Review (CDR) [Ref-2g].

- Appendix-F of the SDP [Ref-2h] documents the modifications to the Texel-Williams Process.

CSCI software design produces five main products during the seventeen phases of the TWP; these products are listed here and discussed in more detail in the following sub-paragraphs.

- Requirements Engineering Analysis,

- Use cases,

- Simulation Model Design Paradigm,

- Category diagrams,

- Software source file coding,

### Requirements Engineering Analysis

Engineering analysis of the specification requirements for the JPATS FTD produced results from three activities.

- A definition of the external interfaces.

- A list of requirements specific to the FTD.

- An allocation of requirements to FTD hardware and software.

#### External Interfaces

Figure 3 describes the interfaces external to the FTD; a copy of this diagram is located in Appendix-A [Ref-8a], CD-ROM labeled: JPATS-ATD-SPS-A.

Figure 3: System Context Diagram

- Path and filename is: \flight-training-devices\hst-computer\1-requirements\system-context-diagram.pdf

#### Requirements Identification

The specification documents were parsed to produce a list of requirements specific to the FTD; these requirements were identified by the presence of the verb “shall”.

- A total of 1,667 “shall” statements were identified.

Each requirement was assigned to a functional area within the program and defined as hardware, software, or both hardware and software.

A copy of this list is located in Appendix-A [Ref-8a], CD-ROM labeled: JPATS-ATD-SPS-A.

- Path and filename is: \flight-training-devices\hst-computer\1-requirements\requirements-identification.pdf

#### Requirements Allocation

585 specification requirements were allocated to functional departments within the engineering organization, as defined here.

Allocated requirements are located in Appendix-A [Ref-8a], CD-ROM labeled: JPATS-ATD-SPS-A.

- Aircraft Systems: path and filename is: \flight-training-devices\hst-computer\1-requirements\requirements-allocation-aircraft-systems.pdf.

- Navigation Systems: path and filename is: \flight-training-devices\hst-computer\1-requirements\requirements-allocation-navigation.pdf.

- Instructor Operating System: path and filename is: \flight-training-devices\hst-computer\1-requirements\requirements-allocation-ios.pdf.

- Flight Systems: path and filename is: \flight-training-devices\hst-computer\1-requirements\requirements-allocation-flight.pdf.

- Propulsion System: path and filename is: \flight-training-devices\hst-computer\1-requirements\requirements-allocation-propulsion.pdf.

- Computer Systems: path and filename is: \flight-training-devices\hst-computer\1-requirements\requirements-allocation-computer-systems.pdf.

### Use Cases

Analysis of the allocated requirements identified the Use Cases associated with the FTD.

The Use Cases are located in Appendix-A [Ref-8a], CD-ROM labeled: JPATS-ATD-SPS-A, path and filename is: \flight-training-devices\hst-computer\2-use-cases\use-case-analysis.pdf

- Aircraft Systems: path and filename is: \flight-training-devices\hst-computer\2-use-cases\use-cases-aircraft-systems.pdf

- Navigation Systems: path and filename is: \flight-training-devices\hst-computer\2-use-cases\use-cases-navigation.pdf

- Instructor Operating Station: path and filename is: \flight-training-devices\hst-computer\2-use-cases\use-cases-instructor-operating-station.pdf

- Flight Systems: path and filename is: \flight-training-devices\hst-computer\2-use-cases\use-cases-flight-systems.pdf

- Propulsion System: path and filename is: \flight-training-devices\hst-computer\2-use-cases\use-cases-propulsion.pdf

- Computer Systems: path and filename is: \flight-training-devices\hst-computer\2-use-cases\use-cases-computer-systems.pdf

- Visual System: path and filename is: \flight-training-devices\hst-computer\2-use-cases\use-cases-visual.pdf

- Primary Flight Controls: path and filename is: \flight-training-devices\hst-computer\2-use-cases\use-cases-primary-flight-controls.pdf

### Simulation Model Design Paradigm

Systems analysis of the JPATS GBTS ATD FTD resulted in the simulation model design paradigm shown in Figure 4.

Figure 4: Simulation Design Paradigm

A larger, printable copy of this diagram is located in Appendix-A [Ref-8a], CD-ROM labeled: JPATS-ATD-SPS-A, path and filename is: \flight-training-devices\hst-computer\design-paradigm-2-0a.pdf

### Category Diagrams

Categories are groups of related classes.

The Category Diagrams are located in Appendix-A [Ref-8a], CD-ROM labeled: JPATS-ATD-SPS-A.

Category diagrams are divided in to two types: Static, and Dynamic.

- Static Category Diagrams: Path is: \flight-training-devices\hst-computer\4-category-diagrams\static-category-diagrams.zip

- Dynamic Category Diagrams: Path is: \flight-training-devices\hst-computer\4-category-diagrams\dynamic-category-diagrams.zip

### Software Source File Coding

The programmer, working at the ATD-SS, has a view of the Clear Case repository that is accessible through the file explorer.

The ATD section of the MUSS SUM [Ref-2b] describes how to check out the software files from the Clear Case repository into view of the programmer.

- Once these files are checked out, the programmer can edit them with the editor supplied as part of the Greenhills AdaMULTI development environment.

## Compilation and Build Procedures

Compilation and build procedures for the Aircrew Training Devices is described in the ATD section of the MUSS SUM [Ref-2b].

The Ada-Multi Make system and ClearCase configuration management system work together to produce a consistent, reproducible load.

## Modification Procedures

Modification of the CSCI is conducted at the Aircrew Training Device Support System (ATD-SS) element of the Modification and Update Support System (MUSS).

Refer to the ATD section of the MUSS SUM [Ref-2b] for detailed instructions to modify the CSCI.

## Computer Hardware Resource Utilization

As delivered, the CSCI meets the requirements for processor and memory spare capacity.

# Requirements Traceability

Requirements traceability is maintained at the ATD-SS using the Telelogic Dynamic Object Oriented Requirements System (DOORS).

# Notes

General information and aids to understanding are presented in this section.

An illustration of the host computing system is provided here.

Figure 5: Host Computer Chassis

## Acronyms

Acronyms and abbreviations used in this document are listed here.

| ACS | Aural Cueing System |
| --- | --- |
| ATG | Automatic Test Guide |
| ATD | Aircrew Training Device |
| ATD-SS | Aircrew Training Device – Support System |
| CASE | Computer Aided Software Engineering |
| CDR | Critical Design Review |
| CD-ROM | Compact Disk – Read Only Memory |
| COTS | Commercial-Of-The-Shelf |
| CSCI | Computer Software Configuration Item |
| DI | Development Item |
| DID | Data Item Description |
| DOORS | Dynamic Object Oriented Requirements System |
| DOS | Disk Operating System |
| ECLDS | Electric Control Loading & Dynamic Seat |
| FDK | Flight Deck I/O computer |
| FSI-SSD | FlightSafety International – Simulation Systems Division |
| FSI-VSS | FlightSafety International – Visual Systems Division |
| FSSC | FlightSafety Services Corporation |
| FTD | Flight Training Device |
| GBTS | Ground Based Training System |
| HST | Host computer |
| I/O | Input-Output |
| IFT | Instrument Flight Trainer |
| IOS | Instructor Operating Station |
| JPATS | Joint Primary Aircraft Training System |
| MUSS | Modification and Update Support System |
| NDI | Non-Development Item |
| OFT | Operation Flight Trainer |
| OOA | Object-Oriented Analysis |
| OOD | Object-Oriented Design |
| PC | Personal Computer |
| PDP | Programmed Data Processor |
| PDR | Preliminary Design Review |
| PIT | Pilot Instructor Training |
| RTOS | Real-Time Operating System |
| SDRL | Sub-contract Data Requirements List |
| SPS | Software Product Specification |
| SUM | Software User Manual |
| TWP | Texel-Williams Process |
| UNIX | “<operating system> /yoo'niks/ (Or "UNIX", in the authors' words, "A weak pun on Multics") Plural "Unices". An interactive time-sharing operating system invented in 1969 by Ken Thompson after Bell Labs left the Multics project, originally so he could play games on his scavenged PDP-7. Dennis Ritchie, the inventor of C, is considered a co-author of the system.” Reference Internet URL [http://www.nightflight.com/cgi-bin/foldoc.cgi?query=unix] |
| UPT | Undergraduate Pilot Training |
| URL | Uniform Resource Locator |
| UTD | Unit Training Device |
| VCC | Visual Control Computer |
| VOB | Version Object Base |

## Software Integrity Testing

Open each archive with the WinZip utility.

Test the integrity of the data by invoking the test function by pressing Shift-T on the keyboard.

Verify that each file in the archive is marked as OK in the test results window.

# Appendices

- JPATS-ATD-SPS-A – FlightSafety International-Simulation Systems Division, Computational Systems Software for the JPATS GBTS ATD Flight Training Devices (CD-ROM).

- JPATS-ATD-SPS-B – Microsoft Corporation Windows-2000, Operating System (CD-ROM).

- JPATS-ATD-SPS-C – Microsoft Corporation Windows-2000, Service Pack 2 (CD-ROM).

- JPATS-ATD-SPS-D – Microsoft Corporation Windows Services for UNIX v2.0 (CD-ROM).

- JPATS-ATD-SPS-E – Adobe Systems Incorporated, Acrobat document management system (CD-ROM).

- JPATS-ATD-SPS-F – Microsoft Corporation Windows-98, Operating System (CD-ROM).

- Reserved.

- JPATS-ATD-SPS-H – Flight Deck I/O, Windows-98, DOS-mode Boot-disk (Floppy Disk).

- JPATS-ATD-SPS-I – Flight Deck I/O Operating Software (Floppy Disk).